3GPP TSG-RAN WG2#106	R2-1907311

# Reno, Nevada, USA, 13 – 17 May, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4009 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing messages in “Protection of RRC messages” table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_HRLLC, MBMS_LTE_enh2-Core, LTE_QMC_Streaming-Core, LTE_feMTC-Core, NB_IOTenh-Core |  |  |  |  |  |  | Date: |  | 2019-05-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Annex A.6 includes a table providing information on which messages can be sent (unprotected) prior to AS security activation and which messages can be sent without integrity protection/unciphered after AS security activation. Some RRC messages are not listed in the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The table in Annex A.6 is updated including the following missing messages with the relevant information: - FailureInformation - MasterInformationBlock-MBMS - MeasReportAppLayer - SCPTMConfiguration-BR - SystemInformationBlockType1-MBMS Impact analysis Impacted functionality: RRC procedures initiation prior to/after AS security Inter-operability: - If the UE implements the change but the NW does not, no issue on the NW side is expected. - If the NW implements the change but the UE does not, no issue on the UE side is expected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence is the table in Aneex A.6 does not include all the existing RRC messages. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6 Protection of RRC messages |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## A.6		Protection of RRC messages (informative)

The following list provides information which messages can be sent (unprotected) prior to security activation and which messages can be sent unprotected after security activation. Those messages indicated "-" in "P" column should never be sent unprotected by eNB or UE. Further requirements are defined in the procedural text.

P…Messages that can be sent (unprotected) prior to security activation

A - I…Messages that can be sent without integrity protection after security activation

A - C…Messages that can be sent unciphered after security activation

NA… Message can never be sent after security activation

| Message | P |  | A-I | A-C | Comment |
| --- | --- | --- | --- | --- | --- |
| CSFBParametersRequestCDMA2000 | + |  | - | - |  |
| CSFBParametersResponseCDMA2000 | + |  | - | - |  |
| CounterCheck | - |  | - | - |  |
| CounterCheckResponse | - |  | - | - |  |
| DelayBudgetReport | - |  | - | - |  |
| DLInformationTransfer | + |  | - | - |  |
| FailureInformation | - |  | - | - |  |
| HandoverFromEUTRAPreparationRequest (CDMA2000) | - |  | - | - |  |
| InDeviceCoexIndication | - |  | - | - |  |
| InterFreqRSTDMeasurementIndication | - |  | - | - |  |
| LoggedMeasurementsConfiguration |  | - | - | - |  |
| MasterInformationBlock | + |  | + | + |  |
| MasterInformationBlock-MBMS | + |  | + | + |  |
| MBMSCountingRequest | + |  | + | + |  |
| MBMSCountingResponse | - |  | - | - |  |
| MBMSInterestIndication | + |  | - | - |  |
| MBSFNAreaConfiguration | + |  | + | + |  |
| MeasReportAppLayer | - |  | - | - |  |
| MeasurementReport | - |  | - | - | Measurement configuration may be sent prior to security activation. But: In order to protect privacy of UEs, MEASUREMENT REPORT is only sent from the UE after successful security activation. |
| MobilityFromEUTRACommand | - |  | - | - |  |
| Paging | + |  | + | + |  |
| ProximityIndication | - |  | - | - |  |
| RNReconfiguration | - |  | - | - |  |
| RNReconfigurationComplete | - |  | - | - |  |
| RRCConnectionReconfiguration | + |  | - | - | The message shall not be sent unprotected before security activation if it is used to perform handover or to establish SRB2, SRB4 and DRBs |
| RRCConnectionReconfigurationComplete | + |  | - | - | Unprotected, if sent as response to RRCConnectionReconfiguration which was sent before security activation |
| RRCConnectionReestablishment | - |  | + | + | This message is not protected by PDCP operation. |
| RRCConnectionReestablishmentComplete | - |  | - | - |  |
| RRCConnectionReestablishmentReject | - |  | + | + | One reason to send this may be that the security context has been lost, therefore sent as unprotected. |
| RRCConnectionReestablishmentRequest | - |  | - | + | This message is not protected by PDCP operation. However, a short MAC-I is included. |
| RRCConnectionReject | + |  | + | + | Except for UP-EDT, A-I and A-C are NA. |
| RRCConnectionRelease | + |  | - | - | Justification for P: If the RRC connection only for signalling not requiring DRBs or ciphered messages, or the signalling connection has to be released prematurely, this message is sent as unprotected. For UP-EDT, the message is only sent after successful security activation. RRCConnectionRelease message sent before security activation cannot include rrc-InactiveConfig, redirectedCarrierInfo, idleModeMobilityControlInfo information fields when UE is connected to 5GC. |
| RRCConnectionRequest | + |  | NA | NA |  |
| RRCConnectionResume | - |  | - | + | When this message is transmitted, security is activated but suspended. Integrity verification is done after the message received by RRC. For UP-EDT, the message is only sent after successful security activation. For RRC_INACTIVE state, the message is protected with both integrity and ciphering. |
| RRCConnectionResumeRequest | - |  | - | + | This message is not protected by PDCP operation. However, a short MAC-I is included. |
| RRCConnectionResumeComplete | - |  | - | - |  |
| RRCConnectionSetup | + |  | NA | NA |  |
| RRCConnectionSetupComplete | + |  | NA | NA |  |
| RRCEarlyDataRequest | + |  | NA | NA |  |
| RRCEarlyDataComplete | + |  | NA | NA |  |
| SCGFailureInformation | - |  | - | - |  |
| SCGFailureInformationNR | - |  | - | - |  |
| SCPTMConfiguration | + |  | + | + |  |
| SCPTMConfiguration-BR | + |  | + | + |  |
| SecurityModeCommand | + |  | NA | NA | Integrity protection applied, but no ciphering (integrity verification done after the message received by RRC) |
| SecurityModeComplete | - |  | NA | NA | Integrity protection applied, but no ciphering. Ciphering is applied after completing the procedure. |
| SecurityModeFailure | + |  | NA | NA | Neither integrity protection nor ciphering applied. |
| SidelinkUEInformation | + |  | - | - |  |
| SystemInformation | + |  | + | + |  |
| SystemInformationBlockType1 | + |  | + | + |  |
| SystemInformationBlockType1-MBMS | + |  | + | + |  |
| UEAssistanceInformation | - |  | - | - |  |
| UECapabilityEnquiry | + |  | - | - |  |
| UECapabilityInformation | + |  | - | - |  |
| UEInformationRequest | - |  | - | - |  |
| UEInformationResponse | - |  | - | - | In order to protect privacy of UEs, UEInformationResponse is only sent from the UE after successful security activation |
| ULHandoverPreparationTransfer (CDMA2000) | - |  | - | - | This message should follow HandoverFromEUTRAPreparationRequest |
| ULInformationTransfer | + |  | - | - |  |
| ULInformationTransferMRDC | - |  | - | - |  |
| WLANConnectionStatusReport | - |  | - | - |  |

END OF CHANGE