3GPP TSG-RAN WG2#106	Tdoc R2-1907319

Reno, Nevada, US, 13th – 17th May 2019	Revision of R2-1903511

Agenda Item:	11.9.2.1

Source:	Ericsson

Title:	Comparison of MBB and DC based handover

Document for:	Discussion

# Introduction

The Rel-16 NR mobility enhancements WI aims to reduce handover interruption time and improve mobility robustness. Currently two main solutions have been proposed for achieving reduced handover interruption:

- DC based handover aka split-bearer solution

- MBB (Make-before-break) aka non-split bearer solution

A detailed description of these two solutions can be found in [2], [3]. In this contribution we compare DC based handover and MBB based on the evaluation criteria agreed in RAN2#105:

Agreements

1	Solution proposals should consider at least the following evaluation criteria:

- Mobility robustness

- Interruption time

2	Other criteria to be considered are:

- Applicable deployment scenarios

- Signalling overhead

- Specification effort

- UE/network complexity

# Discussion

Interruption time

One main difference between the NR and LTE work items on mobility enhancements is the requirement on reduced handover interruption time. In LTE, the target for reduced user data interruption time during handover is to come as close as possible to 0ms, while for NR, the target is 0ms interruption time.

Based on [2] and [3] it seems both DC based handover and MBB can achieve 0ms interruption.

Applicable deployment scenarios

Both DC based handover and MBB require that the UE can receive and transmit on two cells simultaneously which means the applicable deployment scenarios are the same. In [4] we analyze in which scenarios simultaneous reception/transmission can be supported based on the LS replies from RAN1 [5] and RAN4 [6].

One potential issue in DC based handover is that the target node needs to be added as a secondary node while the radio conditions are still good. This means the solution may be less suitable for scenarios where the radio environment is changing rapidly or in small cell deployments where there are many potential target cells. This problem can potentially be avoided with the optimized version of DC based handover suggested in [7] where the SN addition and role change are merged into a single procedure.

Signalling overhead

The table below shows the signaling overhead for DC based handover and MBB as well as for regular Xn handover. From these numbers, it is clear that DC based handover requires more radio and network signaling than MBB.

It should be noted here that the exact number of Uu/Xn messages in MBB depends on the MBB solution chosen. In some descriptions of MBB the source cell is released using a separate RRC reconfiguration procedure which means that two additional Uu messages would be needed. Even with this change though the signaling overhead for MBB is still less than for DC based handover.

As mentioned under “Applicable deployment scenarios”, an optimized version of DC based handover is suggested in [7] where the SN addition and role change are performed together. In this case the signaling overhead of DC based handover would be comparable to that of MBB. However, there are many details of this solution that still need to be explained. In particular, it is unclear what purpose the split bearer serves in this case. If the split bearer can be replaced with a non-split bearer, the resulting solution may be similar to MBB.

|  | Nr of Uu messages | Nr of Xn messages |
| --- | --- | --- |
| Regular Xn HO (baseline) | RA + 2 (HO) | 2 (Handover preparation) + 1 (SN status transfer) + 1 (UE Context Release) |
| MBB | RA + 2 (HO) | 2 (Handover preparation) + 1 (SN status transfer) + 1 (HO complete indication) + 1 (UE Context Release) |
| DC based HO | RA + 2 (SN addition) + 2 (Role change) + 2 (SN removal) | 2 (SN addition) + 2 (Role change) + 1 (SN status transfer) + 2 (SN removal) |

Specification impact

One benefit of DC based handover is that it builds on the existing DC framework specified as part of NR Rel-15 late-drop. Except for the role switch procedure, it appears most of the functionality is already supported by existing procedures. On the other hand, the role switch is the most complex part and ensuring that it is interruption free is not trivial. Specifying the DC based handover therefore does not seem any simpler than specifying MBB.

We also note that (enhanced) MBB has already been chosen as the solution for reduced handover interruption in the Rel-16 LTE mobility enhancements WI.  If MBB is adopted also for NR, parts of the specification work for LTE can potentially be re-used for NR.

UE/network complexity

The fact that DC based handover relies on DC is also a drawback in that the whole DC framework needs to be implemented to support the feature. This is not a problem if DC is anyway implemented but from LTE experience we know this may not be the case. For this reason, MBB may be simpler to implement since it is a smaller and independent feature that only targets a specific use case.

Due to the use of split bearer, the DC based handover also suffers from so called key confusion (see [3]). The same confusion also exists for the ROHC context if header compression is applied for a DRB. Another issue with split bearer is that it is more difficult to avoid duplicate DL transmissions from the target cell (see [3]).

Mobility robustness

Reduced handover interruption and mobility robustness are separate WI objectives and both do not necessarily need to be fulfilled at the same time or by the same solution, although that would obviously be beneficial.

In our view the main solution for achieving mobility robustness is conditional handover. To achieve both mobility robustness and reduced handover interruption, conditional handover can be combined with MBB.

DC and DC based handover can provide some level of mobility robustness as the UE is connected to two cells at the same time and you have features such as PDCP duplication that can increase reliability. However, it is not a replacement for conditional handover since DC is limited to two nodes.

# Conclusion

The analysis in section 2 is summarized in the table below.

|  | DC based handover | MBB |
| --- | --- | --- |
| Reduced handover interruption | 0 ms (assuming dual Rx/Tx) | 0 ms (assuming dual Rx/Tx) |
| Applicable deployment scenarios | Same scenarios (i.e. intra-frequency, inter-frequency etc.) supported by both solutions. DC based handover may be less suitable for scenarios where the radio environment is changing rapidly or in small cell deployments where there are many potential target cells due to the need to add the SN early. Note: Currently only inter-band DC is supported, i.e. the two cell groups use different non-overlapping frequency bands | Same scenarios (i.e. intra-frequency, inter-frequency etc.) supported by both solutions. |
| Signalling overhead | RA + 6 Uu + 7 Xn An optimized variant of DC based handover is proposed in [7] which requires less signalling. However, the details of this solution and how it differs from MBB still needs to be explained. | RA + 2 Uu + 5 Xn Two extra Uu messages are needed if the source cell is released using RRC reconfiguration. |
| Specification impact | Similar impact for both solutions (?). Can re-use parts of the DC framework but the role switch needs to be specified. | Similar impact for both solutions (?). May be possible to re-use parts of the LTE work. |
| UE/network complexity | Depends on if UE and network vendors already implement DC. If not, then implementation effort will be higher. Open issue on how to avoid key confusion during role switch. Open issue on how to avoid duplicate DL transmissions from target during role switch. | MBB can be seen as a trimmed down version of DC only solving one particular use case. Hence the implementation will be lower. |
| Mobility robustness | DC and DC based handover provides limited level of mobility robustness. Can not replace CHO due to the two node restriction. | Must be combined with CHO to provide mobility robustness. |

Based on the analysis we propose:

Specify MBB (i.e. non-split bearer solution) for reaching the 0ms interruption time target.

# References

RP-181433, New WID: NR mobility enhancements

R2-1907316, Make-Before-Break handover, Ericsson, RAN2#106

R2-1907317, DC based handover, Ericsson, RAN2#106

R2-1907315, On the support of simultaneous reception and transmission, Ericsson, RAN2#106

R4-1904826, Reply LS on NR mobility enhancement, from RAN4 to RAN2 and RAN1

R2-1905423, Reply LS on NR mobility enhancements, From RAN1 to RAN2, cc RAN4

R2-1903563, Procedure analysis of DC-based handover, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, RAN2#105bis