3GPP TSG-RAN2 Meeting #106 	R2-1907327

Reno, Nevada, USA, May 13th – 17th, 2019	Revision of R2-1904004

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RRC Segmentation – Downlink (38.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS_RAN |  |  |  |  |  |  | Date: |  | 2019-05-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In connection to introduction of RRC Segmentation for purposes of capability signaling, it is also proposed to allow segmentation of downlink messages, RRCResume and RRCReconfiguration. This CR introduce RRC segmentation for these two DL messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding support of RRC Segmentation for RRCResume and RRCReconfiguration - Adding capability indication for RRC Segmentation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC Segmentation for RRCResume and RRCReconfiguration will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.3.3, (6.3.0, 6.3.4*) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 38.306 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | *Note that parts of this CR is conditional on that the framework for RRC Segmentation is agreed. This relates to inclusions in chapters 6.3.0 and 6.3.4 but these changes are expected to be introduced already with the CR related to UL segmentation (R2-1907328). If this uplink CR is not agreed, but this CR is, then the change needs to be made for DL also for chapter 6.3.0 and 6.3.4, included herein. Intention is to merge this dCR into running CR |  |  |  |  |  |  |  |  |

First CHANGE

### 6.2.2	Message definitions

#### === Unmodified sections omitted ===

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=              SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcReconfiguration                  RRCReconfiguration-IEs,

criticalExtensionsFuture            SEQUENCE {

rrcReconfigurationSegment			RRCSegment { RRCReconfiguration-IEs, 5 },

criticalExtensionsFuture2           SEQUENCE {}

}

}

}

RRCReconfiguration-IEs ::=          SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540        OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}              OPTIONAL

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE. The UE may assume that configuration in dedicatedSIB1-Delivery has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8 to the UE.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message.

keySetChangeIndicator True is used in an intra-cell handover when a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. False is used in an intra-NR handover when the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

otherConfig Contains configuration related to other configurations.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

secondaryCellGroup Configuration of secondary cell group (EN-DC).

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is not present in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case of Handover with change of the AS security algorithms (as indicated in SecurityAlgorithmConfig in SecurityConfig, included in the received RadioBearerConfig). If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is not present otherwise. |

#### === Unmodified sections omitted ===

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=                       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcResume                           RRCResume-IEs,

criticalExtensionsFuture            SEQUENCE {

rrcResumeSegment	                RRCSegment { RRCResume-IEs, 5 },

criticalExtensionsFuture2           SEQUENCE {}

}

}

}

RRCResume-IEs ::=                   SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                                       OPTIONAL, -- Need M

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                               OPTIONAL, -- Need M

measConfig                          MeasConfig                                                              OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                                       OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

RRCResume-IEs field descriptions

masterCellGroup Configuration of the master cell group (NR Standalone):

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

NEXT CHANGE – Same as in R2-1907328

## 6.3	RRC information elements

### 6.3.0	Parameterized types

#### –	RRCSegment

The IE RRCSegment is used to convey segments of large information elements that exceed the maximum RRC message size (e.g. due to the PDCP SDU size limit of 9000 bytes). The ElementTypeParam determines the IE type to be segmented by the transmitting RRC entity and reassembled by the receiving RRC entity. The maxNrofElements determines the maximum number of segments for an instance of ElementTypeParam.

The RRCSegment type should be used on the top level of an RRC message. The RRC messages containing the segments of the same original RRC information element shall contain the same rrc-TransactionIdentifier.

The transmitting entity (UE or NW) uses the RRCSegment only if the unsegmented message exceeds the size limit of PDCP (9000 bytes).

RRCSegment information element

-- ASN1START

-- TAG-RRCSEGMENT-START

RRCSegment { ElementTypeParam, INTEGER:maxNrofSegments } ::=          SEQUENCE {

segmentNumber           INTEGER(1..maxNrofSegments),

isLastSegment           BOOLEAN,

segment                 OCTET STRING

}

-- TAG-RRCSEGMENT-STOP

-- ASN1STOP

RRCSegment field descriptions

isLastSegment Indicates that segment contains the last segment of the segmented information element.

segment One segment of a contained information element.

segmentNumber The ID of the segment which the receiving RRC entity uses to reconstruct the original information element.

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUPRELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

-- TAG-SETUPRELEASE-STOP

-- ASN1STOP

NEXT CHANGE

### 6.3.3	UE capability information elements

#### === Unmodified sections omitted ===

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                      OPTIONAL,

mac-Parameters                  MAC-Parameters                      OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode         OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode         OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode         OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode         OPTIONAL,

featureSets                     FeatureSets                         OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530              OPTIONAL

}

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530      OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530      OPTIONAL,

dummy                                    ENUMERATED {supported}                 OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                    OPTIONAL,

inactiveState                            ENUMERATED {supported}                 OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                 OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                 OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                         OPTIONAL,

overheatingInd                          ENUMERATED {supported}                  OPTIONAL,

ims-Parameters                          IMS-Parameters                          OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540       OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540       OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode             OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                  OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                 OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-16xy                    OPTIONAL

}

UE-NR-Capability-16xy ::=				SEQUENCE {

rrcSegmentationReconfigResume-r16		ENUMERATED {supported}                  OPTIONAL,

nonCriticalExtension					SEQUENCE {}                             OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=         SEQUENCE {

phy-ParametersXDD-Diff                  Phy-ParametersXDD-Diff                  OPTIONAL,

mac-ParametersXDD-Diff                  MAC-ParametersXDD-Diff                  OPTIONAL,

measAndMobParametersXDD-Diff            MeasAndMobParametersXDD-Diff            OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::= SEQUENCE {

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                      OPTIONAL,

measAndMobParametersFRX-Diff        MeasAndMobParametersFRX-Diff                OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                 OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for NR (not for MR-DC). The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

rrcSegmentationReconfigResume Indicates that the UE supports segmentation of RRCReconfiguration and RRCResume messages, i.e., that the network may use the field rrcReconfigurationSegment in the RRCReconfiguration message and the field rrcResumeSegment in the RRCResume message.

NEXT CHANGE – Same as in R2-1907328

### 6.3.4	Other information elements

#### === Unmodified sections omitted ===

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type and together with the segmentNumber (if the message contains an RRCSegment), for the identification of an RRC procedure (transaction). If the RRC message contains an RRCSegment, the RRC-TransactionIdentifier shall be set to the same value in all segments of the same information element.

RRC-TransactionIdentifier information element

-- ASN1START

-- TAG-RRC-TRANSACTIONIDENTIFIER-START

RRC-TransactionIdentifier ::=       INTEGER (0..3)

-- TAG-RRC-TRANSACTIONIDENTIFIER-STOP

-- ASN1STOP

END OF CHANGES