3GPP TSG-RAN WG2 Meeting #106	R2-1907337

# Reno, USA, 13th - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0120 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DL BWP restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current RAN1 feature list R1-1905916 does not specify any UL BWP restriction. This may seem obvious for FDD since there is no way UL BWP can include initial DL BWP and SSB. UL BWP should have the full freedom in frequency location and BW with UE mandatory support. For TDD, however, paired UL BWP and DL BWP need to share the same center frequency. From 38.306 it is not clear that UL BWP is not impacted. Since there is no UL BWP restriction defined in R1-1905916, it is proposed to clarify this in 38.306. Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: BWP Inter-operability: If the network implements the CR and the UE does not, there is no inter-operability issue, since there should be no restriction for UL BWP, even though not clear from 38.306. If the UE implements the CR and the network does not, there is no inter-operability issue, since there should be no restriction for UL BWP, even though not clear from 38.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 4.2.7.2 BandNR parameters - Include “DL” on BWP description of fields bwp-DiffNumerology, bwp-SameNumerology and bwp-WithoutRestriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no clear indication in 38.306 that BWP restriction is not aplicable to UL BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 4.2.7.2	BandNR parameters

<<<<<<<<<<<<<<<<<<<<<<< Omitted unchanged parts >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

| bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | No | No |
| --- | --- | --- | --- | --- |
| bwp-SameNumerology Defines type A/B BWP adaptation (up to 2/4 BWPs) with the same numerology, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | No | No |
| bwp-WithoutRestriction Indicates support of BWP operation without bandwidth restriction. The Bandwidth restriction in terms of DL BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of initial DL BWP and SSB. For SCell(s), it means that the bandwidth of DL BWP may not include SSB. | Band | No | No | No |

END OF FIRST CHANGE