3GPP TSG-RAN2 Meeting #106	 R2-1907411

# Reno, USA, 13 – 17 May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1083 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to delta configurations of security in RadioBearerConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RadioBearerConfig, the fields securityConfig, keyToUse and securityAlgorithmConfig are optional. It is stated in the field descriptions e.g. “bearers configured with the list in this radioBearerConfig”. However, this refers to just one specific field (radioBearerConfig) of the type (IE) RadioBearerConfig. However, multiple fields are of the same type (IE) RadioBearerConfig, namely radioBearerConfig in 38.331, but also nr-RadioBearerConfig1 and nr-RadioBearerConfig2 in 36.331. Consequently, these field descriptions should instead refer to the IE name, rather than a field name, i.e. “bearers configured with the list in this RadioBearerConfig”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated field descriptions of RadioBearerConfig to refer to the IE RadioBearerConfig instead of the field radioBearerConfig, to make the field descriptions valid also for EN-DC. Impact analysis Impacted functionality: - EN-DC Inter-operability: If the network is implemented according to this CR while the UE is not, the UE may fail to apply correct security configurations for all listed DRBs if the network uses delta configurations. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE in EN-DC may not apply delta configurations for the security configurations for all listed DRBs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 RadioBearerConfig |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR was agreed in principle in R2-1905365. |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

START OF CHANGES

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIOBEARERCONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...

}

SRB-ToAddModList ::=                    SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=                        SEQUENCE {

srb-Identity                            SRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

discardOnPDCP                           ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),                                                -- EPS-DRB-Setup

sdap-Config                             SDAP-Config                                                     -- 5GC

}                                                                                               OPTIONAL,   -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

DRB-ToReleaseList ::=                   SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=                      SEQUENCE {

securityAlgorithmConfig                 SecurityAlgorithmConfig                                 OPTIONAL,   -- Cond RBTermChange

keyToUse                                ENUMERATED{master, secondary}                           OPTIONAL,   -- Cond RBTermChange

...

}

-- TAG-RADIOBEARERCONFIG-STOP

-- ASN1STOP

DRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured.

recoverPDCP Indicates that PDCP should perform recovery according to TS 38.323 [5].

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included after AS security has been activated, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig except for mobility from NR to E-UTRA/5GC. The field is not included when configuring SRB1 before AS security is activated.

srb3-ToRelease Release SRB3. SRB3 release can only be done over SRB1 and only at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this IE RadioBearerConfig are using the master key or the secondary key for deriving ciphering and/or integrity protection keys. For EN-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this IE RadioBearerConfig except for mobility from NR to E-UTRA/5GC.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this IE RadioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this IE RadioBearerConfig except for mobility from NR to E-UTRA/5GC.

SRB-ToAddMod field descriptions

discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS 38.323 [5].

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to reconfiguration with sync, for SRB2 when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment in NR. For LTE SRBs using NR PDCP, it could be for handover, RRC connection reestablishment or resume.

srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only.

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of set up of signalling and data radio bearer and change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB is being setup or corresponding RB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| HO-Conn | The field is mandatory present in case of inter-system handover from E-UTRA (connected to EPC) to E-UTRA (connected to 5GC) or NR, or when the fullConfig is included in the RRCReconfiguration message, or in case of RRCSetup; otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | The field is mandatory present in case of inter-system handover from E-UTRA (connected to EPC) to E-UTRA (connected to 5GC) or NR, or when the fullConfig is included in the RRCReconfiguration message. In case of RRCSetup, the field is not present; otherwise the field is optionally present, need N. |

END OF CHANGES