3GPP TSG-RAN2 Meeting #106	R2-1907473

# Reno, Nevada, US, 13th – 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1450 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Length field for DPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IoT-Core |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The L-field indicates the length of the corresponding MAC SDU or variable-sized MAC control element in bytes. For the case of DPR transmission there is one MAC subheader which is applicable for both a CCCH MAC SDU and the DPR MAC CE. It may be ambiguous which of these two parts the L-field indicates. The intention is that the L-field shall indicate only the size of the CCCH MAC SDU, but not the DPR MAC CE since the DPR MAC CE is neither a “MAC SDU” nor a “variable-sized MAC control element”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified how the CCCH MAC SDU Length field is calculated in case DPR is transmitted. Impact analysis Impacted functionality: DPR Inter-operability: There are no interoperability issues foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may be ambigious how the L-field is set for the case of DPR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.3.10	Data Volume and Power Headroom Report MAC Control Element

The Data Volume and Power Headroom Report (DPR) MAC control element is identified by the MAC PDU subheader used for the CCCH MAC SDU, as specified in table 6.2.1-2. It does not add any additional subheader and is always placed before the CCCH MAC SDU. If the MAC PDU subheader used for the CCCH MAC SDU includes an L-field, the DPR MAC control element is not included for the calculation of L-field.

It has a fixed size and consists of a single octet defined as follows (figures 6.1.3.10-1 and 6.1.3.10-1a):

-	Data Volume (DV): The Data Volume field identifies the total amount of data available across all logical channels and of data not yet associated with a logical channel after all MAC PDUs for the TTI have been built. The amount of data is indicated in number of bytes. It shall include all data that is available for transmission in the RLC layer, in the PDCP layer, and in the RRC layer; the definition of what data shall be considered as available for transmission is specified in TS 36.322 [3], TS 36.323 [4] and TS 36.331 [8] respectively. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field is 4 bits. The values taken by the Data Volume field are shown in Table 6.1.3.10-1;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 2 bits or 4 bits. The reported PH and the corresponding power headroom and extended power headroom levels are shown in Table 6.1.3.10-2 and Table 6.1.3.10-2a, respectively, below (the corresponding measured values in dB can be found in TS 36.133 [9]);

-	R: reserved bit, set to "0".

Figure 6.1.3.10-1: Data Volume and Power Headroom Report MAC control element

Figure 6.1.3.10-1a: Data Volume and Power Headroom Report MAC control element for Extended Power Headroom level reporting

Table 6.1.3.10-1: Data Volume levels for DV

| Index | Data Volume (DV) value [bytes] | Index | Data Volume (DV) value [bytes] |
| --- | --- | --- | --- |
| 0 | DV = 0 | 8 | 67 < DV <= 91 |
| 1 | 0 < DV <= 10 | 9 | 91 < DV <= 125 |
| 2 | 10 < DV <= 14 | 10 | 125 < DV <= 171 |
| 3 | 14 < DV <= 19 | 11 | 171 < DV <= 234 |
| 4 | 19 < DV <= 26 | 12 | 234 < DV <= 321 |
| 5 | 26 < DV <= 36 | 13 | 321 < DV <= 768 |
| 6 | 36 < DV <= 49 | 14 | 768 < DV <= 1500 |
| 7 | 49 < DV <= 67 | 15 | DV > 1500 |

Table 6.1.3.10-2: Power Headroom levels for PH

| PH | Power Headroom Level |
| --- | --- |
| 0 | POWER_HEADROOM_0 |
| 1 | POWER_HEADROOM_1 |
| 2 | POWER_HEADROOM_2 |
| 3 | POWER_HEADROOM_3 |

Table 6.1.3.10-2a: Extended Power Headroom levels for PH

| PH | Extended Power Headroom Level |
| --- | --- |
| 0 | EXTENDED_POWER_HEADROOM_0 |
| 1 | EXTENDED_POWER_HEADROOM_1 |
| 2 | EXTENDED_POWER_HEADROOM_2 |
| 3 | EXTENDED_POWER_HEADROOM_3 |
| 4 | EXTENDED_POWER_HEADROOM_4 |
| 5 | EXTENDED_POWER_HEADROOM_5 |
| 6 | EXTENDED_POWER_HEADROOM_6 |
| 7 | EXTENDED_POWER_HEADROOM_7 |
| 8 | EXTENDED_POWER_HEADROOM_8 |
| 9 | EXTENDED_POWER_HEADROOM_9 |
| 10 | EXTENDED_POWER_HEADROOM_10 |
| 11 | EXTENDED_POWER_HEADROOM_11 |
| 12 | EXTENDED_POWER_HEADROOM_12 |
| 13 | EXTENDED_POWER_HEADROOM_13 |
| 14 | EXTENDED_POWER_HEADROOM_14 |
| 15 | EXTENDED_POWER_HEADROOM_15 |