3GPP TSG-RAN WG2 Meeting #106	R2-1907475

Reno, USA, 13th – 17th May 2019

Agenda Item:	12.2.8

Source:	Ericsson

Title:	Adding new system information on IRAT cell selection for NB-IoT

Document for:	Discussion, Decision

# Introduction

In order to prepare a NB-IoT device for inter-RAT cell selection, the UE needs to be configured with supported radio access technologies. Cell selection occurs, e.g., at power on. Hence, the information needs to made available already with system information. Whether this information is added to existing SIB or to a new SIB is debatable; arguments are presented in the accompanying contribution. In this contribution we discuss the if a new SIB is needed or if existing SIBs are good enough.

# Discussion

The objective with this activity is to introduce NB-IoT functionality

System information is broadcasted (SIB) to RRC_IDLE UEs comprising essential information on how the UE is to function in a particular cell. There is a number of SIBs that each includes certain pieces of information needed for the UE to operate in a cell.

Looking at LTE SIBs [3] for cell (re)selection, there are some SIBs of interest for this particular case, namely SIB3,  SIB5 and SIB7. SIB3 contains cell reselection information, mainly about the serving cell. SIB5 contains information on other E-UTRAN frequencies relevant for cell re-selection as well cell reselection parameters. Finally, SIB7 contains frequencies for GERAN.

Focussing only on NB SIBs the available SIBs (SIB3 and SIB5) only provide information on inter-frequency and intra-frequency. However, there is no information on other radio access technologies.

What we need is a NB SIB (or SIBs), i.e. the device is primarily a NB-IoT device, that provides the information given in the three LTE SIBs. There is possibility to add the information in existing NB SIBs such as SIB3 and SIB5. However, this will cause the SIB size to be big and that may not be desired. It would cause increase of legacy UE power consumption and initial access time, and also can impact the network capacity. A new SIB to host Inter-RAT cell selection information is more apt.

Using existing SIBs for Inter-RAT cell selection may cause the SIB size to increase which will have other consequences such as network capacity and increase in UE power to read large SIBs.

Add a new SIB to host inter-RAT cell selection information.

# Conclusion

With respect to the discussion in the previous section we make the following observations.

- Using existing SIBs for Inter-RAT cell selection may cause the SIB size to increase which will have other consequences such as network capacity and increase in UE power to read large SIBs.

With respect to the discussion in the previous section we make the following proposals.

Proposal 1	Add a new SIB to host inter-RAT cell selection information.

# References

RP-190770. Revised WID: Additional MTC enhancements for LTE 3GPP TSG RAN meeting #83 Shenzhen, China, March 18-21, 2019

RP-190757 WID revision: Additional enhancements for NB-IoT, 3GPP TSG RAN meeting #83, Shenzhen, China, March 18-21, 2019	(revision of RP-182902)

TS 36.300 Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 15)