3GPP TSG-RAN WG2#106	R2-1907547

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | DraftCR | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fast MCG recovery in (NG)EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-05-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of the Rel-16 WI on CA & DC enhancements is to add support for fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. In RAN2#105bis the first agreements were taken for fast MCG recovery: 0. MCG fast recovery targets all MRDC architecture options 1. When MCG failure occurs, UE follows SCG failure-like procedure: i. UE does not trigger RRC connection re-establishment. ii. UE triggers an MCG failure procedure in which a failure information message is transmitted to the network via SCG. 2. MCG fast recovery targets the following use cases: MCG leg RLF 3. MCG fast recovery can only be triggered after AS security has been activated and the SRB2 and at least one DRB have been setup 4. MCG failure indication should include: i. Available measurement results of MCG ii. MCG link failure cause iii. Available measurement results of SCG iv. Available measurement results of non-serving cells 5. For MCG failure indication, new RRC message in introduced, e.g. MCGFailureInformation. 6. SCG leg of the split SRB1 can be used for MCG fast recovery. 7. New SRB is not introduced for MCG fast recovery. This CR illustrates changes needed to cover above agreements and further functionality proposed in R2-1905986 and R2-1905990. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.6.x is added to introduce the necessary functionality to report MCG failure information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item objective is not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.x MCG failure information |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 37.340, TS 38.331 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.6.x	MCG failure information

#### 5.6.x.1	 General

Figure 5.7.x.x-x: MCG failure information

The purpose of this procedure is to inform E-UTRAN about an MCG failure the UE has experienced i.e. MCG radio link failure, or failure of MCG reconfiguration with sync.

#### 5.6.x.2 Initiation

If MCG transmissions are not already suspended, a UE initiates the procedure to report MCG failures when SCG transmission is not suspended and when the following condition is met:

1>  upon detecting radio link failure of the MCG, in accordance with 5.3.11;

Upon initiating the procedure, the UE shall:

1>	suspend MCG transmission for all SRBs and DRBs;

1>	reset MCG-MAC;

1>	stop T304, if running;

#### 5.6.x.3 Actions related to transmission of MCGFailureInformation message

The UE shall set the contents of the MCGFailureInformation message as follows:

1>	if the UE initiates transmission of the MCGFailureInformation message to provide MCG radio link failure information:

2>	include failureType and set it to the trigger for detecting MCG radio link failure;

1>	set the measResultListServFreqList to include for each EUTRA MCG cell that is configured, if any, within measResultCell the quantities of the concerned SCell, if available according to performance requirements in TS 36.133 [16];

1>	for each EUTRA MCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving EUTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

1>	if SRB1 is configured as Split SRB;

2>	if the primaryPath is associated with the MCG and pdcp-Duplication is not configured in accordance with TS 38.331 [82, 6.3.2];

3> the UE shall change the primaryPath to SCG;

The UE shall submit the MCGFailureInformation message to lower layers for transmission.

END OF CHANGES