3GPP TSG-RAN WG2#106	R2-1907548

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | DraftCR | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fast MCG recovery via SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-05-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of the Rel-16 WI on CA & DC enhancements is to add support for fast recovery of MCG link. One way of achieving this is to utilise the SUL for transmission of the failure information message. See R2-1905993 for more detailed motivation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sections 5.3.10.3 and 5.7.5 are extended to support the reporting of random access problem or RLC failure via Failure Information message for UE configured with SUL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3, 5.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2> else if the indication is from MCG RLC or random access problem from MCG MAC, and the serving cell is configured with a supplementary uplink:

3>	select the SUL carrier for uplink transmission;

3> initiate the failure information procedure as specified in 5.7.5 to report random access problem or RLC failure;

2>	else:

3>	consider radio link failure to be detected for the MCG i.e. RLF;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

3>	else if AS security has been activated but SRB2 and at least one DRB have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from SCG RLC and CA duplication is configured and activated; and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

END OF CHANGES

START OF CHANGES

### 5.7.5	Failure information

#### 5.7.5.1	General

Figure 5.7.5.1-1: Failure information

The purpose of this procedure is to inform the network about a failure detected by the UE.

#### 5.7.5.2	Initiation

A UE initiates the procedure when there is a need inform the network about a failure detected by the UE. In particular, the UE initiates the procedure when the following condition is met:

1>	upon detecting failure for an RLC bearer or random access problem on MCG, in accordance with 5.3.10.3;

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the FailureInformation message as specified in 5.7.5.3;

#### 5.7.5.3	Actions related to transmission of FailureInformation message

The UE shall:

1>	if initiated to provide RLC failure information:

2>	set logicalChannelIdentity to the logical channel identity of the failing RLC bearer;

2>	set cellGroupId to the cell group identity of the failing RLC bearer;

2>	set failureType to the type of failure that was detected;

1>	if initiated to provide random access problem information:

2>	set cellGroupId to MCG;

2>	set failureType to the type of failure that was detected;

1>	if used to inform the network about a failure for an MCG RLC bearer or random access problem on MCG:

2>	submit the FailureInformation message to lower layers for transmission via SRB1;

1>	else if used to inform the network about a failure for an SCG RLC bearer: and if the UE is in EN-DC:

2>	if SRB3 is configured;

3>	submit the FailureInformation message to lower layers for transmission via SRB3;

2>	else;

3>	submit the FailureInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

END OF CHANGES