3GPP TSG-RAN WG2 #106	R2-1907577

Reno, USA, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0273 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the PDCP structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 36.331, the PDCP entity for the split bearer can be associated with two UM RLC entities for the same direction, and the PDCP entity configured with PDCP duplication can be associated with two RLC UM entities for the same direction. However, according to the TS 36.323, for the split bearer and radio bearer configured with PDCP duplication, only two AM RLC entities and four UM RLC entities can be mapped with the PDCP entity. In order to align the specification between TS 36.331 and TS 36.323, the current text should be updated. In addition, to enhance the readability, this CR tries to align with Rapporteur CR (R2-1905231), which was submitted in NR PDCP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that each PDCP entity for split bearer and radio bearer configured with PDCP duplication is assocated with two RLC UM entities (for same direction). Impact analysis Impacted functionality: Impacted functionality: Description of PDCP structure Inter-operability: If only the network is implemented according to the CR, no interoperability problems are foreseen. If only the UE is implemented according to the CR, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS 36.331 on the configuration of PDCP entity for the split bearer and the radio bearer configured with PDCP duplication remains. Furthermore, description will not be aligned with Rapporteur CR (R2-1905231) for NR PDCP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.1	PDCP structure

Figure 4.2.1.1 represents one possible structure for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 36.300 [2].

Figure 4.2.1.1 - PDCP layer, structure view

Each RB (i.e. DRB, SLRB and SRB, except for SRB0 and SRB1bis) is associated with one PDCP entity. Each PDCP entity is associated with one, two, or four (e.g uni-directional/bi-directional or split/non-split) RLC entities depending on the RB characteristic (i.e. uni-directional or bi-directional) or RLC mode:

- For split bearers or for RBs configured with PDCP duplication, each PDCP entity is associated with two (bi-directional) RLC AM entities, two (for same direction) RLC UM entities or four (uni-directional) RLC UM entities.

- For LWA bearers, each PDCP entity is associated with one (bi-directional) RLC AM entity or two (uni-directional) RLC UM entities and the LWAAP entity. The PDCP entities are located in the PDCP sublayer.

- Otherwise, each PDCP entity is associated with one UM RLC entity, two UM RLC entities (one for each direction), or one AM RLC entity (bi-directional).

The PDCP entities are located in the PDCP sublayer. The PDCP sublayer is configured by upper layers, see TS 36.331 [3].