3GPP TSG-RAN WG2 #106	Tdoc R2-1907599

Reno, US, 13 – 17 May 2019	(Revision of R2-1904741)

Title:	Draft LS to RAN1 on the need for COT sharing between RA msg2 and msg3

Response to:

Release:	Release 16

Work Item:	NR_unlic-Core

Source:	Ericsson (to be RAN2)

To:	RAN1

Cc:

Contact Person:

#### Name:                   Robert Karlsson

E-mail Address:	robert.s.karlsson@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 has discussed COT sharing for NR-U. The intention is to avoid unnecessary LBT operations for subsequent transmission burst within a COT period, which is initiated by a gNB.

RAN2 has discussed the possibility to allow a UE to transmit a Msg3 within a COT initiated by the gNB with the transmission of Msg2.

In this case, the UE may avoid the ordinary Category 4 LBT for the Msg3 transmission if the switch gap between DL and UL is sufficiently short. This is beneficial to reduce latency for Msg3 due to avoidance of LBT.

Since the gNB is in control of DL and UL transmissions, when required, it can ensure the gap between the end of the DL transmission to the beginning of the Msg3 transmission to be sufficiently short. For example, the gNB may schedule other transmissions to other UEs that occupy the channel before the intended transmission time for Msg3 resulting in a gap sufficiently short for the intended UE.

In the COT sharing scenario, if the gap is equal or shorter than 16 sec, the UE can apply an immediate transmission for Msg3 skipping LBT operation; while if the gap is not greater than 25 sec, the UE applies a Cat2 LBT for a Msg3 transmission. If the gNB do not have other DL transmissions to schedule, it may schedule the UE to perform a Category 4 LBT for the transmission of a Msg3.

RAN2 would like to ask RAN1 to support COT sharing between msg2 and msg3, and how the UE will know what type of channel access to use.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take below actions

- To take the above analyses into account to support the gNB to schedule a UE to transmit a Msg3 within a COT initiated by the gNB with the transmission of Msg2.

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#107		26 - 30 Aug 2019		Prague, Czech Republic

3GPPRAN2#107bis		14 - 18 Oct 2019		China