3GPP TSG-RAN WG2 Meeting #106	R2-1907758

Reno, US, 13-17 May 2019

Agenda item:	11.8.2.1

Source: 	Huawei, HiSilicon

Title: 	Considerations on the framework for RAT-dependent positioning method

Document for:	Discussion and Decision

Introduction

In RAN1 AH#1901 meeting [1], the following agreements about NR supporting positioning techniques were reached:

Agreement: The following candidate techniques are considered for study of DL positioning: - Timing based techniques - Timing of arrival path(s) - Phase difference based techniques - Note: feasibility needs to be further assessed - Angle-based techniques - Downlink angle(s) of departure - Downlink angle(s) of arrival - Carrier-phase based techniques - Note: feasibility needs to be further assessed - Received reference signal power based techniques - Cell ID and TRP related information (e.g. RS resource and/or resource set ID) Agreement: The following candidate techniques are considered for study of UL positioning: - Timing based techniques - Timing of arrival path(s) - Angle-based techniques - Uplink angle(s) of departure - Uplink angle(s) of arrival - Carrier-phase based techniques - Note: feasibility needs to be further assessed - Received reference signal power based techniques Agreement: The following candidate techniques are considered for study of DL and UL positioning: - Timing based techniques - Round trip time measurement including support for multiple TRPs - Combination of DL and UL techniques for NR positioning - e.g. E-CID like techniques (including one or multiple cells)

According to the WID of NR positioning in [2], one objective of RAN2 is to functional interfaces, signalling and procedures including UE reporting, to support NR RAT-dependent positioning for the NR positioning techniques listed in RAN1 objectives. In the last RAN2 meeting, there were some discussions about the framework of RAT-dependent positioning techniques in NR, however, no conclusion was made. In this contribution, we provide our consideration on the framework for RAT-dependent positioning method in NR.

Discussion

In Rel-15 LTE, RAT-dependent positioning techniques include OTDOA, UTDOA and E-CID. OTDOA is a downlink based positioning techniques and needs UE to measure RSTD of PRS from different TPs. UTDOA is a uplink based positioning technique and needs LMU to measure UL relative time of arrival. E-CID is a downlink and uplink based positioning technique and both UE and eNB could perform related measurements. The following table shows the RAT-dependent positioning techniques framework in LTE.

Table 1 Mapping between LTE positioning techniques and RS/measurements

| Positioning method | Positioning technique | Reference signal | UE measurement | Network measurement |
| --- | --- | --- | --- | --- |
| Downlink positioning method | OTDOA | DL PRS | Downlink timing measurements | NA |
| Uplink positioning method | UTDOA | UL SRS | NA | LMU: UL Difference Time of Arrival |
| E-CID positioning method | Downlink E-CID |  | - RSRP - RSRQ - UE Rx-Tx time difference | NA |
|  | Uplink E-CID |  | - NA | - eNB Rx – Tx time difference - Timing Advance - Angle of Arrival |

According to the agreements of RAN1, RAT-dependent positioning techniques in NR includes DL-TDOA, DL-AoD, UL-TDOA, UL-AoA, multi-cell RTT and E-CID. The following table show the mapping between NR positioning techniques and RS/measurement, which is referring to [3].

Table 2 Mapping between NR positioning techniques and RS/measurements

| Positioning technique | Reference signal | UE measurement | Network measurement |
| --- | --- | --- | --- |
| DL-TDOA | DL PRS | RSTD | NA |
| DL-AoD | DL PRS | RSRP | NA |
| UL-TDOA | UL SRS | NA | UL-RTOA |
| UL-AoA | UL SRS | NA | AoA, ZoA |
| Multi-cell RTT | DL PRS, UL SRS | UE Rx-Tx time difference | gNB Rx-Tx time difference |
| E-CID | Not specified | - RSRP - RSRQ - UE Rx-Tx time difference | - gNB Rx – Tx time difference - Angle of Arrival |

According to this table, both DL-TDOA and DL-AoD estimate UE’s location based on UE’s measurement on DL PRS, and the difference is that UE measure RSTD for DL-TDOA but RSRP for DL-AoD. Both UL-TDOA and UL-AoA needs the transmission of UE SRS and network performing measurement but with different measurement objective. To minimize the work of the specification, it is proposed to define downlink-only positioning method to cover DL-TDOA and DL-AoD, and define uplink-only positioning method to cover UL-TDOA and UL-AoA.

Proposal 1: Define downlink-only positioning method in stage-2 spec to cover DL-TDOA and DL-AoD.

Proposal 2: Define uplink-only positioning method in stage-2 spec to cover UL-TDOA and UL-AoA.

Multi-cell RTT needs not only DL PRS based measurement but also SRS based measurement, which is combination of DL & UL based positioning. Although the needed procedures to support multi-cell RTT may be same to downlink only based positioning procedure plus uplink based positioning procedure, it is necessary to define multi-cell RTT as a separately positioning method in stage-2 spec to show every positioning technique more clearly.

Proposal 3: Define multi-cell RTT positioning as a separate positioning method in stage-2 spec.

For E-CID, although it includes uplink E-CID and downlink E-CID, it can be entirely based the reference signals within a cell. No LPP or NRPP specification supported is needed in terms of RS configuration. This is different from other positioning techniques. So, it is proposed to define E-CID as a positioning method like LTE.

Proposal 4: Define E-CID as a separate positioning method in stage-2 spec to cover uplink E-CID and downlink E-CID like in LTE.

Conclusion

In this contribution, we provided our consideration on RAT-dependent positioning method in NR, and we get the following observation and proposals:

Proposal 1: Define downlink-only positioning method in stage-2 spec to cover DL-TDOA and DL-AoD.

Proposal 2: Define uplink-only positioning method in stage-2 spec to cover UL-TDOA and UL-AoA.

Proposal 3: Define multi-cell RTT positioning as a separate positioning method in stage-2 spec.

Proposal 4: Define E-CID as a separate positioning method in stage-2 spec to cover uplink E-CID and downlink E-CID like in LTE.

References

[1] Chairman’s Notes of RAN1 AdHoc 1901.

[2] RP-190752, New WID: NR Positioning Support Chairman’s Notes

[3] R1-190xxxx NR positioning measurement