3GPP TSG-RAN WG2 Meeting #106	R2-1907762

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	11.8.2

Source: 	Huawei, HiSilicon

Title: 	Considerations on E-CID positioning in NR

Document for:	Discussion and Decision

Introduction

In RAN1#95 [1], the following candidate techniques were agreed for study of downlink and uplink positioning.

Agreement: The following candidate techniques are considered for study of DL and UL positioning: - Timing based techniques - Round trip time measurement including support for multiple TRPs - Combination of DL and UL techniques for NR positioning - e.g. E-CID like techniques (including one or multiple cells)

In RAN1 #AH1901 meeting, the following agreement has been made on the additional aspects to consider.

Agreement: - Additional aspects that have been proposed during the study: - Receive signal waveform reporting - UE-based positioning - Multipath measurements - SFN transmissions for positioning - Techniques to lower UE complexity for OTDOA - Broadcast of assistance data - LMUs

In this contribution, we provide our consideration on E-CID in NR from RAN2 perspective.

E-CID

In the legacy E-CID in LTE and R15, two types of T_ADV have been defined:

- Type1 introduced in Rel-9: T_ADV = (eNB Rx – Tx time difference) + (UE Rx – Tx time difference)

- Type2 introduced in Rel-8: T_ADV = eNB Rx – Tx time difference

Correspondingly, the UE Rx-Tx time difference has been defined in RRC spec and LPP spec in Rel-8 and Rel-9, respectively.

TS 36.331:

TS 36.355:

While in the LPPa spec, the TADV result obtained by the eNB is transmitted to the E-SMLC in the LPPa message, as specified in TS 36.455:

As one of the most important positioning techniques and supported both in LTE and R15 NR, there is no reason for us to exclude it from the study of R16 positioning.

Proposal 1: The procedure in TS 36.305 for E-CID can be adopted for the E-CID positioning in R16 as baseline.

Proposal 2: UE could report beam index to LMF besides Rx-Tx time to assist downlink E-CID positioning.

Conclusion

In this contribution, we considered our consideration on E-CID in NR from RAN2 perspective, and we get the following observation and proposals:

Proposal 1: The procedure in TS 36.305 for E-CID can be adopted for the E-CID positioning in R16 as baseline.

Proposal 2: UE could report beam index to LMF besides Rx-Tx time to assist downlink E-CID positioning.

References

[1] Chairman’s Notes of RAN1 AdHoc 1901.

TP for Downlink E-CID in TS 38.305

- X.Y.Z	Downlink E-CID Positioning Procedures

The procedures described in this subclause support E-CID related measurements obtained by the UE and provided to the E-SMLC using LPP. The term "downlink" is intended to indicate that from the E-SMLC perspective the involved measurements are provided by the UE; this set of procedures might also be considered as "UE-assisted, E-SMLC-based E-CID".

- X.Y.Z.1	Capability Transfer Procedure

The Capability Transfer procedure for E-CID positioning is described in subclause 7.1.2.1.

X.Y.Z.1.1	Void

- X.Y.Z.2	Assistance Data Delivery Procedure

Assistance data delivery is not required for UE- or eNB-assisted forms of E-CID positioning.

- X.Y.Z.3	Location Information Transfer Procedure

The purpose of this procedure is to enable the E-SMLC to request position measurements from the UE, or to enable the UE to provide location measurements to the E-SMLC for position calculation (e.g., in case of basic self location where the UE requests its own location).

X.Y.Z.3.1	E-SMLC-initiated Location Information Transfer

Figure 8.3.3.3-1 shows the Location Information Transfer operations for the E-CID method when the procedure is initiated by the E-SMLC.

Figure 8.3.3.3-1: E-SMLC-initiated Location Information Transfer Procedure.

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of E-CID positioning. This request includes the E-CID measurements requested by the E-SMLC and supported by the UE as listed in Table 8.3.2.2-1 together with a required response time.

(2)	The UE performs the requested measurements and sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step 1 elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

X.Y.Z.3.2	UE-initiated Location Information Delivery procedure

Figure 8.3.3.3.2-1 shows the Location Information Delivery procedure operations for the E-CID method when the procedure is initiated by the UE.

Figure 8.3.3.3.2-1: UE-initiated Location Information Delivery Procedure.

(1)	The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include any UE measurements already available at the UE.

TP for Uplink E-CID in TS 38.305

- X.Y.Z	Uplink E-CID Positioning Procedures

The procedures described in this sub clause support E-CID related measurements obtained by the eNodeB and provided to the E-SMLC using LPPa. The term "uplink" is intended to indicate that from the E-SMLC point of view, the involved measurements are provided by the eNodeB; this set of procedures might also be considered as "eNB-assisted E-CID". An example of this uplink E-CID positioning method is AoA+TADV.

NOTE:	Uplink E-CID positioning not using TADV type 1 does not require LCS support in the UE and therefore, works also with legacy UEs (assuming that any involved measurements delivered in RRC are supported by the legacy UEs).

- X.Y.Z.1	Position Capability Transfer Procedure

The position capability transfer procedure is not applicable to uplink E-CID positioning not using TADV type 1. For uplink E-CID positioning using TADV type 1, the Capability Transfer procedure for E-CID positioning is described in subclause 7.1.2.1.

- X.Y.Z.2	Assistance Data Delivery Procedure

The assistance data delivery procedure is not applicable to uplink E-CID positioning.

- X.Y.Z.3	Position Measurement Procedure

The purpose of this procedure is to enable the E-SMLC to request position measurements from the eNodeB.

X.Y.Z.3.1	E-SMLC-initiated Position Measurement

Figure 8.3.4.3.1-1 shows the Position Measurement operations for the uplink E-CID method when the procedure is initiated by the E-SMLC.

Figure 8.3.4.3.1-1: E-SMLC-initiated Position Measurement Procedure

(1)	The E-SMLC sends an LPPa E-CID MEASUREMENT INITIATION REQUEST message to the eNodeB. This request includes indication of E-CID measurements requested and whether the result is expected only once or periodically.

(2)	If the E-SMLC in step (1) requested UE measurements (i.e., RSRP, RSRQ measurements), or if the eNodeB requires UE measurements associated with the measurements requested by the E-SMLC (e.g., TADV type 1, which requires a UE Tx-Rx time difference measurement to be delivered from the UE to the eNodeB), the eNodeB may configure the UE to report the measurement information requested as specified in TS 36.331 [14].

(3)	If the result is expected only once and the eNodeB initiates at least one of the E-CID measurements as requested, the eNodeB sends an LPPa E-CID MEASUREMENT INITIATION RESPONSE to the E-SMLC, which includes the obtained E-CID measurements. If the result is expected periodically and the eNodeB is able to initiate at least one of the E-CID measurements as requested, the eNodeB sends an LPPa E-CID MEASUREMENT INITIATION RESPONSE to the E-SMLC, which does not include any result. The eNodeB reports then the obtained measurements by initiating the E-CID Measurement Report procedure, with the requested periodicity. If the eNodeB is unable to initiate any of the requested measurements as requested from the E-SMLC, or is unable to instigate any of the required RRC procedures to obtain the requested measurements from the UE, the eNodeB sends an LPPa E-CID MEASUREMENT INITIATION FAILURE message providing the error reason. If the failure occurs during a periodic reporting, the eNodeB sends an LPPa E-CID MEASUREMENT FAILURE INDICATION message.