3GPP TSG-RAN WG2 Meeting #106	R2-1907780

Reno, US, 13-17 May 2019	Resubmission of R2-1904165

Agenda item:	11.6.4.1.1

Source: 	Huawei, HiSilicon

Title: 	Consideration on cell measurement in NTN

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss cell measurement in NTN.

Discussion

In terrestrial networks, mobility management is based on cell quality measurement. This is reasonable and efficient for NR since a clear near-far effect is expected in terrestrial networks. As depicted in Figure 1(a), a UE can approximately determine whether it is located in the centre or edge of the serving cell by measuring the value of RSRP. However, regarding the NTN context shown in Figure 1(b), the near-far effect is not clearly exhibited within a cell.

Figure 1. A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN

The unclear near-far effect in NTN is further verified by the path loss value shown in Table 1. For instance, in an NTN with a height of 600km and a cell range of 200km, the difference of free space loss between the cell centre and cell edge is only around 0.12dB. Considering a 3dB EIRP difference within a cell due to the antenna pattern, the overall differential signal power within a cell is around 3.12dB. Besides, the typical measured intra-frequency/inter-frequency RSRP accuracy requirements for FR1 under normal condition is ±4.5dB [1]. Therefore, it is very hard to distinguish where a UE is located in an NTN cell by pure received signal power based measurement. As such, cell boundary determination can be restricted to some extent or even ineffective sometimes.

Table 1: Path loss in different network scenarios

| Scenarios |  | LEO based NTN |  | GEO based NTN |
| --- | --- | --- | --- | --- |
| Altitude |  | 600 km | 1200 km | 35786 km |
| Beam footprint size |  | 200 km | 200 km | 500 km |
| Path loss | Cell centre | 154.05 dB | 160.05 dB | 206.74 dB |
|  | Cell edge | 154.17 dB | 160.09 dB | 206.75 dB |

Observation 1: the near-far effect is not very obvious in NTN compared with terrestrial network.

The current handover mechanism is based on the A3 measurement event. Generally, 3dB is needed for target cell to be higher than source gNB. In terrestrial networks, A3 event could be triggered in overlapping area of two cells during UE moving to the target gNB like Figure 2(a). However, it is difficult to trigger A3 event in overlapping area of two cells due to the lack of clear near-far effect in NTN like Figure 2(a). That means that A3 event based handover is difficult to be triggered in overlapping area of two cells.

Observation 2: A3 event based handover is difficult to be triggered in overlapping area of two cells in NTN.

Figure 1 A sketch of triggering handover in different scenarios: a) terrestrial network; b) NTN

According to observation 2, it is not suitable to continue to use A3 event as trigger condition of handover. One possible solution is to design a new measurement event to trigger handover. Another possible solution is to introduce new handover decision mechanism. For example, network could decide whether to trigger handover based on UE’s location and the footprint track of satellite. In a word, a new handover trigger mechanism should be study for NTN.

Proposal: capture the TP in Annex into TR.

Conclusion

In this contribution, we discussed cell measurement in NTN, and we get the following proposals:

Observation 1: the near-far effect is not very obvious in NTN compared with terrestrial network.

Observation 2: A3 event based handover is difficult to be triggered in overlapping area of two cells in NTN.

Proposal: capture the TP in Annex into TR.

References

[1] TS 38.133 V15.4.0, “Requirements for support of radio resource management”.

Appendix

- 7.3.2 	Connected mode mobility enhancements

- 7.3.2.x cell measurement issue

In terrestrial networks, mobility management is based on cell quality measurement. This is reasonable and efficient for NR since a clear near-far effect is expected in terrestrial networks. A UE can approximately determine whether it is located in the centre or edge of the serving cell by measuring the value of RSRP. However, the near-far effect is not clearly exhibited within a cell. The unclear near-far effect in NTN is further verified by the path loss value shown in Table 1. For instance, in an NTN with a height of 600km and a cell range of 200km, the difference of free space loss between the cell centre and cell edge is only around 0.12dB. Considering a 3dB EIRP difference within a cell due to the antenna pattern, the overall differential signal power within a cell is around 3.12dB. Besides, the typical measured intra-frequency/inter-frequency RSRP accuracy requirements for FR1 under normal condition is ±4.5dB. Therefore, it is very hard to distinguish where a UE is located in an NTN cell by pure received signal power based measurement. As such, cell boundary determination can be restricted to some extent or even ineffective sometimes.

Table 1: Path loss in different network scenarios

| Scenarios |  | LEO based NTN |  | GEO based NTN |
| --- | --- | --- | --- | --- |
| Altitude |  | 600 km | 1200 km | 35786 km |
| Beam footprint size |  | 200 km | 200 km | 500 km |
| Path loss | Cell centre | 154.05 dB | 160.05 dB | 206.74 dB |
|  | Cell edge | 154.17 dB | 160.09 dB | 206.75 dB |

The current handover mechanism is based on the A3 measurement event. Generally, 3dB is needed for target cell to be higher than source gNB. In terrestrial networks, A3 event could be triggered in overlapping area of two cells during UE moving to the target gNB like Figure 2(a). However, it is difficult to trigger A3 event in overlapping area of two cells due to the lack of clear near-far effect in NTN. That means that A3 event based handover is difficult to be triggered in overlapping area of two cells.