3GPP TSG-RAN WG2#106	R2- 1907855

# Reno, USA, 12th – 17th May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of handover procedure from EN-DC to NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed that inter-RAT handover from E-UTRA with EN-DC configuration to NR is supported in Rel 15. For operators deploy 5G network option3 and evolve quickly to option2, handover from EN-DC to NR can be a very common scenario. However, EN-DC to NR handover procedure is not included in 37.340. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.8 Master Node to eNB/gNB change - Addition of 10.8.x to include handover from EN-DC to NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of EN-DC to NR handover is unclear, especially the data forwarding procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 10.8.x	EN-DC to NR

The EN-DC to NR change procedure is used to transfer UE context data from a source eNB MN and En-gNB SN to a target gNB node.

Figure 10.8.x-1: EN-DC to gNB change procedure

Figure 10.8.x-1 shows an example signalling flow for EN-DC to gNB change procedure:

1. Step 1-15 from clause 4.11.1.2.2.2 (EPS to 5GS handover using N26 interface, preparation phase) in TS 23.502.

2. The source MME sends a Handover Command message to the source eNB. The Handover Command is constructed using the Target to Source transparent container. If indirect forwarding applies, the message also contains information of bearers subject to forwarding and forwarding addresses and TEID.

3. The source MN initiates the release of the source SN resources towards the source SN including a Cause indicating MCG mobility. The source SN acknowledges the release request. If data forwarding is needed, the MN provides data forwarding addresses to the source SN. Reception of the SgNB Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

4. The source MN triggers the UE to perform handover with MobilityFromEUTRACommand message and apply the new configuration. Upon receiving the new configuration, the UE releases the entire SCG configuration.

5/6. The UE synchronizes to the target gNB node and replies with RRCReconfigurationComplete message to the gNB.

7a.	The source SN sends the Secondary RAT Data Volume Report message to the source MN and includes the data volumes delivered to the UE over the NR radio for the related E-RABs.

7b.	The source MN sends a RAN Usage data Report (Secondary RAT usage data, handover flag) message to the source MME. The handover flag indicates to the MME that it should buffer the report before forwarding the Secondary RAT usage data.

8. For SN terminated bearers using RLC AM, the SN sends the SN Status transfer. The source MN sends the eNodeB Status Transfer message to the target gNB via the MME and AMF to convey the PDCP and HFN status of the E-RABs for which PDCP status preservation applies.

9. If applicable, data forwarding takes place from source MN and SN to target MN.

10.	Step 4-13 from clause 4.11.1.2.2.3 in TS 23.502 for handover from EPS to 5GS.

11.	Upon reception of the UE Context Release Command message from the MME (from clause 4.11.1.2.2.3 in TS 23.502, which is part of the handover execution phase), the source MN initiates UE Context Release towards the source SN, which releases radio and C-plane related resource associated to the UE context. Any ongoing data forwarding may continue.

END OF FIRST CHANGE