3GPP TSG-RAN WG2 Meeting #106			     R2-1907948

Reno, NV, USA, 13th – 17th May 2019				resubmission of R2-1905062

Agenda Item:	11.1.4 (NR_IAB-Core)

Source: 	LG Electronics Inc.

Title:         	Consideration on LCG space extension

Document for: 	Discussion and Decision

Introduction

As shown in IAB WID below, LCID space and potentially LCG space extension are considered for IAB. This contribution discusses LCG space extension and proposes our view.

- Extension of LCID space and potentially LCG space to support one-to-one mapping of UE bearers to BH RLC channels. The extension of LCID space and LCG space is applicable only to IAB-nodes.

Discussion

As indicated above WID, introducing extended LCID space for IAB is inevitable to support one-to-one bearer mapping and companies proposed how many LCID space should be extended on their contributions. According to [1~5], company’s proposal for the extended LCID space varies from 8bits to 16bits, i.e., additional 256 LCID ~ 65535 LCID space extension can be considered as a candidate.

Observation 1. For LCID extension in IAB, companies identify that 8bits ~ 16bits may be needed, i.e., additional 256 LCID ~ 65535 LCID space extension are considered.

Based on the observation 1 above, it should be analyzed whether the current LCG space is enough to support the extended LCIDs. In Rel-15 NR MAC, 8 LCGs can be configured to support 32 logical channels and buffer status per LCG is reported by a UE to a network using BSR MAC CE. The network can know the buffer status of each LCG after receiving BSR MAC CE and provide appropriate scheduling.

In this condition, however, although LCID space is extended, if LCG space is not extended, at least 256 logical channels should be split and included over 8 LCGs, i.e., one LCG may have logical channels more than 32. The problem is that Buffer size level defined in the current Rel-15 NR MAC specification would not be sufficient because the total buffer size of over 32 logical channels may be much higher than the maximum value of the current Buffer size level, e.g., 6bits for short BSR and 8bits for long BSR. If data volume in the BAP is considered for Buffer size level, it would be more serious.

Observation 2. If LCG space is not extended according to the extended LCID space, the Buffer size level defined in the current MAC specification would not be sufficient.

Of course, the increased Buffer size level for IAB may be defined without LCG extension, but this would not be the complete solution for the observation 2 because if the number of extended LCID is much larger than 256, i.e., 65535 LCID is possible by observation 1, and one LCG may have over 1000. Even though it sounds weird, it is possible with LCG extension by observation 1 and the Buffer size level should also increase drastically according to the number of logical channels included into one LCG. Thus, we think that LCG extension is better approach rather than increased Buffer size field. If LCG space is extended, each LCG may have logical channels as many as Rel-15 NR MAC has. The current Buffer size field and level can be reused without modification or update.

Furthermore, from QoS point of view, if logical channels having same or similar QoS are included into the same LCG, each Buffer size level of the BSR can indicate total data amount of each QoS. We think that it would be helpful to manage QoS requirement for transmission between two IAB nodes. Thus, LCG space should be extended as many as the extended LCID space as in current NR MAC.

Proposal 1. LCG space should be extended according to the extended LCID space.

One more important point is that considering the IAB topology as in below figure 1, an IAB node which is close to the edge (e.g., IAB node 3 in below figure 1) may need less LCID space than an IAB node which is close to the IAB donor node (e.g., IAB node 1b in below figure 1) because IAB node 1b in the figure 1 have to assign LCID to a logical channel for all DRBs coming from descendant UEs to support one-to-one bearer mapping.

Observation 3. The required extended LCID space is different according to the IAB node placement in the topology.

Figure 1. Example topology of IAB

As explained above, the required number of extended LCG space should be determined by the required number of extended LCID space because if the required LCID space is small but LCG is big, there should be unnecessary overhead. Thus, the number of extended LCG space can be different according to the IAB node placement in the topology. It would be good that the maximum number of LCG can be configurable according to the required extended LCID space and the IAB node placement in the topology. The BSR MAC CE format can be selected by the configured maximum number of LCG. For example, if 16 LCG is configured, the IAB node should use extended BSR MAC CE format for 16 LCG.

Proposal 2. The maximum number of LCG can be configurable according to the required extended LCID space.

Proposal

In this contribution, we discussed LCG space extension and present observations and proposals:

Observation 1. For LCID extension in IAB, companies identify that 8bits ~ 16bits may be needed, i.e., additional 256 LCID ~ 65535 LCID space extension are considered.

Observation 2. If LCG space is not extended according to the extended LCID space, the Buffer size level defined in the current MAC specification would not be sufficient.

Observation 3. The required extended LCID space is different according to the IAB node placement in the topology.

Proposal 1. LCG space should be extended according to the extended LCID space.

Proposal 2. The maximum number of LCG can be configurable according to the required extended LCID space.

- References

[1] R2-1901187	Some considerations on LCID space extension	Samsung

[2] R2-1901258	Discussion on MAC subheader structure for IAB	Nokia

[3] R2-1901329	LCID and LCG extension for IAB nodes	Ericsson

[4] R2-1901832	LCID extension for IAB backhaul link	Huawei

[5] R2-1905061	Consideration on LCID space extension and MAC format	LG Electronics Inc.