3GPP TSG-RAN WG2 Meeting #106	R2-1907981

# Reno, US, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1107 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reception of RRCReject with T302 upon RRC resume request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In C1-192607, the following change was agreed: When the UE in 5GMM-CONNECTED mode with RRC inactive indication has requested to resume, but the resume has failed, UE behaves based on the information received from the lower layer: - If NAS receives both resume failure indication and “access barring is applicable for all access categories except categories '0' and '2',” UE stays in 5GMM-CONNECTED mode with RRC inactive indication; - If NAS receives resume failure indication only, UE enters 5GMM-IDLE mode and then perform NAS recovery. To maintain AS-NAS synchronization, the UE RRC shall indicate NAS about access baring information if RRCReject is received in response to an RRCResumeRequest. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.15.2, add the following statement for the case that RRCReject is received in response to an RRCResumeRequest or RRCResumentRequest1: - inform the upper layer that access barring is applicable for all access categories except categories '0' and '2'.. Impact Analysis: Impacted 5G architecture option: NR standalone Impacted functionality: RRC inactive state procedure Inter-operability: - If the UE is implemented according to this CR but the network is not, the network and the UE may be out-of-sync on the UE state. - If the UE is implemented according to this CR but the network is not, the network and the UE may be out-of-sync on the UE state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are two possible consequences: - internally in the UE, AS may stay in RRC Inactive state but NAS may enter 5GMM-IDLE mode. - Between the UE and the network, the network may not understand the UE state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.15.2	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if running;

1>	stop timer T319, if running;

1>	stop timer T302, if running;

1>	reset MAC and release the default MAC Cell Group configuration;

1>	if waitTime is configured in the RRCReject:

2>	start timer T302, with the timer value set to the waitTime;

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if RRCReject is received in response to an RRCSetupRequest:

2>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

1>	else if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

2>	if resume is triggered by upper layers:

3>	inform upper layers about the failure to resume the RRC connection;

3> inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

2>	if resume is triggered due to an RNA update:

3>	set the variable pendingRnaUpdate to true;

2>	discard the current KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key derived in accordance with 5.3.13.3;

2>	suspend SRB1, upon which the procedure ends;

The RRC_INACTIVE UE shall continue to monitor paging while the timer T302 is running.