3GPP TSG-RAN WG2 Meeting #106					R2-1908035

Reno, USA,13th - 17th May 2019			(resubmission of R2-1905143)

Agenda item:	11.9.3.1

Source:	Samsung

Title:	Deconfiguration of the configuration for conditional handover in NR

Document for:	Discussion and Decision

Introduction

In RAN2#105, there was discussion on release method of CHO resource configuration. There were two ways from the companies on this. One is: Network only can always configure the release of the CHO resource. The other is: implicit way of using timer at the UE. Even most companies agree with the network based deconfiguration should be the baseline, but whether UE implicit way should also be the same important.

In this paper, we would like to note that NR has more pressure on maintaining more number of candidate target cells, and this obviously would be stronger motive for using UE implicit way in NR than LTe.

We discuss on this aspect.

Resource deconfiguration

One of important aspects is the resource reservation at the target node. CHO is basically different with legacy HO in terms of the HO execution time at UE, i.e., there could be an arbitrary time until the configured condition is met. During this time, it is obvious that the target node should reserve the resource which could be used by UE after CHO execution. And this resource should be tracked by the target node so that unlimited reservation be prohibited. One clear issue to be discussed is that how this reservation is traced.

Opt 1. Based on MR from the UE, and target node’s own request, the network re-/de-configure the CHO resource at any time. UE can execute CHO whenever the configuration is there.

Opt 2. During the timer running, CHO configuration is kept and network re-/de-configuration is not necessary. UE executes CHO to the cell among candidate cells meeting the condition at that time. After the timer expiry the network can re-/de-configure CHO configuration.

|  | Pros | Cons |
| --- | --- | --- |
| Opt 1 | - In principle, network should always have the control on the UE. - Radio condition of the target cell can be reflected to the CHO configuration by MR transmitted by UE. | - There could be increased X2 and Uu signalling for tracking the channel status and network negotiation, and finally configuring to UE. |
| Opt 2 | - It can reduce signalling overhead (Uu and X2) when multiple candidate target cells are configured for CHO. | Need effort to specify the timer operation at UE |

It seems that network has the control on the de-configuration is natural for CHO since the network can know the whole HO related situation i.e., source cell channel status as well as target cell link quality. However, regardless of channel condition either at source cell or the target cell, the resource reservation at the target node will be limited, and this would be timer based one at the target node too. Therefore, using timer at the UE can reduce the possible signalling to/from the target node.

Moreover, if the multiple candidate target cells are configured for CHO, reporting link quality change and determining the best target cell at the source node, and corresponding signalling between source and target cell for resource management per each candidate target cell could make the surge of the signalling not only at Uu but also X2. One simple way to resolve this is thought to use timer at the UE and multiple candidate target cells are configured for this timer duration. And there is no further CHO configuration for this duration, and UE only autonomously finds the best cell based on its measurements among condition-met-candidate cells in CHO configuration for CHO execution.

Observation 1. Using timer can reduce the signalling overhead normally and especially when multiple candidate target cells are configured for CHO and their link quality is frequently changed.

We have the estimation on the signalling overhead and the HO performance specific to the NR high frequency cases. The summary of that is [1]:

Table I. A5 (serving cell threshold: -87 dBm, and target cell threshold: -87 dBm) for Phase 2 event

|  | A4 entering condition threshold |  |
| --- | --- | --- |
|  | -87 dBm | -80 dBm |
| Handover failure rate | 1.31 % | 7.98 % |
| # of addition MR per sec | 3.47 | 1.11 |
| # of average candidate target cells | 4.58 | 3.19 |

Table II. A3 (offset value is 3 dB) and A5(serving cell threshold: -87 dBm, and target cell threshold: -87 dBm dB) for Phase 2 event

|  | A4 entering condition threshold |  |
| --- | --- | --- |
|  | -87 dBm | -80 dBm |
| Handover failure rate | 0.58 % | 3.69 % |
| # of addition MR per sec | 4.5 | 1.78 |
| # of average candidate target cells | 5.45 | 3.67 |

Based on the above observation and results of simulation, we have the following proposal.

Proposal 1. RAN2 adopts that CHO resource de-configuration using timer at the UE for the CHO especially in NR high frequency.

Conclusion

Based on the above discussion, we have the following proposal:

Proposal 1. RAN2 adopts that CHO resource de-configuration using timer at the UE for the CHO especially in NR high frequency.

References

[1] R2-1905141, NR specific simulation for the number of candidate cells in conditional handover, Samsung, RAN2#105b