3GPP TSG-RAN WG2 Meeting #106	R2-1908155

# Reno, USA, 13–17 May, 2019

Agenda item:	10.3.1

Source:	Samsung

Title:	Report of offline discussion 102 in RAN2#106

Document for:	Discussion and Agreement

1	Introduction

This contribution is to report outcome of offline discussion 102 during RAN2#106 meeting [1]:

R2-1908014 Configured grants overlapping with DL symbols in TDD operation Ericsson discussion Rel-15 NR_newRAT-Core R2-1907633 Handling of configured grants in TDD Samsung discussion Rel-15 NR_newRAT-Core … R2 confirm: The MAC entity should not generate a MAC PDU for configured grants overlapped with DL slots/symbols which are indicated by TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated. This seems to be according to the RAN1 specification and no TS change needed. Offline 102, can discuss if to capture something on dynamic TDD and or repetition (Samsung)

2	Conclusion

After having offline discussion, we can conclude to capture the following understanding in the meeting minutes which covers all the cases including gray-highlighted one above, and hence we can also remove it from the meeting minutes.

Proposal 1:	Add the following to the Chairman's minutes:
R2 confirms: The MAC entity should NOT generate a MAC PDU if a configured grant and all its repetitions (if any) are partially overlapped with downlink symbol/slots including the ones configured as 'D' by RRC or indicated as 'D' by SFI DCI (i.e. after processing SFI DCI), otherwise it may generate a MAC PDU, according to TS 38.213 subclause 11.1.1. No TS change is needed.

Proposal 2:	Remove the previous agreements under R2-1907633 from the Chairman's minutes.

3	References

[1]	RAN2-106-Notes NRUP IIOT IAB NRU Johan 190514 1300