3GPP TSG-RAN2 Meeting #106	R2-1908164

Reno, Nevada, USA, 13 May - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0133 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability signalling for FD-MIMO processing capabilities for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR is based on LS from RAN1 in R2-1908429/R1-1907628, section 1.1. Background: RAN1 has previously sent another LS to RAN2 (in R2-1905505/R1-1905577) indicating the need to add FD-MIMO capability signalling. As a result, RAN2 agreed CRs in R2-1908425 (TS 36.331) and R2-1908428 (TS 36.306). The CRs introduced the following parameters in UE-EUTRA-Capability on a per-UE basis: - relWeightTwoLayers-r13 - relWeightFourLayers-r13 - relWeightEightLayers-r13 - totalWeightedLayers-r13 Further, the CR also introduced the capability restriction inequality in TS 36.306. See the CRs indicated above for further details and signalling examples. Current change: The new LS is requesting to specify that the “Total number of weighted layers” can also be signalled in EN-DC band combination. It is also noted that this applies to only LTE part of EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of totalWeightedLayers in per BC capability signalling in CA-ParametersEUTRA. Impact Analysis: Impacted 5G architecture options: EN-DC Impacted Functionality: UE EUTRA capability for FD-MIMO Interoperability: - If the network implements the CR but the UE does not, no interoperability is foreseen as the UE will not include the FD-MIMO processing capabilities per band combination, i.e., legacy capabilities apply. - If the UE implements the CR but the network does not, UE may include the FD-MIMO processing capability per band combination that the network ignores, which means the network may wrongly overestimate the UE’s FD-MIMO processing capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 specification remains misaligned with RAN1 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.306 CR 1708 TS 36.331 CR 1114 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Equation 4.3.28.yy-1 in TS 36.306 is introduced by CR 1695. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.2.7.3	CA-ParametersEUTRA

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| additionalRx-Tx-PerformanceReq additionalRx-Tx-PerformanceReq defined in 4.3.5.22, TS 36.306 [15]. | BC | No | No | No |
| multipleTimingAdvance multipleTimingAdvance defined in 4.3.5.3, TS 36.306 [15]. | BC | No | No | No |
| simultaneousRx-Tx simultaneousRx-Tx defined in 4.3.5.4, TS 36.306 [15]. | BC | No | No | No |
| supportedBandwidthCombinationSetEUTRA Indicates the set of supported bandwidth combinations for the LTE part for inter-band EN-DC. The field is encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. The UE shall neither include the field for a EN-DC combination which has only one LTE carrier, nor for a EN-DC combination which has more than one LTE carrier for which the UE only supports Bandwidth Combination Set 0 for the LTE part. If the inter-band EN-DC has more than one LTE carrier, the UE shall support at least one bandwidth combination for the supported LTE part. | BC | CY | No | No |
| supportedNAICS-2CRS-AP supportedNAICS-2CRS-AP defined in 4.3.5.8, TS 36.306 [15]. | BC | No | No | No |
| totalWeightedLayers Indicates total number of weighted layers for the LTE part of the concerned EN-DC band combination the UE can process for FD-MIMO, as described in TS 36.306 [15] equation 4.3.28.yy-1 and TS 36.331 [5] subclause 6.3.6, NOTE X in UE-EUTRA-Capability field descriptions. For the EN-DC band combination for which this field is not included, totalWeightedLayers-r13 as described in TS 36.331 [5] applies, if included. | BC | No | No | No |
| ue-CA-PowerClass-N ue-CA-PowerClass-N defined in 4.3.5.1.3, TS 36.306 [15]. | BC | No | No | No |