3GPP TSG-RAN WG2 Meeting #107	R2-1908168

# Prague, Czechia, 26 – 30 August 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | CRNum | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 running CR for support of NR Industrial IoT WI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT |  |  |  |  |  |  | Date: |  | 2019-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the enhancements specified as part of the Work Item on support of Industrial Internet of Things. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New function of MAC to handle prioritization between overlapping resources of a UE is captured in section 6.2.1. - Description of downlink scheduling (section 10.2) is updated to account for support of multiple active SPS configurations for a BWP of a UE. - PDCP duplication description (section 16.1.3) is updated to account for the possibility to confgure up to 3 secondary RLC entities and support for delivery of up to 4 copies of the same PDCP PDU. - Section 16.1.4 is added to describe enhancements related to Time Sensitive Communications support, i.e. accurate reference time information provisioning, TSC Assitance Information provisioning and Ethernet header compression. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancements agreed as part of WI on support of NR Industrial IoT are not specified in stage-2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 10.2, 10.3, 16.1.3, 16.1.4 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 6.2	MAC Sublayer

### 6.2.1	Services and Functions

The main services and functions of the MAC sublayer include:

-	Mapping between logical channels and transport channels;

-	Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

-	Scheduling information reporting;

-	Error correction through HARQ (one HARQ entity per cell in case of CA);

-	Priority handling between UEs by means of dynamic scheduling;

-	Priority handling between logical channels of one UE by means of logical channel prioritisation;

-	Priority handling between overlapping resources of one UE for the cases where MAC  is involved in the prioritization;

-	Padding.

A single MAC entity can support multiple numerologies, transmission timings and cells. Mapping restrictions in logical channel prioritisation control which numerology(ies), cell(s), and transmission timing(s) a logical channel can use (see subclause 16.1.2).

Next Modified Subclause

## 10.2	Downlink Scheduling

In the downlink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible assignments when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may pre-empt an ongoing PDSCH transmission to one UE with a latency-critical transmission to another UE. The gNB can configure UEs to monitor interrupted transmission indications using INT-RNTI on a PDCCH. If a UE receives the interrupted transmission indication, the UE may assume that no useful information to that UE was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this UE.

In addition, with Semi-Persistent Scheduling (SPS), the gNB can allocate downlink resources for the initial HARQ transmissions to UEs: RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the downlink assignment can be implicitly reused according to the periodicity defined by RRC, until deactivated.

NOTE:	when required, retransmissions are explicitly scheduled on PDCCH(s).

The dynamically allocated downlink reception overrides the configured downlink assignment in the same serving cell, if they overlap in time. Otherwise a downlink reception according to the configured downlink assignment is assumed, if activated.

The UE may be configured with up to [8 or 16] active configured downlink assignments for a given BWP of a serving cell.  Network decides which of these configured downlink assignments are active at a time (including all of them). Activation and deactivation of each configured downlink assignment can be done using separate DCI commands. Activation and deactivation of configured downlink assignments are independent among the serving cells.

Editor’s note: FFS (in RAN1) whether joint activation/deactivation of multiple SPS is also supported. The exact mechanism is FFS and pending RAN1 discussions.

Editor’s note: FFS whether the maximum number of supported active configured downlink assignments for a given BWP of a serving cell is 8 or 16. FFS whether there are other restrictions of how many SPS configurations are supported, e.g. per cell / per UE.

## 10.3	Uplink Scheduling

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

In case configured uplink transmission overlaps in time with dynamically allocated uplink transmission or with another configured uplink transmission in the same serving cell, the UE prioritizes the transmission with higher priority. In case the MAC PDU associated with a deprioritized transmission has already been generated, the UE shall store it to allow the gNB to schedule a retransmission.

Editor’s note: FFS how transmission with higher priority is determined exactly.

Editor’s note: FFS other ways of treating de-prioritized data.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s).

The UE may be configured with up to 12 active configured uplink grants for a given BWP of a serving cell.  Each configured uplink grant can either be of Type 1 or Type 2.

Editor’s note: FFS (in RAN1) feasibility of joint release for 2 or more Type-2 configured grants based on DCI.

Editor’s note: FFS feasibility of simultaneous Type 1 & 2 configurations in a BWP from L2 perspectives.

Editor’s note: FFS whether there are other restrictions of how many CG configurations are supported, e.g. per cell / per UE.

For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. For Type 2, each configured grant can be activated and deactivated separately. When SUL is configured, configured uplink grants can only be signalled for one of the 2 ULs of the cell.

Next Modified Subclause

## 16.1	URLLC

<UNCHANGED TEXT REMOVED>

### 16.1.3	Packet Duplication

When duplication is configured for a radio bearer by RRC, at least one secondary RLC entity is added to the radio bearer to handle the duplicated PDCP PDUs as depicted on Figure 16.1.3-1, where the logical channel corresponding to the primary RLC entity is referred to as the primary logical channel, and the logical channel corresponding to the secondary RLC entity(ies), the secondary logical channel(s). All RLC entities have the same RLC mode. Duplication at PDCP therefore consists in submitting the same PDCP PDUs multiple times: once to each activated RLC entity for the radio bearer. With independent transmission paths, packet duplication therefore increases reliability and reduces latency and is especially beneficial for URLLC services.

Editor’s note: It is FFS whether primary and secondary RLC entity and logical channel concept is kept or not, which may impact the description in this section as well.

Figure 16.1.3-1: Packet Duplication

NOTE:	PDCP control PDUs are not duplicated and always submitted to the primary RLC entity.

When configuring duplication for a DRB, RRC also sets the initial state (either activated or deactivated). After the configuration, the states of the RLC entities configured for the DRB can then be dynamically controlled by means of a MAC control element and in DC, the UE applies the MAC CE commands regardless of their origin (MCG or SCG). When duplication is configured for an SRB the state is always active and cannot be dynamically controlled.

When activating duplication for a DRB, NG-RAN should ensure that at least one serving cell is activated for each logical channel associated to an activated RLC entity of the DRB; and when the deactivation of SCells leaves no serving cells activated for a logical channel of the DRB, NG-RAN should ensure that duplication is also deactivated.

When duplication is activated, the original PDCP PDU and the corresponding duplicate(s) shall not be transmitted on the same carrier. The primary and secondary logical channels can either belong to the same MAC entity (referred to as CA duplication) or to different ones (referred to as DC or DC+CA duplication). CA duplication can be configured together with DC duplication. In CA duplication, logical channel mapping restrictions are used in MAC to ensure that the primary and secondary logical channels are not sent on the same carrier.

When duplication is deactivated for a DRB, the secondary RLC entity is not re-established, the HARQ buffers are not flushed, and the transmitting PDCP entity should indicate to the secondary RLC entity to discard all duplicated PDCP PDUs. In addition, in case of CA duplication, the logical channel mapping restrictions of the primary and secondary logical channels are lifted for as long as duplication remains deactivated.

When an RLC entity acknowledges the transmission of a PDCP PDU, the PDCP entity shall indicate to the other RLC entity to discard it. In addition, in case of CA duplication, when the RLC entity restricted to only SCell(s) reaches the maximum number of retransmissions for a PDCP PDU, the UE informs the gNB but does not trigger RLF.

### 16.1.4	Support for Time Sensitive Communications

Time Sensitive Communications (TSC), as defined in TS 23.501 [3], is a communication service that supports deterministic communication and/or isochronous communication with high reliability and availability. Examples of such services are the ones in the area of Industrial Internet of Things, e.g. related to cyber-physical control applications as described in TS 22.104 [REF].

To support strict synchronization accuracy requirements of TSC applications, the gNB may signal 5G system time reference information to the UE using unicast or broadcast RRC signalling with a granularity of 10ns. Uncertainty parameter may be included in reference time information to indicate accuracy.

Editor’s note: FFS whether more information should be captured in Stage-2.

The gNB may also receive TSC Assistance Information (TSCAI), see TS 23.501 [3], from the Core Network, e.g. during QoS flow establishment, or from another gNB during handover. TSCAI contains additional information about the traffic flow such as burst arrival time and burst periodicity. TSCAI knowledge may be leveraged in the gNB’s scheduler to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants.

In TSC, small Ethernet frames are often used to carry application messages. To increase radio resource efficiency when these messages are sent over NR Uu interface, compression may be performed for Ethernet header in the PDCP protocol [REF].

Editor’s Note: FFS (in SA2) whether burst size is also included in TSCAI.

Editor’s Note: FFS what the [REF] is, e.g. PDCP or a new specification where the algorithm is defined.

End of changes

# Annex	- collection of RAN2 agreements on NR IIoT WI

Agreements from RAN2#105bis meeting (Xi’an, China, 08 – 12 April 2019):

| Accurate reference timing | Confirm that we use LTE rel-15 SIB and RRC unicast based methods for reference time delivery The reference time information shall correspond to a reference SFN, explicitly indicated in unicast signalling, FFS if inferred from the transmission of the SIB for SIB signalling. R2 assumes the UE shall use the end of the reference SFN value as the precise point in time to which the reference time corresponds. FFS whether the reference SFN refers to time in the future, past or whether this need to mandated one way or another. R2 assumes that some propagation delay compensation may be needed for distance > 200m. FFS what would be the method, e.g. based on current TA, and whether this can be left for UE implementation or something need to be specified. |
| --- | --- |
| Scheduling Enhancements | RAN2 think that knowledge of survival time is beneficial to gNB. FFS whether there would be any impact to AS specifications to make use of this, and such discussions would have lower priority, as it is not explicitly a WI objective. There are also concerns that QoS framework may be impacted due to survival time being provided explicitly. R2 assumes that the maximum number of active SPS configurations for a given BWP of a serving cell in the specification is 8 or 16 (FFS). R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption. Will support “short” SPS periodicities, at least down to 0.5ms Ask R1 on feasibility, and additionally the feasibility to go down to even lower values, e.g. 2 symb. R2 assumes that activation/deactivation is done by DCI. RAN1 should address activation/deactivation DCIs related with configured grant Type 2 and SPS in the case of multiple configurations When multiple UL CG or DL SPS configurations is configured, an offset for each configuration is needed for the calculation of the HARQ process ID |
| Ethernet Header Compression | IETF see no issues for 3GPP to develop and maintain a ROHC profile. Also, it seems feasible in the time frame of Rel-16. We develop Ethernet header compression 100% in 3GPP TS (not by extending ROHC) |

Agreements from RAN2#106 meeting (Reno, US, 13 – 17 May 2019):

| Accurate reference timing | SFN boundary at or immediately after the ending boundary of the SI-window in which SIB is transmitted is always used as a reference in case the time reference information is provided by broadcast signalling (as in LTE) The UE considers the frame indicated by the referenceSFN nearest to the frame where the time information is received, which can be either in the past or in future, in case the time reference information is provided by unicast signalling Signalling to support 10ns granularity. R2 assumes that either SIB9 or a new SIB is used for reference time information broadcast delivery, depending on R3 discussion outcome. “00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time)” as the origin of the time reference information, at least for the baseline case where time info type is not present or used (as in LTE). The field used for reference time information delivery is excluded from estimation of changes in system information. Specify uncertainty parameter in the reference time information in NR, encoding FFS We will have the clock type field, similar to LTE. R2 considers that this have no relation to ongoing discussions in SA2 on TSC |
| --- | --- |
| Ethernet Header Compression | Ethernet Header Compression (EHC) is configured per DRB, separately for UL and DL. Use context ID concept such that compressor and decompressor associates a context ID with Ethernet header contents. Compression is done with following principle: - For Ethernet flow resulting in creation of new context, compressor transmits at least one packet with full header and context id (to establish context in decompressor). - After above, compressor starts transmits compressed packets. FFS if multiple transmissions and/or feedback is needed. EHC header format is designed to include following mandatory fields: Context ID, Indication of header format (i.e. full header and compressed header), FFS other field, e.g. profile ID |
| PDCP duplication enhancements | Intention is that Copies are sent on different legs Dynamic Network control of DRB duplication is by MAC CE By the MAC CE, Network to control which of the configured RLC entities that is/are active Support the case that no of copies = no of active RLC entities |
| Intra-UE prioritization / multiplexing | For de-prioritized PUSCH on dynamic grant, the UE should store the de-prioritized MAC PDU in the HARQ buffer, to allow gNB to schedule re-transmission using the same HARQ process. For de-prioritized PUSCH on configured grants, a) the UE could store the de-prioritized MAC PDU in the HARQ buffer, to allow gNB to schedule re-transmission. b) FFS if the UE could transmit it using the subsequent radio resources e.g. associated with the same HARQ process The above agreements are at least applicable for cases when MAC has already generated the de-prioritized MAC PDU |