3GPP TSG-RAN2 Meeting #106	R2-1908180

# Reno, Nevada, USA, 13th May 2019 - 17th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4020 | rev | 1 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SI update notification and access barring in NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core, TEI15 |  |  |  |  |  | Date: |  |  | 2019-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#104, #105, and #105bis, it was discussed whether an NB-IoT UE is required to acquire MIB-NB to check if access barring is enabled before triggering access to the network when configured with a DRX cycle length shorter than the modification period. There seems to be different interpretations regarding whether an NB-IoT UE is required to acquire MIB-NB or to check a valid version of MIB-NB assuming that the UE, when configured with a DRX cycle length shorter than the modification period, needs to be notified if access barring is enabled. This CR clarifies the UE behaviour for the case above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following behaviour is captured with the change: an NB-IoT UE is required to acquire MIB to check if access barring is enabled when SIB14-NB is scheduled before triggering access to the network when configured with a DRX cycle length shorter than the modification period. Impact analysis: Impacted functionality: Access barring and notification of changes Interoperability: If the network implements the CR but the UE does not, there is no interoperability issue. If the UE implements the CR but the network does not, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT UE behaviour regarding the requirement to acquire MIB-NB to check if access barring is enabled when SIB14-NB is scheduled before triggering access to the network is not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.1.7	Access Barring parameters change in NB-IoT

Change of Access Barring (AB) parameters can occur at any point in time. The AB parameters are contained in SystemInformationBlockType14-NB. Update of the AB parameters does not impact the systemInfoValueTag in the MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB or the systemInfoValueTagSI in SystemInformationBlockType1-NB.

An NB-IoT UE is required to acquire MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB to check ab-Enabled indication when SystemInformationBlockType14-NB is scheduled to know whether access barring is enabled. If access barring is enabled the UE shall not initiate the RRC connection establishment / resume for all access causes except mobile terminating calls until the UE has acquired the SystemInformationBlockType14-NB.