3GPP TSG-RAN WG2 Meeting #106	R2-1908399

# Reno, USA,  13th - 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1703 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE capability for eFD-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_EBF_FDMIMO-Core, LTE_eFDMIMO-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per the incoming LS on Rel-14 UE feature list (R2-1702472), some new UE capabilities on eFD-MIMO are introduced, however, in current specification, some of these UE capabilities and the corresponding description are missing. In addition, description of dmrs-Enhancements-r13 is not accurate for the ENUMERATED (different) usage in MIMO-CA-ParametersPerBoBCPerTM. Furthermore, description of nonPrecoded-r13 can be further improved by clarifying the number of supported antenna ports. Moreover, the descrption of ‘a capability parameter may be provided per UE ‘ can be removed for the UE capabilites which can be configured perTM and/or perBoBCperTM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the missed UE capabilities on eFD-MIMO and the corresponding description. - Amend the description of dmrs-Enhancements-r13 and nonPrecoded-r13. - The descriptions on the configuration for the UE capabilites which can be configured perTM and/or perBoBCperTM are improved. Impact analysis Impacted functionality: eFD-MIMO capability signaling Inter-operability: There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some UE capabilities on eFD-MIMO remains missing, and the description of dmrs-Enhancements-r13 and nonPrecoded-r13 remain inaccurate. And some descriptions on the configuration for the UE capabilites which can be configured perTM and/or perBoBCperTM are not entirely correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.28.1, 4.3.28.4, 4.3.28.6, 4.3.28.x (new), 4.3.28.y (new), 4.3.28.z (new), 4.3.28.xx (new), 4.3.28.yy (new), 4.3.28.zz (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4015 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

4.3.28.1	beamformed-r13

Indicates the UE capabilities concerning beamformed EBF/ FD-MIMO operation (class B), see TS 36.213 [22], clause 7.2.5. The capabilities comprise of a list of pairs of {k-Max, n-MaxList} values with the nth entry indicating the values that the UE supports for each CSI process in case n CSI processes would be configured, with:

-	k-Max: Indicating the maximum number of NZP CSI RS resource configurations supported

-	n-Max: Indicating the maximum number of NZP CSI RS ports supported within a CSI process.

The capability parameters are provided separately per transmission mode (TM9, TM10), which is applicable for all bands of band combinations except when additionally included per band of band combination per TM indicating the concerned capability is different from the per TM capability.

4.3.28.2	channelMeasRestriction-r13

Indicates whether the UE supports channel measurement restriction, see TS 36.213 [22], clause 7.2.3. The capability parameter is provided separately per transmission mode (TM9, TM10).

4.3.28.3	csi-RS-EnhancementsTDD-r13

Indicates whether the UE supports CSI-RS enhancements applicable for TDD, see TS 36.211 [17], clause 6.10.5. The capability parameter is provided separately per transmission mode (TM9, TM10).

4.3.28.4	dmrs-Enhancements-r13

Indicates whether the UE supports DMRS enhancements for the indicated transmission mode, see TS 36.213 [22], clause 7.1.5B and TS 36.212 [26], clauses 5.3.3.1.5C/ D.

The capability parameter is provided separately per transmission mode (TM9, TM10), which is applicable for all bands of band combinations except when additionally included per band of band combination per TM indicating the concerned capability is different from the per TM capability.

This field is absent when the FD-MIMO capability is provided as part of sTTI/sPT band combinations.

4.3.28.5	interferenceMeasRestriction-r13

Indicates whether the UE supports interference measurement restriction, see TS 36.213 [22], clause 7.2.

4.3.28.6	nonPrecoded-r13

Indicates the UE capabilities concerning non-precoded EBF/ FD-MIMO operation (class A) for CSI-RS and CSI reporting using 8, 12 and 16 antenna ports, see TS 36.213 [22], clause 7.2.

-	config1: Indicates support of codebook configuration 1.

-	config2: Indicates support of codebook configuration 2.

-	config3: Indicates support of codebook configuration 3.

-	config4: Indicates support of codebook configuration 4.

The capability parameters are is provided separately per transmission mode (TM9, TM10), which is applicable for all bands of band combinations except when additionally included per band of band combination per TM indicating the concerned capability is different from the per TM capability.

4.3.28.7	srs-Enhancements-r13

Indicates for a particular transmission mode whether the UE supports SRS enhancements, see TS 36.211 [17], clause 5.5.3.

4.3.28.8	srs-EnhancementsTDD-r13

Indicates for a particular transmission mode whether the UE supports TDD specific SRS enhancements, see TS 36.211 [17], clauses 4.2 and 5.5.3.

4.3.28.9	csi-ReportingAdvanced-r14, csi-ReportingAdvancedMaxPorts-r14

Indicates the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting. The field csi-ReportingAdvanced-r14 is included to indicate 32 CSI-RS ports whereas csi-ReportingAdvancedMaxPorts-r14 is included to indicate 8, 12, 16, 20, 24 or 28 CSI-RS ports (i.e., UE shall not include both csi-ReportingAdvanced-r14 and csi-ReportingAdvancedMaxPorts-r14).The capability parameter is provided separately per transmission mode (TM9, TM10), which is applicable for all bands of band combinations except when additionally included per band of band combination per TM indicating the concerned capability is different from the per TM capability.

4.3.28.10	mimo-CBSR-AdvancedCSI -r15

Indicates whether UE supports CBSR for advanced CSI reporting with and without amplitude restriction as defined in TS 36.213 [22], clause 7.2.

4.3.28.x	csi-ReportingNP-r14

Indicates whether the UE supports CSI reporting on non-precoded CSI-RS with 20, 24, 28 or 32 antenna ports, see TS 36.213 [22, Table 7.2.4-9]. The capability parameter is provided separately per transmission mode (TM9, TM10), which is applicable for all bands of band combinations except when additionally included per band of band combination per TM indicating the concerned capability is different from the per TM capability. A UE indicating support of csi-ReportingNP-r14 shall also indicate support of nonPrecoded-r13.

4.3.28.y	zp-CSI-RS-AperiodicInfo-r14

Indicates whether the UE supports aperiodic ZP-CSI-RS transmission for the indicated transmission mode, see TS 36.213 [22], clause 7.2.1. The capability parameter is provided separately per transmission mode (TM9, TM10).

4.3.28.z	ul-dmrs-Enhancements-r14

Indicates whether the UE supports UL DMRS enhancements, see TS 36.211 [17], clause 6.10.3A. The capability parameter is provided separately per transmission mode (TM9, TM10).

4.3.28.xx	densityReductionNP-r14, densityReductionBF-r14

Indicates whether the UE supports CSI-RS density reduction with values 1, 1/2 and 1/3 for non-precoded CSI-RS and beamformed CSI-RS respectively, see TS 36.213 [22], clause 7.2.5. The capability parameter is provided separately per transmission mode (TM9, TM10).

4.3.28.yy	hybridCSI-r14

Indicates whether the UE supports hybrid CSI transmission, see TS 36.213 [22], clauses 7.2.1 and 7.2.2. The capability parameter is provided separately per transmission mode (TM9, TM10).

4.3.28.zz	semiOL-r14

Indicates whether the UE supports semi-open-loop transmission for the indicated transmission mode, see TS 36.213 [22], clause 7.2.4. The capability parameter is provided separately per transmission mode (TM9, TM10).