3GPP TSG-RAN2 Meeting #106	R2-1908421

# Reno, USA, 13th May – 17th May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0639 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PUCCH spatial relation Activation/Deactivation MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is some misalignment between RRC and MAC specs for “PUCCH spatial relation Activation/Deactivation MAC CE”. In 38.331, the value range of PUCCH-SpatialRelationInfoId is (1…8). However, in 38.321, it is stated “Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i”. And the value range of subscript of Si is (0…7). So for example, if RRC configures a sequence of PUCCH-SpatialRelationInfoId = 3 and 8, MAC would activate PUCCH-SpatialRelationInfoId 3 via below MAC CE according to the sentence “Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i”: But it is not possible to activate PUCCH-SpatialRelationInfoId 8 with current MAC CE mapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the mapping between Si and PUCCH-SpatialRelationInfoId. Impact analysis: Impacted 5G architecture options: Both EN-DC and standalone. Impacted functionality: PUCCH spatial relation Activation/Deactivation MAC CE Inter-operability: - If the network is implemented according to the CR and the UE is not, there is different understandings on the activation status of PUCCH Spatial Relation Info between gNB and UE, and consequently PUCCH tranmission might fail. - If the UE is implemented according to the CR and the network is not, there is different understandings on the activation status of PUCCH Spatial Relation Info between gNB and UE, and consequently PUCCH tranmission might fail. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PUCCH-SpatialRelationInfoId 8 cannot be activated by PUCCH spatial relation Activation/Deactivation MAC CE even if it is configured by RRC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 6.1.3.18	PUCCH spatial relation Activation/Deactivation MAC CE

The PUCCH spatial relation Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 24 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	PUCCH Resource ID: This field contains an identifier of the PUCCH resource ID identified by PUCCH-ResourceId as specified in TS 38.331 [5]. The length of the field is 7 bits;

-	Si: If there is a PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId as specified in TS 38.331 [5], configured for the uplink bandwidth part indicated by BWP ID field, Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId equal to i + 1, otherwise MAC entity shall ignore this field. The Si field is set to 1 to indicate PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId equal to i + 1 shall be activated. The Si field is set to 0 to indicate PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId equal to i + 1 shall be deactivated. Only a single PUCCH Spatial Relation Info can be active for a PUCCH Resource at a time;

-	R: Reserved bit, set to 0.

Figure 6.1.3.18-1: PUCCH spatial relation Activation/Deactivation MAC CE

End of change