3GPP TSG RAN WG2#106		R2-1908441

Reno, Nevada, USA, 13th – 17th May, 2019

# Title:	[DRAFT] Reply LS on Handling of UE radio network capabilities in 4G and 5G

# Response to:	R2-1908112/CVD Doc 2019-0018_001 Handling of UE radio network capabilities in 4G and 5G from GSMA CVD Governance Team

# Release:

# Work Item:

Source:	Intel [to be RAN2]

To:	GSMA CVD Governance Team

CC:	SA3, RAN, SA

Contact Person:

#### Name:	Sudeep Palat

Tel: 		+44-1793402336

E-mail Address:	sudeep.k.palat@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 thanks GSMA CVD Governance Team for the LS on Handling of UE radio network capabilities in 4G and 5G.

There are some benefits in retrieving the UE AS capability before security activation (e.g., it allows network to apply the same sequence of combined Security Mode Command + Reconfiguration for RRC connection setup for Registration and Service request procedures) and, in this scenario, to verify against any man-in-the-middle attack, it is possible in current LTE and NR specifications to rerun the UECapabilityEnquiry procedure after security activation.

Further, RAN2 confirms that the RAN2 specifications for LTE (Rel-15 as well as previous releases) and NR already allow running the UECapabilityEnquiry procedure after activation of security.

RAN2 can make the necessary updates to the specifications to include a recommendation on network implementation that security should be applied for UECapabilityEnquiry procedure if requested by SA3.

2. Actions:

None

3. Date of Next RAN2 Meetings:

3GPP RAN2#107			26th – 30th August 2019			Prague, CZ

3GPP RAN2#107bis			14th – 18th October 2019			China