3GPP TSG-RAN2 Meeting #106	R2-1908503

Reno, Nevada, US, 13 - 17 May 2019	was R3-193196

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0139 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | In-order delivery when QoS flows offloaded from SN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Huawei, Nokia, Nokia Shanghai Bell, Intel Corporation) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last meetings, RAN3 has agreed the in-order delivery when the QoS flows are offloaded from MN to SN. Furthermore, the in-order delivery when the QoS flows are offloaded from SN to MN is also needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the SN requests the QoS flow offloading from SN to MN, the SN proposes to apply forwarding of UL data and then MN replies with the UL PDU session tunnel information. If the MN requests the QoS flow offloading from SN to MN, the SN proposes to apply forwarding of UL data and then MN replies with the UL PDU session tunnel information in the XN-U ADDRESS INDICATION message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under funtional point of view. The impact can be considered isolated because the clarification affect only the in-order delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The in-order delivery is not supported when the QoS flows are offloaded from SN to MN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.3,10.14.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.423 CR 0126 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is modified based on the agreed CR R3-192094. The modifications in the R3-192084 were accepted. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-193196. |  |  |  |  |  |  |  |  |

-------------------------------------------Change Start-------------------------------------------

### 10.14.3	PDU Session Split at UPF (RAN initiated QoS flows offloading from MN to SN)

When the MN decides to split the PDU session into more than one NG-U tunnels, the MN sends the SN Addition/Modification Request message including UPF UL TEID address used at MN. Later on, if the MN receives a new UL TEID in the PDU Session Modification Confirm message, the MN may provide the new UL TEID to SN.

Figure 10.14.3-1: PDU Session Split at UPF during RAN initiated PDU session resource modify (QoS flows offloading from MN to SN)

1-2.	The MN decides to split the PDU session, it uses the SN Addition procedure or SN modification procedure, including current UPF UL NG-U tunnel used at the MN. If in-order delivery is required for some of the QoS flows, UL forwarding tunnels may be setup at this stage.

3-6d.	If in-order delivery is required, the SN buffers the first packets received from the UE for a certain QoS flow until it receives GTP-U end marker packets over the UL forwarding tunnel indicating that the MN has delivered all UL packets from the source side to UPF for that QoS flow. Then SN starts delivering UL packets to UPF for that QoS flow using the UPF UL TEID address used at the MN received at step 1.

7-8.	The MN uses the PDU Session Resource Modify Indication to inform the 5GC that the PDU session is split into two tunnels and indicate which QoS flows are associated with which DL tunnel. The 5GC confirms with PDU Session Resource Modify Confirm and allocates corresponding uplink tunnels.

9-10.	If the MN receives a new UL TEID in the PDU Session Modify Confirm message for itself, the MN will use it. If the MN receives a new UL TEID for the SN then the step 9 and step 10 (i.e., SN Modification procedure) are used to provide the new UL TEID to the SN in a subsequent SN Modification Request message and then the SN switches to use the new UL TEID to deliver UL packets.

### 10.14.X	PDU Session Split at UPF (RAN initiates QoS flows offloading from SN to MN)

When some QoS flows are offloaded from the SN to the MN, the MN may decide to split the PDU session into more than one NG-U tunnels. If the MN requests to offload, the MN sends SN Modification Request message to the SN. In case the SN requests to offload, the SN sends the SN Modification Required message to the MN.

Figure 10.14.X-1: PDU Session Split at UPF during RAN initiated PDU session resource modify (QoS flows offloading from SN to MN)

1a-1c.	When the MN requests to offload QoS flows from the SN to the MN and decides to split the PDU session into more than one NG-U tunnels, it sends the SN Modification Request message. If in-order delivery is required for some of the QoS flows, UL forwarding tunnels may be setup at this stage and the MN provides UL forwarding tunnels address information in the Xn-U Address Indication message.

2a-2b.	When the SN requests to offload QoS flows from SN to MN, the SN sends the SN Modification Required message. If in-order delivery is required for some of the QoS flows, UL forwarding tunnels may be setup at this stage and the MN provides the UL forwarding tunnels address information in the SN Modification Confirm message.

3-6d. 	If in-order delivery is required, the MN buffers the first packets received from the UE for a certain QoS flow until it receives GTP-U end marker packets over the UL forwarding tunnel indicating that the SN has delivered all UL packets from the source side to UPF for that QoS flows.

7-8.	The MN uses the PDU Session Resource Modify Indication to inform the 5GC that the PDU session is split into two tunnels and indicate which QoS flows are associated with which DL tunnel. The 5GC confirms with PDU Session Resource Modify Confirm and allocates corresponding uplink tunnels.

9-10.	If the MN receives a new UL TEID in the PDU Session Modify Confirm message for itself, the MN will use it to deliver UL packets to UPF. If the MN receives a new UL TEID for the SN then the step 9 and step 10 (i.e., SN Modification procedure) are used to provide the new UL TEID to the SN in a subsequent SN Modification Request message and then the SN switches to use the new UL TEID to deliver UL packets.

-------------------------------------------Change End-------------------------------------------