3GPP TSG-RAN WG2 Meeting #107                                                 	R2-1908761

Prague, Czech Republic, 26th - 30th Aug 2019              	  (Revision of R2-1905867)

Source:	CATT

Title:	Logged MDT in IDLE or INACTIVE mode

Agenda Item:	11.12.2

Document for:	Discussion and Decision

Introduction

In RAN2#105 meeting, the MDT modes of immediate MDT and logged MDT have been agreed to be introduced to NR, and almost follow the definitions in LTE. Especially, for the INACTIVE mode which is started to be defined in NR, some agreements have been achieved:

2.Logged MDT should also be supported for RRC_INACTIVE and RRC_IDLE.

2 The procedures of LTE MDT are the baseline of NR MDT. - for logged MDT: the network sends logged measurement configuration to the UE in connected mode, and then the UE collects measurements in RRC_IDLE/INACTIVE.

1 Apply the Logged MDT configuration, logged measurements and reporting procedures to RRC_INACTIVE state.

And then in RAN2#105bis meeting, the continuity of logged MDT from INACTIVE to IDLE mode has been agreed:

2: For Logged MDT measurement collection for RRC INACTIVE UEs, the actual process of logging within the UE, takes place in RRC INACTIVE and continued in RRC IDLE.

3: The logged measurement stored in UE during RRC INACTIVE and IDLE are kept for a given common period before they are deleted as in LTE MDT.

4: MDT measurement reporting from RRC INACTIVE and IDLE is preceded by logs availability indicator to indicate logs availability.

Therefore in this contribution, we discuss other detail of logged MDT about status transfer and parameters, and intend to achieve some guidelines for IDLE/INACTIVE mode logged MDT.

Discussion

  - Logging Performing in NR IDLE/INACTIVE mode

For logged in LTE, the network sends logged measurement configuration to the UE in connected mode, and then the UE collects measurements after the RRC connection is released. Timer for logged MDT is start immediately after UE receiving the value of Logging duration, and can only be stopped upon receiving another logged measurement configuration, upon the logged measurement information becomes full, or until expiry.

In IDLE mode, to follow LTE, the timer for logged MDT should not be stopped when UE enter “camped on any cell” or “any cell selection” in IDLE.

In NR, the transition among UE states is as shown in the figure [1] below:

Figure 4.2.1-1:	UE state machine and state transitions in NR

A table below lists all the state transition, and the UE action is filled in the second column:

Table1: UE actions of different RRC state transitions

| No. | RRC state transition | UE action | Note |
| --- | --- | --- | --- |
| 1 | CONNECTED -> IDLE | Start to collect logged MDT result | Follow LTE baseline |
| 2 | CONNECTED -> INACTIVE | Start to collect logged MDT result | Approved baseline in RAN2#105 |
| 3 | INACTIVE -> CONNECTED | Stop collecting logged MDT result; If enter IDLE/INACTIVE again, FFS① | Stop collecting logged MDT result when enter CONNECTED is approved as baseline in RAN2#105 |
| 4 | INACTIVE -> IDLE | Continue to collect logged MDT result, agreed in RAN2#105bis | Includes normal resume/release, T319 expires, etc. |
| 5 | IDLE -> CONNECTED | Stop collecting logged MDT result; If enter IDLE/INACTIVE again, FFS② | Follow LTE baseline |
| 6 | IDLE -> INACTIVE | N/A |  |
| 7 | INACTIVE -> INACTIVE | FFS③ | Upon RNAU, or upon network directly transfer UE to INACTIVE mode for any causes after receiving resume request |

From the table, we can see some of the UE actions has been agreed to follow LTE, and others have no conclusion yet:

- FFS①②:INACTIVE -> CONNECTED or IDLE -> CONNECTED:

For the transition from IDLE/INACTIVE to CONNECTED, logging should be stopped, but the logged measurement configuration will not be removed. If new configuration for logged MDT is sent by the network, the UE should replace the old configuration by the new one. But if UE enter the IDLE/INACTIVE mode again without logged MDT configuration update, it is possible to continue performing logged MDT using the old configuration if the Timer of logged MDT is not expire.

- FFS③:INACTIVE -> INACTIVE:

No matter RNAU is successfully or unsuccessfully performed, the network may indicate UE to enter INACTIVE mode again. For other causes, the network could also require the UE to enter INACTIVE mode. The logged measurement configuration cannot be carried in RRCRelease message or in RRCReject message. Therefore since no configuration is changed and the logging timer is still ongoing, the logged MDT should be continued as nothing happened.

Therefore, besides the already agreed logged MDT continuity of UE RRC_INACTIVE to RRC_IDLE [2], we also propose to agree the continuity of two other cases analyzed as below.

Proposal 1: In NR, Logged MDT will be continued after RRC state transition, includes:

- RRC_IDLE/ RRC_INACTIVE to RRC_CONNECTED to RRC_IDLE/ RRC_INACTIVE;

- RRC_INACTIVE to RRC_ INACTIVE.

  - Parameters of Logged MDT Configuration

In LTE, a list of parameters is sent to the UE from network side for the purpose of logged MDT:

Table2: Parameters of logged MDT configuration

| Classification | Parameter | Description |
| --- | --- | --- |
| Trace related | traceReference | Trace related Identifies (PLMN ID, trace ID) and Trace Collection Entity Id. |
|  | traceRecordingSessionRef |  |
|  | tce-Id |  |
| Logging related | absoluteTimeInfo | Absolute time in the current cell. |
|  | areaConfiguration | Restrict the area in which the UE performs measurement logging |
|  | loggingDuration | Duration for which UE is requested to perform measurement logging |
|  | loggingInterval | Periodicity for logging measurement results. |
|  | plmn-IdentityList | A set of PLMNs defining when the UE performs measurement logging. |
| Other feature related | targetMBSFN-AreaList | Logging of MBSFN measurements and further restrict the area and frequencies |
|  | bt-NameList | To log in which names of the Bluetooth beacon. |
|  | wlan-NameList | To log in which names of the WLAN AP. |

In NR, we could first analyze whether the parameters in LTE can be reused, and then consider whether some new parameters should be introduced:

- For the trace related parameters, since trace in NR almost has no difference from LTE, the parameters could be reused;

- For other feature related parameters, it has been agreed on WLAN/Bluetooth measurement use cases of LTE as the baseline of NR, so the bt-NameList and wlan-NameList can be inherited from LTE; Since MBMS has not been introduced in NR currently, the area list of MBSFN could be discussed further after MBMS item start in NR;

- For logging related parameters, the NR logged MDT use LTE logged MDT as baseline. The logging is performed at regular time intervals within the specific area.

Compare the logged MDT solution in LTE and NR, a distinct different point may be in the parameter of areaConfiguration. The areaConfiguration field [3] indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in the variable in UE. If configured, the field is used to restrict the area in which the UE performs measurement logging, the network could choose to send a cellGlobalIdList or a trackingAreaCodeList with allowed PLMNs. Due to the similar network structure, these area levels could be reused in NR, i.e. PLMN-level, TA-level and cell-level.

Proposal 2: In NR, Logged MDT area configuration could be cell level, TA level or PLMN level as LTE does.

In NR, for RNA update purpose, an RNA area is configured, which also includes the TA-level and cell-level configuration. Since the RNA area is only valid in INACTIVE mode, this area configuration cannot be reused for logged MDT purpose. But whether the structure of RAN-NotificationAreaInfo IE could be utilized for logging area configuration could be considered. Some analyses are listed as below.

The IE of AreaConfiguration in 36.331 is:

AreaConfiguration-r10 ::=	CHOICE {

cellGlobalIdList-r10			CellGlobalIdList-r10,

trackingAreaCodeList-r10		TrackingAreaCodeList-r10

}

AreaConfiguration-v1130 ::=		SEQUENCE {

trackingAreaCodeList-v1130		TrackingAreaCodeList-v1130

}

CellGlobalIdList-r10 ::=				SEQUENCE (SIZE (1..32)) OF CellGlobalIdEUTRA

TrackingAreaCodeList-r10 ::=			SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

TrackingAreaCodeList-v1130 ::=	SEQUENCE {

plmn-Identity-perTAC-List-r11			SEQUENCE (SIZE (1..8)) OF PLMN-Identity

}

And the IE of  RAN-NotificationAreaInfo in 38.331 is:

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                           OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                           OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode            TrackingAreaCode,

ran-AreaCodeList            SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode        OPTIONAL    -- Need R

}

A table is listed below to compare the two IEs:

Table3: Comparison between the IE of AreaConfiguration and RAN-NotificationAreaInfo

| IE Name | Contained fields | Content |
| --- | --- | --- |
| AreaConfiguration in LTE | CGI list | At most 32 CGI. |
|  | TA code list | At most 8 TACs, could belong to different PLMNs or all belong to the RPLMN. |
| RAN-NotificationAreaInfo in NR | Cell list | At most 8 PLMNs and at most 32 CellIDs in each PLMN, i.e. PLMN ID+CellID, similar as CGI. |
|  | RAN-Area list | At most 8 PLMNs and at most 16 AreaConfig in each PLMN, 1 TAC + at most 32 RAN-AreaCode in each AreaConfig. |

From the structure of ASN.1, it is concluded that:

- Both configurations could be set to cell level(CGI, or PLMN+CellID);

- Both configuration could be set to PLMN level or TA level;

- RAN-NotificationAreaInfo has the RAN areaConfig which may not be appropriate for Logged MDT configuration.

Observation: To configure the area configuration in Logged MDT, reusing RAN-NotificationAreaInfo IE is not appropriate.

Therefore for a new IE could be defined for area configuration for Logged MDT, and the structure could be defined as below:

AreaConfiguration-r16 ::=	CHOICE {

areaConfigCellList-r16        	AreaConfigCellList-r16,

trackingAreaList-r16			TrackingAreaList-r16

}

AreaConfigCellList-r16 ::=       	SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-AreaConfigCell-r16

PLMN-AreaConfigCell-r16 ::=    		SEQUENCE {

plmn-Identity-r16                   PLMN-Identity     	OPTIONAL,   -- Need S

ran-AreaCells                      	SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

TrackingAreaList-r16 ::=			SEQUENCE (SIZE (1..8)) OF TrackingArea-r16

TrackingArea-r16 ::=				SEQUENCE {

plmn-Identity-r16					PLMN-Identity    	OPTIONAL,   -- Need S

trackingAreaCode-r16        		TrackingAreaCode

}

Proposal 3: Define a new IE for area configuration in Logged MDT, similar as the AreaConfiguration IE structure in LTE.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: In NR, Logged MDT will be continued after RRC state transition, includes:

- RRC_IDLE/ RRC_INACTIVE to RRC_CONNECTED to RRC_IDLE/ RRC_INACTIVE;

- RRC_INACTIVE to RRC_ INACTIVE.

Proposal 2: In NR, Logged MDT area configuration could be cell level, TA level or PLMN level as LTE does.

Observation: To configure the area configuration in Logged MDT, reusing RAN-NotificationAreaInfo IE is not appropriate.

Proposal 3: Define a new IE for area configuration in NR Logged MDT, similar as the AreaConfiguration IE structure in LTE.

Reference

- TS38.331-f40

- RAN2-105bis-Notes LTE-5GC NR-idle RAN-DCU

- TS36.331-f40