3GPP TSG-RAN WG2 Meeting #107                                                       R2-1908827

Prague, Czech Republic, 26th - 30th August 2019

Source:	CATT

Title:	Data/data prioritization timeline

Agenda Item:	11.7.3

Document for:	Discussion and Decision

Introduction

One objective of the IIoT WID, revised in RAN#84 [1], is to specify means to address conflicts between overlapping PUSCH transmissions, aka data/data prioritization:

- The detailed objectives for NR intra-UE prioritization/multiplexing are: - Specify enhancements to address resource conflicts between dynamic grant (DG) and configured grant (CG) PUSCH and conflicts involving multiple CGs [RAN2, RAN1]. - Specify PUSCH grant prioritization based on LCH priorities and LCP restrictions for the cases where MAC prioritizes the grant [RAN2].

In [2], we compare the various framework alternatives and conclude that MAC-based prioritization (where only MAC-selected PDUs should be delivered to PHY) should be favored over PHY-based prioritization (where MAC delivers all PDUs to PHY). However, even with the former approach, different timeline options exist. For example, [3] proposes that LCPs of overlapping grants are run in sequence, in order allocation start time. Another option could as well be that they are run in order of received grants. Thus, in this contribution we specifically address the timeline of MAC-based prioritization.

Discussion

When determining the priority of overlapping grants, whenever possible, for fair comparison, MAC should run the LCPs in parallel on the same buffer status, as opposed to running them in sequence, in order of either received grants or allocation start time [3]. For example, assume two overlapping grants (DG/CG or CG/CG) which UE is aware ahead of time, with data in the buffer. In such case, for a fair prioritization rule, the grants should be processed assuming the same buffer status. Because, if processed sequentially, when processing the first grant, some LCH data will be taken out of the buffer and the 2nd grant will be processed on the leftover data. The LCP will then consider different values of Bj thus resulting in potential different LCH(s) selection and, as a result, different highest priority value compared with running the LCP for that grant on the same buffer status as for the first grant.

Another drawback with the sequential processing of overlapping grants is that it will result in unnecessary delivery of MAC PDUs that will be further dropped. Indeed, as shown in Figure 1, there will be cases where high priority data is already available in the buffer when UE processes the earlier grant but the LCH such data is mapped on can only go in the later grant (e.g. because of shorter PUSCH). However sequential processing will ignore the 2nd grant when treating the 1st grant and will therefore generate and deliver the associated MAC PDU to PHY, and when processing the 2nd grant will prioritize it and cancel/drop the already delivered MAC PDU. We discuss in [2] the drawbacks of unnecessarily generating MAC PDUs to be further dropped, and this should be avoided as much as possible.

Figure 1: Sequential vs parallel prioritization example

Proposal 1: When determining the priority of overlapping grants, whenever possible, for fair comparison, MAC should run the LCPs in parallel on the same buffer status.

A consequence of the above is that, in order to account for the freshest buffer status, when UE is scheduled with overlapping grants, it runs the (first) prioritization rule at the processing deadline (i.e. PUSCH – T’proc,2) of the grant allocation starting earlier (Figure 1).

Proposal 2: When UE is scheduled with overlapping grants, it runs the (first) prioritization rule at the processing deadline (i.e. PUSCH start – T’proc,2) of the grant allocation starting earlier.

Of course, it is not always possible to run the LCPs of competing grants on the same buffer status, when UE is scheduled with overlapping grants with different starting times. A first example is when a DCI for a 2nd grant is received after the PDU for the first grant was assembled (Figure 2 left). Another case is when new data arrives after the earlier allocation has been prioritized by a first run, which could change the outcome of the prioritization (Figure 2 right).

However, for subsequent/late prioritization runs, the earlier grant should keep its priority from the time when its MAC PDU was generated as there is no reason to fill different/new data in such grant.

Note that the above pre-emptions by a later allocation should not be supported for the same HARQ process otherwise the pre-empted MAC PDU is overridden and lost.

Figure 2: Cases of grants comparison on different buffer statuses due to late DCI (left) or new data arrival (right)

Proposal 3: When UE is scheduled with overlapping grants with different starting times, if the grant with the earlier allocation was prioritized upon a first prioritization and the grant for the later allocation is for a HARQ process different than the earlier grant, the UE runs again the prioritization rule at the processing deadline of the later grant allocation.

Proposal 4: Once it has been assembled, the priority of a prioritized grant always remains the same, i.e. based on the same assembled LCHs, even if compared again with the later grant allocation in a 2nd prioritization run.

Conclusion

In this contribution, we discussed the timeline of MAC-based data/data prioritization and concluded on the following proposals:

Proposal 1: When determining the priority of overlapping grants, whenever possible, for fair comparison, MAC should run the LCPs in parallel on the same buffer status.

Proposal 2: When UE is scheduled with overlapping grants, it runs the (first) prioritization rule at the processing deadline (i.e. PUSCH start – T’proc,2) of the grant allocation starting earlier.

Proposal 3: When UE is scheduled with overlapping grants with different starting times, if the grant with the earlier allocation was prioritized upon a first prioritization and the grant for the later allocation is for a HARQ process different than the earlier grant, the UE runs again the prioritization rule at the processing deadline of the later grant allocation.

Proposal 4: Once it has been assembled, the priority of a prioritized grant always remains the same, i.e. based on the same assembled LCHs, even if compared again with the later grant allocation in a 2nd prioritization run.

Reference

- RP-191561, Revised WID: Support of NR Industrial Internet of Things (IoT)

- R2-1908826, Data/data prioritization rules and framework, CATT

- R2-1906187, Analysis of Intra-UE Data Prioritization Schemes, Nokia

We use T’proc,2 to differentiate it from Rel-15 Tproc,2 in case RAN1 sees the need to upgrade it to account for the prioritization delay (if any).