3GPP TSG-RAN2 Meeting #107	R2-1908866

# Prague, Czech Republic, 26th Aug 2019 - 30th Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4030 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the field description of nprach-SubCarrierIndex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the field description of nprach-NumCBRA-StartSubcarriers, the CBRA resource that the UE is allowed to randomly select from, are given by: nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1]. The above part would be CFRA resources. For dedicated SR feature, only CFRA resources can be configured as PRACH resource of dedicated SR. This can be explained in the following figure: According to the above figure, we understand the CFRA subcarrier index should be in the range [nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]. So the PRACH resource index for dedicated SR (e.g, nprach-SubCarrierIndex in SchedulingRequestConfig) should not be configured to a smaller value than nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers. But in the current specification, it is described that “E-UTRAN does not configure nprach-SubcarrierIndex to a smaller value than nprach-NumCBRA-StartSubcarriers for the indicated NPRACH resource.” which is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the field description of parameter nprach-SubCarrierIndex for dedicated SR. Impact Analysis Impacted functionality: The change only impacts the NPRACH resource configuration for dedicated SR. Inter-operability: If the value of nprach-SubcarrierOffset is set to 0, there is no inter-operability problem. If the value of nprach-SubcarrierOffset is set to non-zero: - If the network is implemented according to the CR and the UE is not, as network configuration is subset of the set assumed by UE, there also has no inter-operability problem. - If the UE is implemented according to the CR and the network is not, the UE will think the indicated NPRACH resource as incorrect configuration. This may cause failure of dedicated SR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the value of nprach-SubcarrierOffset is set to non-zero, subcarrier index in the range [0, nprach-NumCBRA-StartSubcarriers], e.g., some CBRA resource, may be configured for PRACH resource for dedicated SR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of the modified section>

–	SchedulingRequestConfig-NB

The IE SchedulingRequestConfig-NB is used to specify the Scheduling Request related parameters.

SchedulingRequestConfig-NB information element

-- ASN1START

SchedulingRequestConfig-NB-r15 ::=	SEQUENCE {

sr-WithHARQ-ACK-Config-r15			ENUMERATED {true}	OPTIONAL,

sr-WithoutHARQ-ACK-Config-r15			SR-WithoutHARQ-ACK-Config-NB-r15	OPTIONAL,	-- Need ON

sr-SPS-BSR-Config-r15				SR-SPS-BSR-Config-NB-r15			OPTIONAL,	-- Need ON

...

}

SR-WithoutHARQ-ACK-Config-NB-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

sr-ProhibitTimer-r15				INTEGER (0..7)	OPTIONAL,	-- Need ON

sr-NPRACH-Resource-r15				SR-NPRACH-Resource-NB-r15	OPTIONAL -- Need ON

}

}

SR-NPRACH-Resource-NB-r15		::=	SEQUENCE {

nprach-CarrierIndex-r15				INTEGER (0..maxNonAnchorCarriers-NB-r14),

nprach-ResourceIndex-r15			INTEGER (1..maxNPRACH-Resources-NB-r13),

nprach-SubCarrierIndex-r15			CHOICE {

nprach-Fmt0Fmt1-r15					INTEGER (0..47),

nprach-Fmt2-r15						INTEGER (0..143)

},

p0-SR-r15							INTEGER (-126..24),

alpha-r15							ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}}

SR-SPS-BSR-Config-NB-r15	 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

semiPersistSchedC-RNTI-r15			C-RNTI,

semiPersistSchedIntervalUL-r15		ENUMERATED {sf128, sf256, sf512, sf1024,

sf1280, sf2048, sf2560, sf5120}

}

}

-- ASN1STOP

SchedulingRequestConfig-NB field descriptions

alpha Parameter: αc. Fractional power control parameter for SR without HARQ-ACK. See TS 36.213 [23], clause 16.2.1.2.1, where value al0 corresponds to 0, value al04 corresponds to 0.4, value al05 to 0.5, value al06 to 0.6, value al07 to 0.7, value al08 to 0.8, value al09 to 0.9 and value al1 corresponds to 1.

nprach-CarrierIndex Index of the carrier in the list of UL non anchor carriers in SystemInformationBlockType22-NB. The first entry in the list has index '1', the second entry has index '2' and so on. Value '0' indicates the anchor carrier.

nprach-ResourceIndex Index of the NPRACH resource in the list of NPRACH resources in NPRACH-ParametersList or NPRACH-ParametersList-Fmt2 for the UL carrier indicated by nprach-CarrierIndex. The first entry in the list has index '1', the second entry has index '2' and so on. E-UTRAN configures a NPRACH resource in NPRACH-ParametersList-Fmt2 only to UEs that have reported support of NPRACH resource Format2.

nprach-SubCarrierIndex Index of the subcarrier in the NPRACH resource in NPRACH-ParametersList or or NPRACH-ParametersList-Fmt2 for the indicated UL carrier. E-UTRAN does not configure nprach-SubcarrierIndex to a smaller value than nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers for the indicated NPRACH resource.

p0-SR Parameter:. Target power for SR without HARQ-ACK. See TS 36.213 [23], clause 16.2.1.2.1, unit dBm.

semiPersistSchedC-RNTI Semi-persistent Scheduling C-RNTI, see TS 36.321 [6].

semiPersistSchedIntervalUL Semi-persistent scheduling interval in uplink, see TS 36.321 [6]. Value in number of sub-frames. Value sf128 corresponds to 128 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

sr-SPS-BSR-Config Activation of SR with SPS BSR, see TS 36.321 [6]. E-UTRAN cannot configure sr-SPS-BSR together with sr-WithoutHARQ-ACK-Config.

sr-NPRACH-Resource NPRACH resource for physical layer SR without HARQ-ACK, see TS 36.211 [21] and TS 36.213 [23].

sr-ProhibitTimer Timer for SR transmission on the NPRACH resource for SR in TS 36.321 [6]. Value in number of SR period, where the SR period is equal to the field nprach-Periodicity of the NPRACH resource. Value 0 means that behaviour as specified in 7.3.2 applies. Value 1 corresponds to one SR period, Value 2 corresponds to 2*SR period and so on.

sr-WithHARQ-ACK-Config Activation of physical layer SR with HARQ ACK, see TS 36.213 [23].

sr-WithoutHARQ-ACK-Config Activation of physical layer SR without HARQ ACK, see TS 36.211 [21] and TS 36.213 [23]. E-UTRAN cannot configure sr-WithoutHARQ-ACK-Config together with sr-SPS-BSR.

### <End of the modified section>