3GPP TSG-RAN WG2 Meeting #107	R2-1908948

Prague, Czech Republic, 26th – 30th August 2019	Revision of R2-1906485

Agenda Item:	11.9.3.5

Source: 	MediaTek Inc.

Title:  	Mobility Failure Recovery in NR

Document for:	Discussion and decision

Introduction

The following objective has been approved for NR mobility enhancements [1].

- Specify further enhancements to achieve following targets, [RAN2/3] - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered. - improve the robustness during handover,

To improve mobility robustness, there are two general aspects:

- Improve mechanisms to reduce handover failures;

- Improve recovery mechanisms when a mobility failure occurs;

To reduce handover failure occurrences, the network may consider conditional handover (CHO), which RAN2 agreed to support for LTE and to study for NR. In this paper, we discuss another aspects, i.e. improving recovery mechanisms when a mobility failure occurs. This paper is a revision of our previous work of R2-1906485; the main difference is that the discussion about “CHO for HoF recovery” is simplified, since related procedures have been considered as working assumptions by RAN2 in previous meeting.

Discussion

Mobility failures in network-controlled handover

Mobility is considered failed in the following cases:

- RLF occurs (T310 expiry): This usually means that the UE has sent a measurement report but fails to receive handover command from serving eNB.

- Handover failure (T304 expiry): This means a handover command has been received (T304 started) but the UE fails to access the target eNB.

In LTE, mobility failure can be recovered by RRC Connection Reestablishment procedure. In addition, fast RLF declaration based on T312 timer allows UE to start recovery procedure before T310 timer expires, and thus reduces interruption time. Even though the overall interruption time is still long, these mechanisms work well in LTE system in which mobility failure is less frequent. In NR, however, more frequent mobility failure is expected, especially in high frequency operation scenario. Thus, improved mobility failure recovery is needed for NR, and the goal is to shorten service interruption time.

Conditional handover for mobility failure recovery

In previous meeting, we have the following working assumptions about UE accessing another CHO candidate cell upon mobility failure.

Working assumption: 8 At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed. 9 At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

We think these working assumptions are reasonable, and should be considered as agreements.

Proposal 1:	The following working assumptions should be considered as RAN2 agreements:

  - At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

  - At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

Enhancement of RRC reestablishment procedures

Accelerated RRC reestablishment with early context transfer

While mobility failure can be recovered by RRC reestablishment, the major concern is long service interruption, which is mainly caused by signalling exchange (request and response) and UE context fetch (from source gNB to target gNB). To accelerate RRC reestablishment procedure, the UE context may be transferred to potential target cells before RRC reestablishment is requested. The idea can be explained with two examples below.

In Figure 1, we consider mobility failure due to UE not receiving handover command.

Figure 1. Mobility failure: fail to receive handover command

In the scenario illustrated in Figure 1:

- t1: UE is served by gNB1 (source gNB).

- t2: UE detects gNB2 and gNB3. The preconfigure measurement event for early context transfer is triggered and UE reports gNB2 and gNB3. The source gNB transfers UE context to gNB2 and gNB3, and sends an indication to UE. Notice that the indication does not contain any RRC configuration of gNB2 and gNB3.

- t3: UE reports gNB3 for normal handover event (say, Event A3).

- t4: UE expects to receive handover command (reconfigurationWithSync) from gNB3, but UE experiences sudden channel degradation (e.g., due to blockage) and never receives the command, and thus never indicated to handover to gNB3.

- t5: T310 expires, and UE declares RLF to gNB1 and then initiates RRC reestablishment to gNB2. UE Context fetch is not needed and faster recovery from mobility failure can be achieved.

In Figure 2, we consider mobility failure due to UE failing to access target gNB indicated by handover command.

Figure 2. Mobility failure: fail to connect to target cell

In the scenario illustrated in Figure 2:

- t1: UE is served by gNB1 (source gNB).

- t2: UE detects gNB2 and gNB3; the preconfigure measurement event for early context transfer is triggered and UE reports gNB2 and gNB3. The source gNB transfers UE context to gNB2 and gNB3, and sends an indication to UE. Notice that the indication does not contain any RRC configuration of gNB2 and gNB3.

- t3: UE reports gNB2 for normal handover event (say, Event A3).

- t4: UE receive handover command from gNB1, indicating handover to gNB2. However, UE fails to access gNB2 due to blockage.

- t5: T304 expires and UE gives up access to gNB2. Based on early indication, UE knows initiates RRC reestablishment to gNB3. UE Context fetch is not needed and faster recovery from mobility failure can be achieved.

From the two examples above, we observe that the network can transfer UE context to potential target cells and reduce the interruption time in RRC reestablishment due to UE context fetch. The cost is Xn signaling overhead due to UE context transfer to potential cells even most of them are eventually not chosen as target cell.

Proposal 2:	Support RRC reestablishment with early context transfer. In this way, RRC reestablishment can be accelerated since no context fetch is needed after RRC reestablishment request.

The message flow for this method is illustrated in the Figure 3, considering the case of unsuccessful handover command delivery.

Figure 3. Accelerated RRC Reestablishment with early context transfer

Signalling optimization for RRCReestablishment multiplexed with RRCReconfiguration message

In NR, since RRCReestablishment is sent on SRB1, it is possible for the network to send RRCReconfiguration message multiplexed with the RRCReestablishment, when UE context is available (via either context fetch or the proposed early context transfer method). In this case, the UE may also respond with a single message. For example, the RRCReestablishmentComplete message can be modified and serve as a single complete message for both RRCReestablishment and RRCReconfiguration.

Proposal 3:	A single message, e.g. modified RRCReconfigurationComplete, is used for UE to respond to an RRCReestablishment message multiplexed with RRCReconfiguration.

Practical usage

In this paper, we discuss two methods for mobility failure recovery. Both methods can be supported in NR, and UE can choose the recovery method when normal handover fails, based on network configurations. For example, if a (fully-prepared) CHO candidate is available, UE can execute CHO towards that candidate for fastest recovery; otherwise, if an early-prepared cell (with UE context) is available, UE can perform reestablishment to that cell for a slightly longer but at least reliable recovery. Finally, if no candidates are available, UE can still apply legacy RRC reestablishment or connection setup.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	The following working assumptions should be considered as RAN2 agreements:

  - At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

  - At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed.

Proposal 2:	Support RRC reestablishment with early context transfer. In this way, RRC reestablishment can be accelerated since no context fetch is needed after RRC reestablishment request.

Proposal 3:	A single message, e.g. modified RRCReconfigurationComplete, is used for UE to respond to an RRCReestablishment message multiplexed with RRCReconfiguration.

Reference

- RP-181433, New WID: NR mobility enhancements,	Intel Cooperation