3GPP TSG RAN WG2 Meeting #107 	R2-1909053

Prague, CZ, 26-30 Aug 2019

Title:	Revision to Non-Terrestrial Networks reference scenarios parameters

Source: 	Thales, HNS

Type:	pCR

Document for:	Agreement

Agenda Item:	11.6.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at revising the TR 38.821

- the Table 4.2-2: Reference scenario parameters of chapter 4.2

- The Table 7.1-1: NTN scenarios versus delay constraints, Source [2] of chapter 7.1.1

# Discussion

In this paper we propose to revise the following parameters of the reference scenarios currently studied:

- Max channel bandwidth (service link): is currently set to 400 MHz for band > 6 GHz while the satellite communication industry suggests setting it to 1 GHz based on state of the art and in orbit satelite resources

- User equipment motion on the earth: The max speed is set to 1000 km/h (e.g. aircraft). Actually, the max aircraft should be increased to 1200 km/h to cover all commercial aircrafts

- User equipment Tx power : It is defined as up to 4 W for terminal with Directive antenna. Actually, this limit should be increased to 20 W to cover aircraft mounted devices

Furthermore, it should be clarified that in table 7.1-1, that the Round Trip Delay below refers actually to the propagation delay contribution to the RTD

- Maximum Round Trip Delay on the radio interface between the gNB and the UE:

- Minimum Round Trip Delay on the radio interface between the gNB and the UE:

# Conclusion

Proposal 1: Reflect the above proposals in the TR 38.821 (See TP below)

# Text Proposal for TR 38.821 v0.7.0

START OF CHANGES

## 4.2	Non-Terrestrial Networks reference scenarios

…

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |
| Payload | Scenario A : Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) |
| Max beam foot print diameter at nadir | 500 km | 200 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) - 25.77 ms (600km) - 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) - 12.89 ms (600km) - 20.89 ms (1200km) |
| Max delay variation within a beam (earth fixed user equipment) | 16ms | 4.44ms (600km) 6.44ms (1200km) |
| Max differential delay within a beam | 1.6 ms | 0.65 ms (600km and 1200km) |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |
| User equipment motion on the earth | 1200km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

NOTE 1:	Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

NOTE 2:	Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment

NOTE 3:	Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir

NOTE 4:	Speed of light used for delay calculation is 299792458 m/s.

END OF CHANGES

START OF CHANGES

### 7.1.1 	Delay

In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 7.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35 786 km |  | 600 km |  |  |  |
| Relative speed of Satellite wrt earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 1000 km |  | 50 km / 500 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |
| Maximum delay difference within a NTN beam as seen by the UE (note 3) | 16 ms (Worst case) |  | 4.44 ms |  |  |  |
| Max rate of hand-over (FFS) |  |  |  |  |  |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario NOTE 3: The delay difference compares the delay (function of UE-satellite-NTN gateway distance) experienced by two different UEs served by the same beam at a given time NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

END OF CHANGES