3GPP TSG-RAN WG2 Meeting #107	R2-1909290

Prague, Czechia, 26th – 30th of August 2019

Agenda item:	11.9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Time to say goodbye to bye message in CHO execution

WID/SID:	NR_Mob_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#106 the following agreement [1] has been taken, concerning UE’s behaviour when CHO execution condition is met:

5 CHO execution does not trigger measurement report.

In this brief TDoc we would like to outline the consequences of such decision.

# 2	Discussion

This may have not been evident to all companies involved in related discussion, but the agreement quoted above is actually equivalent to: “No bye message to the source cell when CHO execution occurs”. Measurement Report (MR) triggered by CHO execution can be interpreted in such a way. Thus, by deciding not to have such reporting, RAN2 has excluded “bye message” as a part of CHO scheme.

Observation 1: As CHO execution does not trigger measurement reporting, “bye message” is not supported for CHO.

Pros and cons of bye message have been widely discussed in numerous RAN2 TDocs. One of our previous papers [2] has also devoted a short section to this topic. Among the others, we have outlined “bye message” may help the source cell, e.g. to start data forwarding at the right point in time, or to stop the transmission to the UE. On the other hand, such “bye message” will have a significant probability of failed reception, since it may be done when the UE is already safely inside the target cell. Thus, we feel there is much more risk linked to this solution than possible gains (which can be also achieved via other means). As a result, we propose the following:

Proposal 1: RAN2 is asked to confirm explicitly that “CHO execution does not trigger measurement report” means “bye message is not supported for CHO”.

Proposal 2: Proposal 1 is applicable to CHO in both E-UTRAN and NR.

# 3	Conclusion

This paper elaborated on the need for bye message in the Conditional Handover scheme. The following observations and proposals have been made:

Observation 1: As CHO execution does not trigger measurement reporting, “bye message” is not supported for CHO.

Proposal 1: RAN2 is asked to confirm explicitly that “CHO execution does not trigger measurement report” means “bye message is not supported for CHO”.

Proposal 2: Proposal 1 is applicable to CHO in both E-UTRAN and NR.

# References

- R2-1908108 Report from session on Rel-16 LTE Mobility Enhancements WI 3GPP TSG-RAN WG2 Meeting #106, Reno, USA, 13th – 17th of May 2019

- R2-1907276 Conditional Handover in E-UTRAN – other aspects 3GPP TSG-RAN WG2 Meeting #106, Reno, USA, 13th – 17th of May 2019