3GPP TSG-RAN WG2 Meeting #107	R2-1909308

Prague,  Czech Republic, 26 – 30 August 2019

Agenda item:	11.18

Source:	Nokia, Nokia Shanghai Bell

Title:	Adding NIDs and CAG Identifiers to SIB

WID/SID:	NG_RAN_PRN - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN agreed that the Non-Public Networks (NPN) related RAN work will be carried out within NG_RAN_PRN work item [1]. The support of NPNs (SNPNs and PNI-NPNs) requires advertising of new additional identifiers for network (re-)selection and cell (re)selection. This paper proposes how to add these new identifiers to SIBs.

# 2	Discussion

According to clause 5.30.2.2 of 3GPP TS 23.501 [2] NG-RANs within SNPN broadcast the following additional identifiers to UEs for network (re)selection and cell (re)selection:

- List of NIDs per PLMN ID identifying the non-public networks NG-RAN provides access to;

According to 5.30.3.2 of 3GPP TS 23.501 [2] NG-RANs within PNI-NPN broadcast the following additional identities to UEs for cell selection and reselection:

- A CAG cell broadcasts one or multiple CAG Identifiers per PLMN;

As the NIDs are needed for network and cell (re)selection, and CAG identifiers are needed for cell (re)selection, our view is they should be added to SIB1.

Proposal 1: Add NIDs and CAG identifiers to SIB1.

In the current structure of SIB1 (6.2.2 of 3GPP TS 38.331 [3]) the IE CellAccessRelatedInfo indicates cell access related information for this cell; more specifically the PLMN-IdentityInfoList contains the PLMN specific identifiers. As NID and CAG are also PLMN specific identifiers related to the cell access, it is logical to encode them within PLMN-IdentityInfoList (see Figure 1).

Proposal 2: Add NIDs and CAG identifiers to PLMN-IdentityInfoList.

CellAccessRelatedInfo   ::=         SEQUENCE {

plmn-IdentityList                   PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}  OPTIONAL,            -- Need R

...

}

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

Figure 1: CellAccessRelatedInfo and PLMN-IdentityInfoList

As there can be multiple NIDs and CAG Identities for a PLMN Identity the following way of extending of PLMN-IdentityInfo is proposed:

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...,

nidList                                 SEQUENCE (SIZE (0..maxNID) of NID)                          OPTIONAL,       -- Need R

cag-IdentifierList                        SEQUENCE (SIZE (0..maxCAG) of CAG-Identifier)             OPTIONAL        -- Need R

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

Figure 2: Extension of PLMN-IdentityInfo

Proposal 3: Adopt the extension of PLMN-IdentityInfo with NIDs and CAG Identifiers as it is proposed in Figure 2.

# 3	Conclusions

The proposals made in this contribution are summarized below.

Proposal 1: Add NIDs and CAG identifiers to SIB1.

Proposal 2: Add NIDs and CAG identifiers to PLMN-IdentityInfoList.

Proposal 3: Adopt the extension of PLMN-IdentityInfo with NIDs and CAG Identifiers as it is proposed in Figure 2.

# References

[1]	RP-1900729: "New Work Item Proposal on Private Network Support for NG-RAN"

[2]	3GPP TS 23.501, System Architecture for the 5G System; Stage 2 (Release 16)

[3]	3GPP TS 38.331, Radio Resource Control (RRC) protocol specification