3GPP TSG-RAN WG2 Meeting #107	R2-1909389

# Prague, Czech, 26 – 30 August 2019 		                             Revision of R2-1906364

Agenda item:		11.7.4

Source:	Intel Corporation

Title:	PDCP duplication with multiple RLC entities

Document for:		Discussion and Decision

# Introduction

NR IIoT WID RP-190728 [2] defines the following scope related to PDCP duplication with multiple RLC entities:

- Specify PDCP duplication with up to 4 RLC entities configured by RRC in architectural combinations including CA only and NR-DC in combination with CA [RAN2, RAN3]. - Specify mechanisms relating to dynamic control of how a set or subset of configured RLC entities or legs are used for PDCP duplication [RAN2, RAN3].

In RAN2#106 meeting, following was agreed:

- Intention is that Copies are sent on different legs - Dynamic Network control of DRB duplication is by MAC CE - By the MAC CE, Network to control which of the configured RLC entities that is/are active - Support the case that no of copies = no of active RLC entities

Email discussion [106#55][IIOT] Network control of PDCP duplication enhancements was held to discuss the details on network control of PDCP duplication.

In this contribution, we discuss PDCP duplication with multiple RLC entities.

# Discussion

## Scenario

For Rel-16 PDCP duplication enhancements, following was captured in TR 38.825 [1]:

RRC configuration can be used to initially configure UEs of a set of RLC entities or legs; for where the NW can dynamically control how configured RLC entities or legs are activated and used for duplicate transmission using signalling such as MAC CE.

In [5], it was proposed that dynamic selection of a subset of RLC entities for PDCP duplication is not applicable for CA duplication. The main argument is that when selecting 2 carriers out of 4 carriers, MAC CE based approach does not bring performance improvements compared with PDCCH based approach.

Firstly it should be noted that the conclusion from study item is to support up to 4 copies for duplication (although it was also mentioned that “use of more than two copies is not expected to be a common configuration”). Excluding CA duplication from dynamic selection of RLC entities essentially eliminates the possibility of supporting more than 2 copies for duplication.

Secondly, from specification perspective, there is no further complexity increase to support dynamic selection of RLC entities for CA duplication if such support is available for DC+CA duplication.

Thirdly as also mentioned by [5], PDCCH based approach has the issue of disabling PDCP duplication when all the carriers of one group experience poor channel condition. There is no such issue for MAC CE based approach.

From above discussion, it is proposed to support dynamic selection of RLC entities for PDCP duplication for both CA only and DC+CA architectural options.

Proposal 1: Dynamic selection of RLC entities for PDCP duplication is supported for both CA only and DC+CA architectural options.

## Number of copies

There were proposals that the number of copies can be less than the number of activated RLC entities. There are already two levels of control for the PDCP duplication: (1) RRC signalling that configures the set of RLC entities associated with the DRB, and (2) MAC based approach (MAC CE and/or UE based approach) that selects a subset of RLC entities among the RRC-configured RLC entities. Therefore there is no need to add another level of control since existing mechanisms already provide sufficient flexibility.

Proposal 2: In PDCP duplication, the number of copies should always be equal to the number of activated RLC entities, which means that from specification point of view, there is no need to specify the number of copies.

## Primary RLC entity

In Rel-15 NR PDCP duplication, the primary RLC entity has to be always activated while the secondary RLC entity can be activated/deactivated by MAC CE. When PDCP duplication is activated, PDCP Control PDUs are only transmitted via primary RLC entity, as in TS 38.323 clause 5.2.1 below:

- else, if the transmitting PDCP entity is associated with two RLC entities: - if the PDCP duplication is activated: - if the PDCP PDU is a PDCP Data PDU: - duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities; - else: - submit the PDCP Control PDU to the primary RLC entity;

For Rel-16 PDCP duplication enhancements, there was discussion in [3] whether primary RLC entity should be configured. In Rel-15, the principle is that PDCP Control PDUs are not duplicated, therefore it is transmitted on primary RLC entity only. If the principle is kept for Rel-16, then there should be one RLC entity where PDCP Control PDUs are transmitted.

Proposal 3: One RLC entity is selected among the activated RLC entities to transmit PDCP Control PDU in order to keep Rel-15 principle that PDCP Control PDUs are not duplicated.

The next question is whether there is one RLC entity always activated like primary RLC entity in Rel-15. Given that a subset of RLC entities can be selected by MAC CE, it would be preferable to allow any RLC entity of a DRB be activated/deactivated as long as there is at least one RLC entity activated for the DRB, to achieve the best performance.

Proposal 4: Any RLC entity of a DRB can be activated/deactivated by MAC CE as long as there is at least one RLC entity activated for a given DRB.

Given the above discussion, it is necessary to select one RLC entity out of activated RLC entities for transmission of PDCP Control PDU. In [4], it was proposed to introduce an identifier of the RLC entity which is unique within the associated PDCP entity. Following this approach, a unique index (0..3) can be assigned for each RLC entity. The RLC entity with the smallest index among the activated RLC entities is used for transmission of PDCP Control PDU.

Proposal 5: A unique index (0..3) is assigned for each RLC entity, and the RLC entity with the smallest index among the activated RLC entities is used for transmission of PDCP Control PDU.

## MAC CE design

In NR Rel-15, Duplication Activation/Deactivation MAC CE is specified as below in TS 38.321:

The Duplication Activation/Deactivation MAC CE of one octet is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing eight D-fields… Di: This field indicates the activation/deactivation status of the PDCP duplication of DRB i where i is the ascending order of the DRB ID among the DRBs configured with PDCP duplication and with RLC entity(ies) associated with this MAC entity.

For Rel-16 PDCP duplication enhancements, there was discussion e.g. in [4] on how MAC CE is designed. One principle of Rel-15 design is that a single MAC CE is used to activate/deactivate PDCP duplication of the DRBs configured with PDCP duplication and with RLC entities associated with the MAC entity. It is proposed to follow the same principle in Rel-16.

Proposal 6: A single MAC CE is used to select RLC entities for PDCP duplication for DRBs configured with PDCP duplication and with RLC entities associated with the MAC entity (following Rel-15 principle).

Given that it is proposed that any RLC entity can be activated/deactivated by MAC CE in Proposal 4, it is proposed to have a new fixed size MAC CE as shown in Figure 1 below. The field REij indicates whether RLC entity j of DRB i are used for duplication transmission, while i is the ascending order of the DRB ID among the DRBs configured with PDCP duplication and with RLC entity(ies) associated with this MAC entity, and j is the unique RLC entity index within the DRB i.

Figure 1: MAC CE to selection a subset of RLC entities for PDCP duplication

Proposal 7: A fixed size MAC CE as in Figure 1 is used to select a subset of RLC entities for PDCP duplication.

# Conclusion

In this contribution, we discuss PDCP duplication with multiple RLC entities. We propose the following:

Proposal 1: Dynamic selection of RLC entities for PDCP duplication is supported for both CA only and DC+CA architectural options.

Proposal 2: In PDCP duplication, the number of copies should always be equal to the number of activated RLC entities, which means that from specification point of view, there is no need to specify the number of copies.

Proposal 3: One RLC entity is selected among the activated RLC entities to transmit PDCP Control PDU in order to keep Rel-15 principle that PDCP Control PDUs are not duplicated.

Proposal 4: Any RLC entity of a DRB can be activated/deactivated by MAC CE as long as there is at least one RLC entity activated for a given DRB.

Proposal 5: A unique index (0..3) is assigned for each RLC entity, and the RLC entity with the smallest index among the activated RLC entities is used for transmission of PDCP Control PDU.

Proposal 6: A single MAC CE is used to select RLC entities for PDCP duplication for DRBs configured with PDCP duplication and with RLC entities associated with the MAC entity (following Rel-15 principle).

Proposal 7: A fixed size MAC CE as in Figure 1 is used to select a subset of RLC entities for PDCP duplication.

# References

[1]	3GPP TR 38.825: "Study on NR Industrial Internet of Things (IoT)"

[2]	RP-190728, Nokia et al: "New WID: Support of NR Industrial Internet of Things (IoT)"

[3]	R2-1900845, vivo: "Discussion on duplication configuration"

[4]	R2-1902174, LG Electronics Inc.: "Dynamic control of PDCP duplication leg"

[5]	R2-1905749, CATT: "Dynamic leg selection with CA-only duplication"