3GPP TSG-RAN2 Meeting #107	R2-1909507

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | CRNum | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing UE capability for MDT in NR in TS 38.306 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_LTE_NR_data_collect |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the previous RAN meetings, there are many good progresses on MDT in NR. In RAN2 #106 meeting, the following was agreed on UE location information in MDT. - In Rel-16, UE shall include the GNSS location information for Logged MDT, DL signal quantities measurement of Immediate MDT, RLF report, accessibility measurement and out-of-coverage logging, if the GNSS location information is available when the measurement was taken for the UEs with the GNSS receiver. UE capability is used as one of the criteria for the RAN node or AMF to perform UE selection. It is straightforward to define the MDT related UE capabilites as in LTE. This CR has added parameters in UE Capability parameters for MDT in NR based on the agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include UE-BasedNetwPerfMeasParameters in the UE-NR-Capability IE, as in LTE. The differences from LTE: - “loggedMeasurementsIdle” is renamed as “loggedMeasurements”. - RRC_INACTIVE is added in the descriptions on logged measurements. - “loggedMBSFNMeasurements” is not included. - “locationReport” is not included. Impact analysis Impacted functionality: Logged MDT and Immdeidate MDT. Inter-operability: This CR is for UE and network to implement and the proposed parameters are included in a message sent from UE to the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network does not know if the UE supports the features of MDT when the network perform UE selection for MDT. The network may request UEs which do not support the desired features to perform MDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.14	RRC buffer size

The RRC buffer size is defined as the maximum overall RRC configuration size that the UE is required to store. The RRC buffer size is 45Kbytes.

### 4.2.15	UE-based network performance measurement parameters

#### 4.2.15.1	loggedMeasurements

This parameter defines whether the UE supports logged measurements in RRC_IDLE or RRC_INACTIVE upon request from the network. A UE that supports logged measurements in RRC_IDLE or RRC_INACTIVE shall also support a minimum of 64kB memory for log storage.

#### 4.2.15.2	standaloneGNSS-Location

This parameter defines whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE or RRC_INACTIVE.

#### 4.2.15.3	loggedMeasBT

This parameter indicates whether the UE supports Bluetooth measurements in RRC_IDLE or RRC_INACTIVE state.

#### 4.2.15.4	loggedMeasWLAN

This parameter indicates whether the UE supports WLAN measurements in RRC_IDLE or RRC_INACTIVE state.

#### 4.2.15.5	immMeasBT

This parameter indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED state.

#### 4.3.13.6	immMeasWLAN

This parameter indicates whether the UE supports WLAN measurements in RRC_CONNECTED state.

# 5	Optional features without UE radio access capability parameters