3GPP TSG-RAN WG2 #107	R2-1909689

Prague, Czech Republic, 26 – 30 August, 2019                       revision of R2-1907431

Source: 	Huawei, HiSilicon

Title: 	Discussion on CHO triggering condition

Agenda Item:	11.9.3.2	Conditional handover - executions details

Document for:	Discussion and decision

Introduction

This is a revision of R2-1907431, and there is no techncial change.

Discussion

RAN2#105bis meeting has agreed that Ax events (entry condition) are used for CHO trigger condition and A3/A5 as baseline, moreover, cell level quality is used as baseline for CHO trigger condition. For example, when A3 event like condition is configured for CHO, the UE would evaluate whether the corresponding candidate cell’s cell quality is offset higher than the source cell, if the condition is met, the UE can try to access to the cell. It is FFS that whether beam quality is also used as input for CHO trigger condition, e.g. when A3 event like condition is configured for CHO, the UE would not only evaluate whether the corresponding candidate cell’s cell quality is offset higher than the source cell, but also evaluate whether the beams belonging to the corresponding candidate cell meet the beam-level condition [3-4], e.g. whether the best beam’s quality is higher than the best one belonging to the source cell, or whether the actual number of good beams belonging to the candidate target cell is bigger than the actual number of good beams belonging to the source cell, etc. When both the cell-level condition and beam-level condition are met, the UE can determine the corresponding candidate cell is the cell that it can try to access to, but if anyone is not met, the corresponding cell is excluded as the one that the UE can handover to. It is obvious that it would introduce the UE implementation complexity since the UE would evaluate different types of trigger condition and it is strict that only when all the different types of trigger condition are met, the UE can determine the target cell. It is simple to reuse A3/A5 event for CHO trigger condition.

On the other hand, since the cell that meets the CHO trigger condition does not mean the cell is the final target cell that the UE would handover to, and we also need to specify the target cell selection rule that beam information can be taken into consideration. Therefore, it is not a good choice to take both cell-level and beam-level trigger condition into consideration when evaluating the CHO trigger condition, i.e. beam related information is excluded as input for CHO trigger condition. It is not a good choice to take both cell-level and beam-level trigger condition into consideration when evaluating whether the CHO trigger condition is met since the beam related information is the input when the UE determines the target cell to access to.

Observation 1: It increases complexity if the UE evaluates both cell-level trigger condition and beam-level trigger condition, and it is too strict that only when all the different types of trigger condition are met, the UE can determine the target cell.

Based on the above analysis, beam related information is unnecessary for CHO trigger condition evaluation, it is simple that only cell level quality is used as baseline for CHO trigger condition.

Proposal: Only cell level quality can be used as baseline for CHO trigger condition, and beam information can be excluded.

Conclusion

This paper mainly discusses whether beam information is used as input for CHO trigger condition. Based on the above discussion, we have the following proposal:

Observation 1: It increases complexity if the UE evaluates both cell-level trigger condition and beam-level trigger condition, and it is too strict that only when all the different types of trigger condition are met, the UE can determine the target cell.

Proposal: Only cell level quality can be used as baseline for CHO trigger condition, and beam information can be excluded.

References

[1] Chairman notes, “Draft Report of 3GPP TSG RAN WG2 meeting #105bis”, China.

[2] Chairman notes, “Draft Report of 3GPP TSG RAN WG2 meeting #106”, USA.

[3] R2-1903517, Triggering of Conditional Handover in NR, Ericsson

[4] R2-1903560, Triggers for Conditional Handover in NR, InterDigital Inc.