3GPP TSG-RAN WG2 #107	R2-1909891

Prague, Czech Republic , 26-30 August 2019

Agenda Item	: 11.9.2.3 (NR_Mob_enh-Core)

Source	: LG Electronics Inc.

Title	: Discussion on PDCP impact for eMOB

Document for	: Discussion and Decision

Introduction

In this contribution, we show the PDCP impacts to support 0ms interruption on UE side.

Discussion

In LTE feMOB, it was discussed whether the single PDCP entity is modelled for 0ms interruption handover or not. As a consequence, it was agreed that “any solution that is specified will be modelled as a single PDCP entity on UE side”.

However, in NR eMOB, it has not been discussed whether the single PDCP entity is modelled for 0ms interruption handover. In our view, if the UE has two PDCP entities with same DRB ID, it would complicate the specification because inter-PDCP operation in a UE side is required between two PDCP entities with same DRB ID. Therefore, it is preferred to apply the same principle in LTE feMOB to NR eMOB.

Proposal 1. For NR eMOB, a single PDCP entity model is used in UE side.

Even if the proposal 1 is agreeable, all functionalities of PDCP in UE would not be separated because, from UE point of view, it is not much different from split bearer operation.

However, from the network point of view, the PDCP entities are located in source and target network. In other words, it is possible that the PDCP PDUs can be transmitted by the source and target network, and the transmitted PDCP PDUs are ciphered with different security keys. In addition, the transmitted PDCP PDUs are compressed by different ROHC contexts. Therefore, these aspects should be considered in UE side.

Observation 1. From UE point of view, the received PDCP PDUs are ciphered with different security keys. In addition, the received PDCP PDUs are compressed by different ROHC contexts.

Security

Let’s firstly discuss the security aspect. According to the current specification, the PDCP entity maintains only one security key, and a new security key is applied only when the PDCP re-establishment is performed.

However, since a PDCP entity in UE may receive the PDCP PDUs from source and target network, the PDCP entity should maintain two security keys, and each security key should be applied for a PDCP PDU based on from which leg it is received.

Considering the above aspect, it is required that a new procedure should be defined to maintain and apply two security keys in PDCP. However, it is unclear whether the security function is separated.

In our view, if the security key is considered as the dynamic parameter for security, only one security function is enough. According to current specification, the COUNT value is one of the security parameters, and the COUNT value is always changed based on the reception of the PDCP PDU. In other words, if the PDCP entity receives the PDCP PDU, the PDCP entity determines the COUNT value and performs the security function based on the COUNT value. Consequently, considering that the PDCP entity can determine which security key should be applied to the PDCP PDU based on from which leg it is received, the security key can be easily changed even though only one security function is used.

With the above reason, we think that a new procedure for maintaining and applying two security keys should be defined with one security function in PDCP.

Proposal 2. A new procedure for maintaining and applying two security keys should be defined with one security function in PDCP.

ROHC

In order to successfully perform the decompression of the PDCP PDU, the ROHC context shall be maintained between the compressor and decompressor. However, since the PDCP entity in UE can receive the PDCP PDUs which are compressed by different ROHC contexts, the PDCP PDUs may be discarded by ROHC due to context mismatch.

In order to prevent the discard of the PDCP PDUs, the following options can be considered.

- Option 1. The decompression is performed to the PDCP PDU based on from which leg it is received.

- Option 2. The source and target network ensure successful decompression of the PDCP PDU in the UE regardless of which network compresses the PDCP PDU.

For Option 1, it is required that two sets of ROHC contexts are maintained in UE side. The PDCP entity should use one of the set of ROHC contexts for decompression for the PDCP PDU based on from which leg it is received. However, considering that the UE can be configured with up to 15 DRBs, having two sets of ROHC contexts in one PDCP entity would require huge additional UE memory.

For Option 2, it is required that only PDCP PDUs containing the IR information are transmitted by the source and target network until releasing the connection to the source network. According to the RFC 3095, the IR packet is transmitted from the compressor to the decompressor in order to replace the header context. In other words, even though the PDCP entity in UE performs the decompression of a PDCP PDU containing the IR information, the PDCP PDU is used for replacing the header context and the PDCP PDU is not discarded due to context mismatch.

As described above, Option 1 requires additional UE memory for maintaining two sets of ROHC contexts, while Option 2 does not require it. With this reason, we prefer Option 2.

Proposal 3. Only PDCP PDUs containing the IR information are transmitted by the network until the UE releases the connection to the source network.

Conclusion

In this contribution, we show the PDCP impacts to support 0ms interruption on UE side. Based on the above discussion, we propose followings.

Proposal 1. For eMOB, a single PDCP entity model is used in UE side.

Observation 1. From UE point of view, the received PDCP PDUs are ciphered with different security keys. In addition, the received PDCP PDUs are compressed by different ROHC contexts.

Proposal 2. A new procedure for maintaining and applying two security keys should be defined with one security function in PDCP.

Proposal 3. Only PDCP PDUs containing the IR information are transmitted by the network until the UE releases the connection to the source network.