3GPP TSG-RAN WG2 Meeting #107	R2-1909924

Prague, Czech Republic, 26-30 August 2019

Agenda Item:	11.4.8

Source: 	MediaTek Inc.

Title:  	RLF in sidelink

Document for:	Discussion and decision

- Introduction

In the RAN2#105 and RAN2 #106 meeting, the following agreement was made for AS Level Link Management for unicast in NR V2X sidelink.

RAN2#105 Agreements on AS Level Link Management for unicast:

1: SL RLM / RLF declaration based AS level link management is supported.

4: The AS level link status (e.g., failure) should be informed to upper layer. The detailed information exchanged between layers should be decided together with SA2.

5: If SL RLC AM is supported for unicast, RLF declaration could be triggered by indication from RLC that the maximum number of retransmissions has been reached.

RAN2#106 Agreements on PC5 RLM/RLF:

1: 	Even though transmission of sidelink signal occur irregularly, RAN2 assumes that the physical layer provides periodic indications of IS/OOS to the upper layer as in Uu RLM.

2:	From RAN2 perspective, both side UEs perform RLM/RLF detection mechanism. FFS on whether periodic indications of IS/OOS based RLM/RLF is reused or any additional new mechanism is needed.

We discuss some aspects about RLF in sidelink in this paper.

- Radio Link Failure

Radio Link Failure in Uu interface

There are three triggering conditions for RLF in Uu interface, we list them below:

- 1) Upon indication from RLC that the maximum number of retransmissions has been reached

- 2) Upon expiry of Timer T310 (this timer is started when physical layer problems are detected i.e. upon receiving N310 consecutive out-of-sync indications from lower layers)

- 3) Upon random access problem indication from MAC while neither T300, T301, T304 nor T311 is running

We start the analysis from above three conditions, for TX UE in unicast, only the first one condition can trigger RLF. For RX UE in unicast and groupcast, only the second condition can trigger RLF. The third condition is not suitable in sidelink since there is no RACH in sidelink. For TX UE in groupcast, there is no triggering condition in this case, since RLC AM is not supported for groupcast. We summarize them below:

|  | TX side | RX side |
| --- | --- | --- |
| Unicast | 1) | 2) |
| Groupcast | N.A. | 2) |

Observation 1: not all of the RLF triggering conditions in Uu interface can be reused in PC5 interface.

Proposal 1: RAN2 to consider whether enhancement on RLF is needed or not.

If proposal 1 is agreed and enhancement on RLF is needed, the following mechanisms could be considered:

- Count of received number of HARQ NACK.

  - PC5 RLF can be determined in consideration of the HARQ NACK count received from the RX UE.

- Count times that no feedback is received.

  - If the TX UE doesn’t receive the feedback message (i.e., HARQ ACK/NACK) from the RX UE, TX UE can decide the RLF of PC5 radio link. This condition may be considered as bad situation for the PC5 Radio Link. Specifically, the situation is that the RX UE fails to receive control channel transmitted by the TX UE (i.e., the channel that transmits scheduling information for data transmission), and the RX UE may not transmit feedback message to the TX UE.

- Count number of sent HARQ feedback from RX UE.

  - This mechanism considers the cases when TX UE ignores the feedback from RX UE under out of range case. RX UE can count the number of sent HARQ feedback to trigger RLF.

- Count number of expiry discard Timer.

  - This mechanism consider the RLF from PDCP layer, which counts how many PDCP SDUs are not delivered successfully within the discard timer.

Proposal 2: RAN2 to consider enhancements on RLF including above mechanisms.

- Conclusion

In this paper, we discuss enhancement on RLF, the following observation and proposals are proposed:

Observation 1: not all of the RLF triggering conditions in Uu interface can be reused in PC5 interface.

Proposal 1: RAN2 to consider whether enhancement on RLF is needed or not.

Proposal 2: RAN2 to consider enhancements on RLF including above mechanisms.