3GPP TSG-RAN2 Meeting #107	R2-1909988

Prague, Czech Republic, 26 - 30 August 2019

Agenda Item:	11.11.5 UE assistance

Source: 	Ericsson

Title:  	UE Assistance Information for Power Preference Indication

Document for:	Discussion and Decision

# Introduction

One of the objectives of the updated NR UE power saving WID, including RAN2 topics, specifies UE assistance information for power preferred information (baseline LTE PPI in a well-defined manner) [1]:

- Specify, if agreed, the mechanism to provide UE assistance information [RAN2, RAN1]:

- Study and select among the following UE assistance information for RAN2 by RAN#85:

power preferred information (baseline LTE PPI in a well-defined manner) and UE's preference on C-DRX, BWP and SCell configuration [RAN2].

UE assistance signalling to indicate power preference information is discussed further in this contribution.

The power preference indication is assumed to be a single bit, i.e. binary (similar as the PPI in LTE). Other UE assistance information (e.g. related to cDRX, BWP and SCell configuration) that contains more information and thus requires multiple bits, is discussed in other contributions [2-4].

# Background

LTE

The UE signals support of power preference indication via UE capability signalling (powerPrefInd-r11). The NW can configure UE assistance signalling for power preference indication via powerPrefIndicationConfig IE in OtherConfig IE in RRCConnectionReconfiguration and RRCConnectionResume message.

If configured the UE may send UEAssistanceInformation message including powerPrefIndication to lowPowerConsumption or normal. The UE may send such indication upon configuration or upon change of power preference. The UE also sends the UEAssistanceInformation message after HO, when it has sent power preference indication just prior to HO (1 second preceding RRCConnectionReconfiguration message including mobilityControlInf).

The UE is not allowed to send UEAssistanceInformation message including powerPrefIndication when T340 (powerPrefIndicationTimer IE in powerPrefIndicationConfig IE) is running. T340 can be configured with {0, 0.5, 1, 2, 5, 10, 20, 30, 60, 90, 120, 300, 600} sec. T340 is set when UE indicates powerPrefIndication set to normal.

The NW considers that the UE prefers a configuration primarily optimised for power saving when powerPrefIndication set to lowPowerConsumption. When the eNB receives a power preference indication lowPowerConsumption the eNB may use this to (re-)configure connected mode parameters (e.g. cDRX) or for connection release handling. When configured with power preference the NW does not consider that the UE prefers a configuration optimized for power saving, until it receives lowPowerConsumption indication.

PPI discussions during study item phase

The introduction of a single bit power preference indication in NR, similar as in LTE, was discussed during email discussion [105bis#28] and reported in [5]. Thirteen companies indicated (some) support, and eight companies indicated no support. The companies that were not supportive, indicated the short comings of the LTE PPI:

- Only “binary” info, i.e. too coarse information from the UE. Some companies indicate a preference to signal more detailed information (e.g. preferred DRX cycle length)

- It is not clear how and when the UE sets the bit. Most likely there will be different UE implementations that signal “lowPowerConsumption” in different cases. Among UE vendors there may be different expectations how the NW acts on the bit (e.g. release the UE or increase DRX cycle length, etc).

# Discussion

Perhaps the limitation that only “binary” information can be signaled via PPI in LTE is not the worst, but the fact that it is not clear how/when the UE sets the bit, and what the UE expects to happen after the indication. It is likely that there will be different UE implementations. Perhaps the UE would prefer a longer DRX, and not to be released. It such case it would then be unfortunate when the NW would release the UE when indicating PPI.

It is not clear whether there is a strong need to introduce PPI-like signalling in NR, i.e. “binary” UE assistance information. RAN2 is discussing more detailed UE assistance information signalling (e.g. for cDRX, BWP and SCell configuration), and it is not clear if a simple 1-bit signalling is needed. In any case it should be clear when the UE sets the bit, and what the UE expects to happen (even when any subsequent action is up to NW implementation):

Proposal 1: RAN2 to discuss specific UE assistance information for UE preferred cDRX, BWP and SCell configuration

# Summary

RAN2 is kindly asked to discuss power preference indication:

Proposal 1: RAN2 to discuss specific UE assistance information for UE preferred cDRX, BWP and SCell configuration

# References

- RP-191607, UE Power Saving in NR, (new) WID, CATT, RAN#84

- R2-1909989, UE Assistance Information for UE preferred BWP configuration, Ericsson, DISC; RAN2#107

- R2-1909990, UE Assistance Information for UE preferred cDRX configuration, Ericsson, DISC; RAN2#107

- R2-1909991, UE Assistance Information for UE preferred SCell configuration, Ericsson, DISC; RAN2#107

- R2-1906426, Report on [105bis#28][NR/Power Savings] UE assistance, Intel, Report, RAN2#106