3GPP TSG-RAN WG2 #107	R2-1910023

Prague, Czech, 26 August - 30 August 2019

Agenda Item:	11.7.4

Source:	OPPO

Title:	Discussion on data duplication enhancement in IIoT

Document for:	Discussion, Decision

Introduction

According to the WID of NR IIoT [1], the WI should address the following objectives for NR PDCP duplication enhancements in Rel-16:

- Specify PDCP duplication with up to 4 RLC entities configured by RRC in architectural combinations including CA only and NR-DC in combination with CA [RAN2, RAN3].

- Specify mechanisms relating to dynamic control of how a set or subset of configured RLC entities or legs are used for PDCP duplication [RAN2, RAN3].

- Specify enhancements for more resource efficient PDCP duplication by enhancing PDCP duplication activation/deactivation mechanisms (e.g. MAC CE based or based on UE configurable criteria), provided that complexity increase is reasonable. Per-packet selective duplication can also be considered. [RAN2].

- Specify enhancements for more efficient DL PDCP duplication without impacting the UE, provided that gains can be confirmed with a reasonable complexity. [RAN3].

- Specify enhancements to address potential impacts of higher-layer multi-connectivity based on SA2 progress and request [RAN2, RAN3].

According to the conclusion in RAN2#106[2], preliminary agreements reached are in the following:

- Intention is that copies are sent on different legs

- Dynamic network control of DRB duplication is by MAC CE

- MAC CE controls which of the configured RLC entities are activated/deactivated

- Support the case that number of copies equals the number of active RLC entities

In this paper, we will further discuss the issue on data duplication enhancement and give our proposals.

Discussion

Scenario

According to the description in IIoT WI, one objective of PDCP duplication enhancement is to support up to 4 RLC entities in architectural combination including CA only and NR-DC combination with CA. One further issue is what the basic architecture is for NR-DC combination with CA. Based on our understanding, the potential architecture is illustrated in Figure 1. In this architecture, to support 4 legs duplication configuration, each MAC entity is configured associated with 2 RLC entities. We think it might be a most straightway extension based on legacy CA or DC duplication architecture for “4 legs” scenario. Thus, it is clear that this architecture is a baseline for NR-DC combination with CA in R16.

Figure 1 NR-DC combination with CA achitecture

To support up to 4 RLC entities in architectural combination including CA only and NR-DC combination with CA.

It is clear that the architecture illustrated in Figure 1 (i.e., at most two legs in each of the two CGs) is to be supported in duplication enhancement.

RAN2 confirms the architecture illustrated in Figure 1(i.e., at most two legs in each of the two CGs) as a baseline for NR-DC combination with CA.

Obviously, legacy DC or CA architecture is applicable in IIoT. However, it is unclear whether it is necessary to support more than 2 RLC entities in CA only architecture. If so, based on our understanding, the potential architecture is that more than 2 RLC entities are configured associated with one MAC entity for one bearer, which is illustrated in the left of Figure 2. Compared with the legacy CA duplication, the only difference, from architecture point of view, is the number of associated RLC entity. In addition, if the above enhancement on CA only architecture is agreed, another NR-DC combination with CA architecture might be considered, which is illustrated in the right of Figure 2. The only difference is the number of associated LCHs per MAC entity compared to the one in Figure 1. And we propose to exclude this architecture out of R16 or put it into the second priority considering possible benefits, complexities, and time restrictions.

Figure 2 Potetial duplication achitecture

Maximum number of duplication copies

It is a common understanding that up to 4 copies/configured legs will be supported in IIoT. If all the configured legs can be used simultaneously for duplication, higher reliability requirement can be achieved while resource overhead is higher than the one in R15. However, resource efficiency is always an important issue for Uu interface transmission. Thus, preform PDCP duplication efficiently is another important objective for “more than 2 copies” scenario. Furthermore, considering the most stringent reliability requirement is 1-10-6~1-10-8 in TR 22.804 and 1-10-6 reliability is part of targets to evaluate in L1 URLLC enhancements SI, we propose that 2-copies is enough to cover the most stringent use case. In addition, considering uplink power limited scenario, it is a better way transmit copies on fewer carriers in good condition with more power, than transmitting copies on more carriers with smaller power.

Resource efficiency and reliability needs to be trade-off for PDCP duplication efficiency.

2-copie is enough to cover the most stringent use case.

2-copie is sufficient to duplication transmission in “more than 2 copies” scenario.

According to the agreement achieved in RAN2#106, the number of copies should equal to the number of active RLC entities. Hence, the maximum number of the activated legs is to be 2.

At most 2 legs are activated simultaneously for duplication transmission considering the reliability requirement and resource consumption.

MN-SN coordination

In R15, duplication transmission coordination between nodes is not specified. Considering the coordination latency due to non-ideal backhaul between nodes and the coordination complexity in multi-SN duplication scenario which might be supported in the future, the multi-node coordination performance might not be desirable. Thus, from our point of view, the multi-node coordination might not be needed for duplication transmission in more than 2 copies” scenario.

Whatever, it is in RAN3 scope and let us waiting for RAN3 conclusion.

In R15, multi-node coordination is not specified for duplication transmission.

In “more than 2 copies” scenario, the multi-node coordination performance might be undesirable due to the complexity and latency of coordination.

Design on duplication activation/deactivation control

In legacy system, duplication activation/deactivation MAC CE (i.e. R15 duplication MAC CE) is defined for duplication status indication. By using R15 duplication MAC CE, all legs are aligned with the same behavior, i.e. all legs are activated or not simultaneously. For example, all legs are to be used for duplication transmission if duplication is activated, otherwise only the primary leg is applicable. However in R16, it is agreed that to dynamic control of a subset of configured RLC entities or legs for PDCP duplication transmission. Thus, the mechanism designed in R15 is not aligned to the objective in R16.

By using R15 duplication MAC CE, all legs are aligned with the same behavior, i.e. all legs are activated or not simultaneously.

Re-using existing Duplication Activation/Deactivation MAC CE for leg indication is not applicable for duplication enhancement in R16.

As concluded in RAN2#106, a new MAC CE is introduced for more than two legs duplication scenario, which is used to activate or deactivate the configured RLC entities. Accordingly, a new LCID should be introduced for R16 duplication MAC CE.

Introducing new MAC CE is approved for “more than 2 copies” scenario, which is used to activate or deactivate the configured RLC entities.

A new LCID should be introduced for R16 duplication MAC CE for “more than 2 copies” scenario.

Based on the above observations, two MAC CEs (R15 duplication MAC CE and R16 duplication MAC CE) exist in R16 to activate/deactivate the configured RLC entities. Two candidate ways are listed in the following for R16 MAC CE functionality design:

- One way is to design a MAC CE only used for sort of duplication status indication. For example, R15 DUPLICATION MAC CE is used to indicate activated or not, and the new MAC CE is designed for leg selection. The first drawback is how to split the R15 / R16 duplication MAC CE functionality, and another one is much complexity are introduced to the network and UE since they need to recognize 2 MAC CE.

- The other way is to design a MAC CE used to indicate activation/deactivation status and any status of multiple legs combination, including one leg activation, 2 legs activation, 3 legs activation, and 4 legs activation. The Pros of this approach is there is no need to consider how to split functionality as required in the above way, and all possible duplication cases can be covered by using this approach.

Table 1 Pros and Cons for potential ways for new MAC CE design

There are two potential solutions for R16 duplication MAC CE design.

A single duplication MAC CE (i.e. R16 duplication MAC CE) is proposed for duplication information indication for “more than 2 copies” scenario.

As we outlined in section 2.3, the multi-node coordination performance might be undesirable due to the complexity and latency. Hence, only legs information associated to one transmission node is necessary to indicate in one R16 duplication MAC CE, since the leg information of another node is unknown by the this node. In details, R16 duplication MAC CE sent by one node would include the leg information associated to the same node. For example, 4 LCHs/RLC entities are configured for DRB1, where LCH 1/LCH 2 are associated to MN, LCH 4/LCH 5 are associated to SN. If R16 duplication MAC CE is sent from MN, only the leg activation/deactivation information of LCH1 and LCH 2 needs to be included/indicated.

R16 duplication MAC CE sent by one node would include leg information associated to the same node.

As we proposed, the architecture of at most two legs in each of the two CGs is as a baseline for NR-DC combination with CA. 2-octct is enough to R16 duplication MAC CE to indicate leg status associated to one node. Considering the possibility of leg activation/deactivation altering indication for more than one DRBs, the most straightforward way is to extend existing duplication activation/deactivation MAC CE (i.e. R15 duplication MAC CE).

Extend existing duplication MAC CE (i.e. R15 duplication MAC CE) for “more than 2 copies” scenario.

Conclusion

Based on the discussion above, we made the following observations:

Observation 1	To support up to 4 RLC entities in architectural combination including CA only and NR-DC combination with CA.

Observation 2	It is clear that the architecture illustrated in Figure 1 (i.e., at most two legs in each of the two CGs) is to be supported in duplication enhancement.

Observation 3	Resource efficiency and reliability needs to be trade-off for PDCP duplication efficiency.

Observation 4	2-copie is enough to cover the most stringent use case.

Observation 5	In R15, multi-node coordination is not specified for duplication transmission.

Observation 6	In “more than 2 copies” scenario, the multi-node coordination performance might be undesirable due to the complexity and latency of coordination.

Observation 7	By using R15 duplication MAC CE, all legs are aligned with the same behavior, i.e. all legs are activated or not simultaneously.

Observation 8	Re-using existing Duplication Activation/Deactivation MAC CE for leg indication is not applicable for duplication enhancement in R16.

Observation 9	Introducing new MAC CE is approved for “more than 2 copies” scenario, which is used to activate or deactivate the configured RLC entities.

Observation 10	There are two potential solutions for R16 duplication MAC CE design.

And propose the following:

Proposal 1	RAN2 confirms the architecture illustrated in Figure 1(i.e., at most two legs in each of the two CGs) as a baseline for NR-DC combination with CA.

Proposal 2	2-copie is sufficient to duplication transmission in “more than 2 copies” scenario.

Proposal 3	At most 2 legs are activated simultaneously for duplication transmission considering the reliability requirement and resource consumption.

Proposal 4	A new LCID should be introduced for R16 duplication MAC CE for “more than 2 copies” scenario.

Proposal 5	A single duplication MAC CE (i.e. R16 duplication MAC CE) is proposed for duplication information indication for “more than 2 copies” scenario.

Proposal 6	R16 duplication MAC CE sent by one node would include leg information associated to the same node.

Proposal 7	Extend existing duplication MAC CE (i.e. R15 duplication MAC CE) for “more than 2 copies” scenario.

References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), 3GPP RAN#81, Gold Coast, Australia, September 10-13, 2018

RP-190728, New WID: Support of NR Industrial Internet of Things (IoT), 3GPP RAN#83, Shenzhen, China, March 18-21, 2019

TR 38.285 v0.2.0

Draft_RAN2_106_Report_v3