3GPP TSG-RAN WG2 Meeting #107	R2-1910181

Prague, Czech Republic, 26th - 30th August 2019

Agenda Item:	12.2.10.2

Source:		Huawei, HiSilicon

Title:	Support of RRC connection Re-establishment for the Control plane for NB-IoT connected to 5GC

Document for:	Discussion and Decision

# Introduction

RAN2#106 discussed RRC Connection Re-establishment for the CP solution for a NB-IoT UE connected to 5GC and made the following agreements:

RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is supported, assuming feasibility is confirmed. Send a LS to SA3, SA2 CC: RAN3, CT1, CT4 - Ask whether they can introduce RRC Connection re-establishment for the Control plane for NB-IoT connected to 5GC. - Ask the feasibility that a truncated 5G S TMSI (e.g. 40 bits) needs to be defined.

RAN2 sent a LS in [1]. SA2 and SA3 replied as follows [2], [3]:

SA2:

SA2 thanks RAN2 for their LS on RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC. SA2 discussed the introduction of RRC Connection Re-Establishment for control plane for NB-IoT connected to 5GC and believe it is feasible. SA2 will further work on the identifier size issue and will notify of any conclusions.

SA3:

SA3 thanks RAN2 for their LS on RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC. SA3 has discussed the LS and agreed to study the security of RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC. As part of the study, the security impacts of truncated 5G S-TMSI will be considered.

SA3 has introduced Key issue #13 Security Handling in RRC Connection Re-Establishment for the control plane for NB-IoT connected to 5GC and solution 20: RRC Connection Re-Establishment for the Control Plane for NB-IoT connected to 5GC in TR 33.861 [4].

Based on the above, our understanding is that SA2 and SA3 have concluded that the introduction of RRC Connection Re-Establishment for control plane for NB-IoT connected to 5GC is feasible but have not concluded on how to define a shortened version of the 5G-S-TMSI.

In this document, we discuss how to support of RRC Connection Reestablishment for the CP solution for NB-IoT connected to 5GC, pending confirmation from SA2 and SA3.

# Discussion

We propose first to confirm the agreement from RAN2#106 based on SA2 reply:

Proposal 1: RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is feasible.

Then we look at the impact to support in RAN2 specification:

Same as in Rel-15 NB-IoT, RRC Connection re-establishment for the control plane is initiated by the UE and thus there is no need for capability reporting. On the other hand, an indication in system information that it is supported by the eNB is needed as the UE should not send a message not understood by the network.

Proposal 2: RRC Connection re-establishment for the control plane for NB-IoT UEs connected to 5GC is optional, without capability reporting.

Proposal 3: Introduce an indication cp-reestablishment-5GC in SystemInformationBlockType2-NB to indicate eNB support when connected to 5GC.

As there is a need for the eNB to know that the re-establishment request is for 5GC and that the UE identity will be different from legacy, we propose to introduce a new critical extension of RRCConnectionReestablishmentRequest-NB message for CP re-establishment when connected to 5GC.

Proposal 4: For NB-IoT connected to 5GC, create a critical version of RRCConnectionReestablishmentRequest-NB message for CP CIoT 5GS optimisation, including the ue-Identity, re-establishmentCause, and cqi-NPDCCH (short).

In solution 20 in TR 33.681 [4], SA3 proposes to reuse the same security solution as in EPS when there is no KAMF change. In case of KAMF change, a new parameter needs to be provided in RRCConnectionReestablishment message for KAMF key derivation.

Proposal 5: For NB-IoT connected to 5GC, define a new IE ReestabUE-Identity-CP-5GC-NB in RRCConnectionReestablishmentRequest-NB message, including shortNg-5G-S-TMSI (40 bits), ul-NAS-MAC (16 bits) and ul-NAS-Count (5 bits).

Proposal 6: For NB-IoT connected to 5GC, introduce a new parameter nas-SecurityParam-5GC-r16, optional, in RRCConnectionReestablishment-NB message that contains the key derivation parameters. Details FFS.

In Rel-15 eLTE, UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment. This has been motivated by the need of cross-CN reestablishment, with many RAN3 and SA2 impacts.

By analogy with Rel-15 eLTE and for the same reasons, the same should apply to NB-IoT and eMTC.

Proposal 7: for NB-IoT, the UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment for the control plane.

# Conclusion

In this document, we have discussed RRC Connection Re-establishment for the control plane solution for NB-IoT connected to 5GC and made the following proposals:

Proposal 1: RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is feasible.

Proposal 2: RRC Connection re-establishment for the control plane for NB-IoT UEs connected to 5GC is optional, without capability reporting.

Proposal 3: Introduce an indication cp-reestablishment-5GC in SystemInformationBlockType2-NB to indicate eNB support when connected to 5GC.

Proposal 4: For NB-IoT connected to 5GC, create a critical version of RRCConnectionResumeRequest-NB message for CP CIoT 5GS optimisation, including the ue-Identity, re-establishmentCause, and cqi-NPDCCH (short).

Proposal 5: For NB-IoT connected to 5GC, define a new IE ReestabUE-Identity-CP-5GC-NB in RRCConnectionReestablishmentRequest-NB message, including shortNg-5G-S-TMSI (40 bits), ul-NAS-MAC (16 bits) and ul-NAS-Count (5 bits).

Proposal 6: For NB-IoT connected to 5GC, introduce a new parameter nas-SecurityParam-5GC-r16, optional, in RRCConnectionReestablishment-NB message that contains the key derivation parameters. Details FFS.

Proposal 7: For NB-IoT, the UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment for the control plane.

# References

R2-1908264 LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC, RAN2,  RAN2#106, Reno, USA, May 2019

R2-1908659 “Reply on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC”, SA2. SA2#134, Sapporo, Japan, June 2019

R2-1908669 “Reply on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC”, SA3. SA3#95bis, Sapporo, Japan, June 2019

S3-192392 TR 33.861 v120 Study on evolution of Cellular IoT security for the 5G System