3GPP TSG-RAN WG2 #107	 TDoc R2-1910296

# Prague, Czech, 26th – 30th August 2019

Agenda Item:	11.4.2

Source:	Ericsson

Title:	On support of HARQ procedure over sidelink

Document for:	Discussion, Decision

# Introduction

The study of the unicast and groupcast sidelink V2X communications is included in the WID [1]. It has been agreed that HARQ feedback will be supported for SL unicast and groupcast. Besides, in RAN1 the following agreements related to HARQ procedure for both mode 1 and mode 2 were made.

Agreements: - (Pre-)configuration indicates whether SL HARQ feedback is enabled or disabled in unicast and/or groupcast. - When (pre-)configuration enables SL HARQ feedback, FFS whether SL HARQ feedback is always used or there is additional condition of actually using SL HARQ feedback. Agreements: - (Pre-)configuration indicates the time gap between PSFCH and the associated PSSCH for Mode 1 and Mode 2. Agreements: - A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB. - A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. - UE decides which TB to transmit in each of the occasions indicated by a given configured grant. - FFS: whether different transmissions of a TB can take place across multiple configured grants. - Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2. Agreements: - Sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported with details FFS. - Note: this reverts the following agreement from RAN1#96: - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16. - SR/BSR report to gNB for the purpose of requesting resources for HARQ retransmission is not supported. Agreements: - NR sidelink does not support performing different transmissions of a TB using different configured grants. Agreements: - For mode 1: - A dynamic grant by the gNB provides resources for transmission of PSCCH and PSSCH. Agreements: - At least for the case when the PSFCH in a slot is in response to a single PSSCH: - Implicit mechanism is used to determine at least frequency and/or code domain resource of PSFCH, within a configured resource pool. At least the following parameters are used in the implicit mechanism: - Slot index (FFS details) associated with PSCCH/PSSCH/PSFCH - Sub-channel(s) (FFS details) associated with PSCCH/PSSCH - Identifier (FFS details) to distinguish each RX UE in a group for Option 2 groupcast HARQ feedback - FFS detailed applicability of the above parameters - FFS: Other parameters (e.g. SL-RSRP/SINR, Layer-1 source ID, location information, etc.) Agreements: - NR V2X Mode-2 supports resource reservation for feedback-based PSSCH retransmissions by signaling associated with a prior transmission of the same TB - FFS impact on subsequent sensing and resource selection procedures - At least from the transmitter perspective of this TB, usage of HARQ feedback for release of unused resource(s) is supported - No additional signaling is defined for the purpose of release of unused resources by the transmitting UE - FFS the behavior of the receiver UE(s) of this TB and other UEs

In general, the design of HARQ procedures affects the physical layer design, but also the L2 protocols. In this paper, we further discuss some of the technical aspects related to SL HARQ procedures, with focus on design of HARQ procedures over SL, for both mode 1 and mode 2 operations.

# Discussion

RAN1 agreed that SL HARQ feedback is enabled/disable by (pre-)configuration, leaving the possibility of having additional conditions for disabling it. In our view, the use of SL HARQ feedback should mostly depend on QoS and, for Mode-2 UEs, on congestion control. For Mode-1 UEs, congestion control should be considered by the gNB when allocating resources and, consequently, the UE should not apply congestion control to Mode-1 grants. Note that the gNB can determine the congestion level via UE reports such as CBR reports.

The use of SL HARQ feedback should mostly depend on QoS. In addition, for Mode-2 UE, it should depend on congestion control.

Given that for Mode-1 UEs, the gNB is in control of the grants and QoS, we believe that the use of HARQ feedback should be decided by the gNB alone. For Mode-2 UEs, the choice should naturally be made by the UE.

For Mode 1 UEs, the use of HARQ feedback is configured by the network. For Mode 2 UEs, the transmitter UE decides whether to request feedback.

The RAN1 agreements do not specify whether the (pre-)configuration indicating if SL HARQ feedback is enabled or disabled are per UE, pool, SLRB/LCH, etc. In our understanding, the motivation to have such enabled/disabled feedback flexibility is to provide different configurations in case of different service requirements. For instance, for services that request high reliability and can tolerate high latency, enabled HARQ feedback is the most proper configuration. On the contrary, for services of low reliability and low latency requirements, the HARQ feedback can be disabled. Therefore, we propose SLRB/LCH specific HARQ configuration.

SLRB/LCH specific HARQ configuration is supported.

In addition, given that HARQ feedback resources at PHY (i.e., PSFCH) are configured per pool, it is straightforward that some pools will (not) support HARQ feedback. This is only relevant for Mode 2, where the UE may be configured with multiple TX pools at the same time. We discuss issues related to pools, resource allocation, and HARQ in Section 2.2.

PHY HARQ feedback resources are configured per pool.

To operate SL HARQ feedback properly, the transmitter UE must include a field in SCI indicating whether HARQ feedback is requested or not. Although in principle the receiver UE can obtain this information from the LCH configuration, there are multiple reasons why this indication needs to be conveyed by lower-layer signalling:

- The receiver UE does not know the LCH(s) until it has decoded the TB and, consequently, it would not know whether to transmit a NACK message or not if it could not decode the TB.

- The LCH configuration only indicates whether SL HARQ feedback is enabled or not. However, additional restrictions may apply, for example, for Mode-2 UEs in congested scenarios. Conveying a HARQ NACK when the transmitter has not possibility to retransmit the TB (e.g., due to congestion control) would just create unnecessary interference in the PSFCH resource.

SCI carries a field indicating the  request of HARQ feedback i.e. ACK/NACK.

This principle presented in this section is valid for both Mode 1 and Mode 2. For Mode 1 and Mode 2 operation, some particular issues are further discussed in the next section.

## Mode 1

To support mode 1 SL HARQ procedure, when gNB receives an ACK/NACK feedback from UE, gNB should be able to identify the corresponding SL grant that was previously assigned to UE. As such, gNB may provide a new SL grant of the same size that can fit the TB in the HARQ buffer waiting for retransmission.  Note that if the provided SL grant mismatches the existing TB size, UE cannot retransmit the TB.

gNB should be able to link the received ACK/NACK feedback with previously assigned SL grant.

One feasible way to enable such linkage (i.e., between the received ACK/NACK feedback and the SL grant) is that gNB can indicate to UE the given SL grant is expecting an ACK/NACK feedback via a specific PUCCH resource. The association between the SL grant and PUCCH resource can be deterministic to be defined in RAN1. From RAN2 perspective, it is worth investigation how to ensure the given SL grant expecting ACK/NACK feedback will not be used for LCHs which HARQ feedback is disabled. Note that, grants for NR uplink transmission are not associated with a specific LCID. The existing specification, however, allows for restricting the LCHs transmitted using the UL grant based on a few parameters. More specifically:

The MAC entity shall, when a new transmission is performed: 1> select the logical channels for each UL grant that satisfy all the following conditions: 2> the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and 2> maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and 2> configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and 2> allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) for which PDCP duplication is deactivated. NOTE: The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

We believe that the logical channel selection framework above must be extended to accommodate SL transmissions with and without HARQ feedback. To this end:

- Each sidelink grant indicates whether SL HARQ feedback is to be requested by the transmitter UE or not.

  - For dynamic grant, this is indicated in DCI. Signaling details are up to RAN1.

  - For configured grant, this is part of the RRC configuration of the grant.

- In terms of MAC PDU multiplexing, the following options are feasible:

  - Only LCHs with same SL HARQ behavior are multiplexed together.

  - LCHs with different SL HARQ behaviors can be multiplexed into the same MAC PDU. Whether the generated MAC PDU requests ACK/NACK feedback follows what indicated by the NW, and LCHs request HARQ feedback cannot be multiplexed into a MAC PDU does not request HARQ feedback.

Given a grant (dynamic or configured), the UE must follow the indication regarding whether to request SL HARQ feedback or not. This is important because, per RAN1 agreements, the SL HARQ feedback is reported to the gNB by the transmitter UE. If the decision of requesting feedback were left to the UE, the gNB would not know whether to expect HARQ ACK/NACK report or not.

For dynamic grant, DCI indicates whether SL HARQ feedback is to be requested or not. Signalling details up to RAN1.

For configured grant (type-1 and type-2), RRC configuration indicates whether SL HARQ feedback is to be requested or not.

For Mode 1 MAC PDU multiplexing, RAN2 discusses the following two options:

  - Only LCHs with same SL HARQ behavior are multiplexed together.

  - LCHs with different SL HARQ behaviors can be multiplexed into the same MAC PDU. Whether the generated MAC PDU requests ACK/NACK feedback follows what indicated by the NW, and LCHs request HARQ feedback cannot be multiplexed into a MAC PDU does not request HARQ feedback.

In Figure 1, we have illustrated the use of two configured grants, one with SL HARQ feedback resources (blue) and one without them, for transmission of different LCHs.

Figure 1. In blue (red), a configured grant with (without) SL HARQ feedback resource used for transmission of TBs carrying LCHs for which SL HARQ feedback is enabled (disabled).

So far, we discuss an aspect related to the interaction between gNB and UE for retransmission. For UL transmissions, both gNB and UE know whether a grant is used for transmission of a new TB or for a retransmission of an existing TB. For sidelink, when gNB assigns a SL grant for UE to retransmit an existing MAC PDU in the HARQ buffer, the grant size has to match a combination of TB size and MCS. One simple way to ensure that is to provide a SL grant of the same size as the previous SL grant for initial transmission. On the other hand, UE has to understand this particular SL grant is for retransmission only while cannot be used for any other initial transmission. For example, assuming gNB assigns SL grant#1 to retransmit TB#1 and waits for ACK/NACK feedback, but UE uses SL grant#1 to generate a new TB#2 and the transmission is successful. In this case, gNB will receive an ACK feedback for SL grant#1 and assumes the related retransmission succeed. Afterwards, gNB will no longer provide SL grant in a way to match the size of TB#1. As a consequence, TB#1 may no longer get a suitable SL grant for retransmission.

Therefore, it is beneficial to introduce another indicator conveyed in e.g. DCI indicating if the given SL grant is for retransmission only or initial transmission only or both are allowed. As one alternative, the indicator can be a flag that represents “retransmission only”, “initial transmission only”, or “initial transmission and retransmission”. As another alternative, the indicator can simply be HARQ process ID, as such gNB fully controls UE’s SL transmission, and a SL grant is given for a specific SL HARQ process.

RAN2 discusses the need to indicate if a Mode 1 SL grant can only be used for initial transmission or retransmission or both. FFS the indicator can be either a flag or a SL HARQ process ID.

## Mode 2

Given that SL HARQ is enabled/disabled for each LCH, when multiplex data from multiple LCHs into the same MAC PDU, there is an issue if what HARQ behaviour will the MAC PDU inherit (e.g., enabled/disabled, ACK/NACK, NACK only). For Mode 2 operation, we believe there are three options:

- Only LCHs with same SL HARQ behavior are multiplexed together.

- LCHs with different SL HARQ behaviors can be multiplexed into the same MAC PDU. LCHs request HARQ feedback cannot be multiplexed into a MAC PDU does not request HARQ feedback.

For Mode 2 MAC PDU multiplexing, RAN2 discusses the following three options:

  - Only LCHs with same SL HARQ behaviour are multiplexed together.

  - LCHs with different SL HARQ behaviours can be multiplexed into the same MAC PDU. LCHs request HARQ feedback cannot be multiplexed into a MAC PDU does not request HARQ feedback.

# Conclusions

In section 2 we made the following observations:

Observation 1	The use of SL HARQ feedback should mostly depend on QoS. In addition, for Mode-	2 UE, it should depend on congestion control.

Observation 2	PHY HARQ feedback resources are configured per pool.

Observation 3	gNB should be able to link the received ACK/NACK feedback with previously assigned SL grant.

Based on the discussion in section 2 we propose the following:

Proposal 1	For Mode 1 UEs, the use of HARQ feedback is configured by the network. For Mode 2 UEs, the transmitter UE decides whether to request feedback.

Proposal 2        SLRB/LCH specific HARQ configuration is supported.

Proposal 3        SCI carries a field indicating the  request of HARQ feedback i.e. ACK/NACK.

Proposal 4	For dynamic grant, DCI indicates whether SL HARQ feedback is to be requested or not. Signalling details up to RAN1.

Proposal 5	For configured grant (type-1 and type-2), RRC configuration indicates whether SL HARQ feedback is to be requested or not.

Proposal 6        For Mode 1 MAC PDU multiplexing, RAN2 discusses the following two options:

- Only LCHs with same SL HARQ behavior are multiplexed together.

- LCHs with different SL HARQ behaviors can be multiplexed into the same MAC PDU. Whether the generated MAC PDU requests ACK/NACK feedback follows what indicated by the NW, and LCHs request HARQ feedback cannot be multiplexed into a MAC PDU does not request HARQ feedback.

Proposal 7	RAN2 discusses the need to indicate if a Mode 1 SL grant can only be used for initial transmission or retransmission or both. FFS the indicator can be either a flag or a SL HARQ process ID.

Proposal 8	For Mode 2 MAC PDU multiplexing, RAN2 discusses the following three options:

- Only LCHs with same SL HARQ behaviour are multiplexed together.

- LCHs with different SL HARQ behaviours can be multiplexed into the same MAC PDU. LCHs request HARQ feedback cannot be multiplexed into a MAC PDU does not request HARQ feedback.

# References

RP-190766, New WID on 5G V2X with NR sidelink, 3GPP TSG RAN83, March 18-21, 2019.