3GPP TSG-RAN WG2 Meeting #107	R2-1910540

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.2.2.2

Source: 	Fraunhofer HHI, Fraunhofer IIS

Title:  	RRM Measurements for Mobility in NR-U

Document for:	Discussion and decision

# Introduction

In RAN2#105bis, the following agreements related to connected mode measurements in NR-U were made based on discussions in [1]

R2 assumes that missing measurements due to LBT failures do not impact the R2 specification of L3 filtering and the subsequent steps.

Handling of delayed SSB transmissions due to LBT does not impact L3

FFS: A new RLF trigger mechanism for missing RLM-RS may be defined at upper layers but RAN2 should wait for RAN1 conclusion on this issue

RSSI and Channel Occupancy configuration and reporting, in particular measurements over an interval (at least for CO) and periodical reporting, are used as a baseline for NR-U

Measurement and reporting of WLAN nodes are not supported in NR-U

Also, during the SI phase, the following objectives were included

RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2)

In this contribution, we discuss a mechanism to handle the missing measurements of the reference signals due to LBT failures for an RRC_CONNECTED UE performing RRM measurements.

# Discussion

Although it was agreed that missing measurements of the reference signals due to LBT failures would not affect the R2 specifications of L3 filtering, there needs to be an account for these missing measurements. The discussions in [1] suggest taking care of this problem from an implementation perspective by appropriately generating samples or changing the input rate to the L3 filter. On the other hand, in the case of RRC_CONNECTED state RRM measurements, one of the more straightforward ways to account for these missing measurements would be to configure a larger value of the Time to Trigger (TTT) for the unlicensed bands. The configuration of the larger value of the TTT could be based on the largest DRS Measurement Time Configuration (DMTC) periodicity of the neighbouring cell(s).

Observation: Configuring a larger value for the TTT for the unlicensed cell as a part of the measurement configuration could be used to account for the missing samples due to LBT failures.

Though a larger TTT would help in recovering the missing samples; for a UE to always use a longer TTT for measurement would be unnecessary and could lead to increased power consumption. In addition, this could also lead to delays in reporting. As an alternative, the UE could be configured with two values of TTT, one short and the other long. The decision to switch between the two could be based on the number of samples measured as shown in an example in Figure 1 below.

Figure 1: Varying between the configured short and long TTT.

In addition to a purely implementation-based solution, a formal approach to account for these missing measurements could help in a common understanding of the neighbouring cell(s) measurements at the network side. Furthermore, based on discussions in [1] and related to the new metric for DRS availability as mentioned in [2] and [3], reporting the type of configured TTT used i.e., short or long can be used as an indication for LBT failures.

Proposal: Configure a short and long value for the TTT with the decision to choose between the two based on missing measurements.

# Conclusion

Based on our analysis carried out in this contribution, we have the following observation:

Observation: Configuring a larger value for the TTT for the unlicensed cell as a part of the measurement configuration could be used to account for the missing samples due to LBT failures.

It is proposed to discuss and decide on the following proposal:

Proposal: Configure a short and long value for the TTT with the decision to choose between the two based on the number of missing measurements.

# Reference

- R2-1903655, Report of Email Discussion [105#48][NR-U] Connected mode measurements, Qualcomm Incorporated

- R2-1906327, RRM and Mobility for NR-U, Qualcomm

- R2-1906741, The metric reflecting the DRS availability in NR-U, Panasonic