3GPP TSG-RAN WG2 Meeting #107	R2-1910570

Prague, Czech Republic, 26-30 August 2019	Resubmission of R2-1907781

Agenda item:	11.6.4.1.1

Source: 	Huawei, HiSilicon

Title: 	RACH-less handover for NTN

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss RACH-less handover for NTN.

Discussion

In the RAN2 #104 meeting, it was agreed that the details of satellite ephemeris can be captured in the TR. The satellite ephemeris could provide the location of satellites in any time. According to [1], the targeted usage-scenarios with satellite access are pedestrian, stationary in building, on board vehicle, high speed train or airplanes. In those scenarios, most of the transmission between satellite and UEs will be LoS transmission. Then, if UE location is available and satellite ephemeris is told to UE, UE could estimate the timing advance value to its serving cell based on its location and the serving satellite location.

Observation 1: UE could estimate the timing advance value to its serving cell based on its location and the satellite ephemeris.

The following table shows the delay constraints of different NTN scenarios by referring to [2].

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35 786 km |  | 600 km |  |  |  |
| Relative speed of Satellite wrt earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 5° for feeder |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 1000 km |  | 50 km / 500 km |  |  |  |
| Maximum Round Trip Delay on the radio interface between the gNB and the UE | 541.75 ms (Worst case) | 270.57 ms | 28.41 ms |  | 12.88 ms |  |
| Minimum Round Trip Delay on the radio interface between the gNB and the UE | 477.14 ms | 238.57 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |

According to this table, the RTT of NTN is much higher than terrestrial network, especially for GEO scenario. In the legacy handover mechanism, after receiving RRC reconfiguration from source gNB, UE will perform contention-free RACH in the target gNB to obtain TA value of target cell. If the legacy handover mechanism is adopted in NTN, the handover interruption latency will very long due to the long RTT. RACH-less handover is discussed in LTE with the condition that UE has same TA value in source gNB and target gNB. According to observation 1, RACH-less handover mechanism could be used in NTN due to the fact that UE could estimate the TA value of target gNB after obtaining its location and satellite ephemeris. In this way, handover interruption latency could be reduced by at least one RTT.

Observation 2: RACH-less handover mechanism could work in NTN when UE could estimate its TA value of target gNB.

Figure 1 Legacy handover mechanism

Figure 2 RACH-less handover mechanism

Hence, RAN2 should study supporting RACH-less handover mechanism in NTN from the perspective of reducing handover interruption latency.

Proposal 1: consider supporting RACH-less handover mechanism in NTN.

Proposal 2: capture the TP in Annex into TR.

Conclusion

In this contribution, we discussed RACH-less handover for NTN, and we get the following proposals:

Observation 1: UE could estimate the timing advance value to its serving cell based on its location and the satellite ephemeris.

Observation 2: RACH-less handover mechanism could work in NTN when UE could estimate its TA value of target gNB.

Proposal 1: consider supporting RACH-less handover mechanism in NTN.

Proposal 2: capture the TP in Annex into TR.

References

[1] RP-190710, Study on solutions for NR to support non-terrestrial networks (NTN)

[2] 3GPP TR 38.811: “Study on New Radio (NR) to support non terrestrial networks”

Appendix

- 7.3.2 	Connected mode mobility enhancements

Editor’s note: RAN2 will study impacts and possible enhancements to Mobility (hand-over)

Editor’s note: NTN specific aspects related CHO can be studied in the RAN2#106meeting

For GEO NTN, mobility management procedures require adaptations to accommodate large propagation delay. In particular radio link management may require specific configuration.

For LEO NTN, mobility management procedures should be enhanced to take into account satellite movement related aspects such as measurement validity, UE velocity, movement direction, large and varying propagation delay and dynamic neighbour cell set.

The RTT of NTN is much higher than terrestrial network, especially for GEO scenario. In the legacy handover mechanism, after receiving RRC reconfiguration from source gNB, UE will perform contention-free RACH in the target gNB to obtain TA value of target cell. If the legacy handover mechanism is adopted in NTN, the handover interruption latency will very long due to the long RTT. RACH-less handover is discussed in LTE with the condition that UE has same TA value in source gNB and target gNB. The targeted usage-scenarios with satellite access are pedestrian, stationary in building, on board vehicle, high speed train or airplanes. In those scenarios, most of the transmission between satellite and UEs will be LoS transmission. Then, if UE location is available and satellite ephemeris is told to UE, UE could estimate the timing advance value to its serving cell based on its location and the serving satellite location. Then, RACH-less handover mechanism could be used in NTN due to the fact that UE could estimate the TA value of target gNB after obtaining its location and satellite ephemeris. In this way, handover interruption latency could be reduced by at least one RTT.