3GPP TSG-RAN2 Meeting #107	R2-1910737

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0145 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Default DRB handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last RAN2#106 meeting, the following agreements were taken about the establishment of the default DRB at the SN. Agreements: 1) The SN is allowed to setup a default DRB for SN-terminated bearers (i.e., and for PDU sessions associated with an SDAP entity mapped to the SN). 2) The MN decides whether the SN is allowed to setup the default DRB. 3) The MN informs the SN whether it can configure a default DRB via an indication included in the Xn signalling (RAN3 to confirm). The stage 2 specification should be updated to reflect what has been agreed. Furthermore, for a QoS flow of a split PDU session, if its UL packets match neither an RRC configured nor a reflective mapping rule, there is problem if the QoS flow is terminated at one NG-RAN node but the default DRB is configured at the other NG-RAN node, since the SDAP entity handling the default DRB cannot map the UL packets from the default DRB to the correct QoS flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 8.2 Bearer type selection: - Add clarification that the default DRB may be configured also a the SN and MN decides whether the SN is allowed to setup the default DRB or not. - Add clarification that, for split PDU sessions, NG-RAN should guarantee that each QoS flow terminated at a node where the default DRB is not configured should be explicitly configured with an UL QoS flow to DRB mapping rule. Impact analysis Impacted 5G architecture options: NGEN-DC, NE-DC, NR-DC Impacted functionality: Bearer type selection and configuration Inter-operability: no inter-operability issue as the UE behaviour is not affected by the CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the CR is not approved, if a QoS flow is terminated at one NG-RAN node but the default DRB is configured at the other NG-RAN node, the SDAP entity handling the default DRB might not be able to map UL packets from the default DRB to the correct QoS flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 8.2	Bearer type selection

In EN-DC, for each radio bearer the MN decides the location of the PDCP entity and in which cell group(s) radio resources are to be configured. Once an SN terminated split bearer is established, e.g. by means of the Secondary Node Addition procedure or MN initiated Secondary Node Modification procedure, the SN may removeSCG resources for the respective E-RAB, as long as the QoS for the respective E-RAB is guaranteed.

In MR-DC with 5GC, the following principles apply:

-	The MN decides per PDU session the location of the SDAP entity, i.e. whether it shall be hosted by the MN or the SN or by both (split PDU session);

-	If the MN decides to host an SDAP entity it may decide some of the related QoS flows to be realized as MCG bearer, some as SCG bearer, and others to be realized as split bearer;

-	If the MN decides that an SDAP entity shall be hosted in the SN, some of the related QoS flows may be realized as SCG bearer, some as MCG bearer, while others may be realized as split bearer. The SN is responsible to assign the corresponding DRB IDs, based on the DRB IDs indicated by the MN. The SN may remove or add SCG resources for the respective QoS flows, as long as the QoS for the respective QoS flow is guaranteed

-	For each PDU session, including split PDU sessions, at most one default DRB may be configured (see [3]) at the MN or SN. The MN decides whether the SN is allowed to setup the default DRB or not. For split PDU sessions, NG-RAN should guarantee that each QoS flow terminated at a node where the default DRB is not configured should be explicitly configured with an UL QoS flow to DRB mapping rule.

END OF CHANGES