3GPP TSG-RAN WG2 #107	Tdoc R2-1910787

Prague, Czech Republic, 26 – 30 Aug 2019                                 (Revision of R2-1907599)

Title:	Draft LS to RAN1 on signalling options of LBT category for Msg3

Response to:

Release:	Release 16

Work Item:	NR_unlic-Core

Source:	Ericsson (to be RAN2)

To:	RAN1

Cc:

Contact Person:

#### Name:                   Robert Karlsson

E-mail Address:	robert.s.karlsson@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

in RAN1#97, RAN1 has made below agreements replying to the RAN2 LS.

Agreement:

Reply to the RAN2 LS informing them of the following:

- RAN1 has made the following agreement which facilitates COT sharing between Msg2 and Msg3:

  - LBT category for msg 3 initial transmission is provided to the UE in RAR.

In case it is feasible for Msg3 to share the DL COT initiated by the gNB with the DL transmission of Msg2, the gNB can decide LBT category for Msg3 depending on the gap duration (according to the Table 7.2.1.3.1-3 in TR 38.889). In other words, the LBT category is determined based on below conditions

- Category 1 immediate transmission for Msg3 if the gap is less than 16us

- Category 2 LBT for Msg3 if the gap is not more than 25us.

However, there may be cases where it is not feasible to apply COT sharing between Msg2 and Msg3 since there is no other DL transmission that can be used to occupy the gap between the end of the DL transmission to the beginning of the Msg3 transmission for a UE. In this case, the UE shall always apply Category 4 LBT with highest priority CAPC for Msg3, which has been already agreed by RAN1.

In total, it is enough to use 2 bits for signaling LBT category in the RAR.

In the LTE LAA, for dynamic scheduling, channel access type and channel access priority class are signaled via uplink grant in DCI (e.g., format 0A and format 0B). It would be beneficial to apply a similar signaling mechanism for the Msg3 transmission in NR-U.  LBT category can be carried via the UL grant field in the RAR message.

It is preferred for RAN2 to signal the LBT category for Msg3 in the UL grant field in the RAR. This needs to be informed to RAN1.

RAN2 would like to ask RAN1

- Is it feasible to signal the LBT category for Msg3 via the UL grant field in the RAR?

- If the answer is yes, will the size of the UL grant field change?

2. Actions:

To RAN1 group.

ACTION:

RAN2 would like to ask RAN1

- Is it feasible to signal the LBT category for Msg3 via the UL grant field in the RAR?

- If the answer is yes, will the size of the UL grant field change?

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#107bis		14 - 18 Oct 2019		China

3GPPRAN2#108                             18 – 22 Nov 19                       Reno, Nevada