3GPP TSG-RAN WG2 #107	R2-1910867

Prague, Czech Republic, 26th August – 30th August 2019

Agenda Item:	11.12.3

Source:	Ericsson

Title:	TP to 38.314 on UL air interface delay measurement

Document for:	Discussion, Decision

# Text Proposal to 38.314

### X.Y.Z	 Packet Delay

#### X.Y.Z.1	Over-the-air packet delay in the UL

The objective of this measurement is to measure air interface UL packet delay for OAM performance observability or for QoS verification of MDT.

Protocol Layer: MAC, RLC

| Definition | Over-the-air packet delay in the UL. This measurement refers to packet delay for DRBs. This measurement provides the average (arithmetic mean) time it takes to get a response back on a HARQ transmission in the uplink direction. Detailed Definition: M(T,qci)=∀itAck(i)-tSched(i)I(T),where explanations can be found in the table x.y.z.1-1 below. |
| --- | --- |

Table x.y.z.1-1

|  | Over-the-air packet delay in the UL per QCI, averaged during time period . Unit: Integer ms. |
| --- | --- |
| tSched(i) | The point in time when the UL RLC SDU i is scheduled as per the scheduling grant provided. |
|  | The point in time when the RLC SDU i was received successfully by the network which in turn triggers the HARQ feedback information. |
|  | A RLC PDU that arrives at the MAC upper SAP during time period . RLC SDU for which HARQ acknowledgement is not received for all parts shall not be included in the calculation. |
|  | Total number of RLC SDUs . |
|  | Time Period during which the measurement is performed |