3GPP TSG-RAN WG2 Meeting#107	R2-1910940

Prague, Czech Republic, 26th – 30th August, 2019

Agenda Item:	11.16

Source: 	Huawei, HiSilicon

Title:	MAC CE signalling enhancement for TCI indication of single-PDCCH based multi-TRP transmission

Document for:	Discussion and Decision

# Introduction

In the LS [1] from RAN1, the following agreement about TCI indication of single PDCCH based multi-TRP transmission is included:

For single PDCCH based multi-TRP/panel transmission,

- TCI indication framework shall be enhanced in Rel-16 at least for eMBB. Each TCI code point in a DCI can correspond to 1 or 2 TCI states. When 2 TCI states are activated within a TCI code point, each TCI state corresponds to one CDM group, at least for DMRS type 1 and FFS design for DMRS type 2. MAC-CE can be enhanced to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2.

According to the agreement, each TCI code point in a DCI can correspond to 1 or 2 TCI states. Thus, there are at most 16 TCI states activated by 3 bit TCI indication.

In this paper, we present our views on MAC-CE signalling enhancement for TCI indication.

# Discussion

In Rel-15, MAC-CE can activate at most 8 TCI states, and map these TCI states to the codepoints of the DCI Transmission Configuration Indication field. Then, each of the eight codepoints (000 to 111) of TCI field in DCI may be mapped with one TCI state by MAC CE. As shown in Figure 1, the MAC-CE used for TCI states activation/deactivation for UE-specific PDCSH includes the following fields: Serving cell ID, BWP ID field, Ti field and R field. The Ti field indicates the activation/deactivation status of TCI state with TCI-StateId i configured by PDSCH-Config, when the Ti filed is set to 1, the corresponding TCI-StateId is activated, otherwise deactivated. The maximum number of TCI states is 128, and the maximum number of activated TCI states is 8 for each BWP. Then, the activated TCI State is mapped to the codepoint sequentially by its ordinal position among all the TCI States with Ti field set to 1.

Figure 1: TCI states Activation/Deactivation for UE specific PDSCH MAC-CE

In Rel-16, to support multi-TRP transmission, MAC-CE should activate at most 16 TCI states, and each codepoint of TCI field in DCI should be mapped with 1 or 2 TCI states. Obviously, the MAC-CE used for TCI states activation/deactivation for UE specific PDSCH in Rel-15 can be extended to support TCI indication of single-PDCCH based multi-TRP transmission easily, where only the Ti field should be enhanced. Firstly, the maximum number of activated TCI states should be relaxed to 16, that is to say at most 16 Ti field can be set to 1. The key issue is how to map the activated TCI states to the 8 codepoints of TCI field in DCI and each codepoints corresponds to 1 or 2 TCI states. A simple method is that the TCI states configured by PDSCH-Config can be divided to 2 TCI state set. Then, the activated TCI State in the first set is mapped to the codepoint sequentially by its ordinal position among all the TCI States in the first set with Ti field set to 1, and the activated TCI State in the second set is mapped to the codepoint sequentially by its ordinal position among all the TCI States in the second set with Ti field set to 1. As shown in Figure 2, there are 128 TCI states configured by PDSCH-Config, and the first TCI state set includes TCI-stateId {0-63}, the second TCI state set includes TCI-stateId {64-127}. The first several TCI state ID in the first TCI state set that set to “1” can be mapped to codepoint {0-7} with increasing order and the subsequent several TCI state ID in the second TCI state set that set to “1” can be mapped to codepoint {0-7} with increasing order. Thus, within the above mapping rule, the value of Ti field in Figure 2 for MAC-CE used for TCI states activation/deactivation for UE specific PDSCH, the relation between codepoint of TCI field in DCI and activated TCI state is listed in Figure 3.

Figure 2: An example of Ti field in enhanced MAC CE used for TCI states Activation/Deactivation for PDSCH

| Codepoint of TCI field | TCI state |  |
| --- | --- | --- |
| 000 | TCI-stateId 0 | TCI-stateId 64 |
| 001 | TCI-stateId 2 | TCI-stateId 65 |
| 010 | TCI-stateId 4 | TCI-stateId 66 |
| 011 | TCI-stateId 6 | TCI-stateId 67 |
| 100 | TCI-stateId 7 | TCI-stateId 68 |
| 101 | TCI-stateId 8 | TCI-stateId 70 |
| 110 | TCI-stateId 10 | TCI-stateId 71 |
| 111 | TCI-stateId 12 | TCI-stateId 120 |

Figure 3: An example of relation between codepoints of TCI field and TCI states

Proposal 1: The MAC-CE for TCI states activation/deactivation for UE specific PDSCH specified in Rel-15 can be reused for TCI indication of signal-PDCCH based multi-TRP/panel transmission. Not introduce new MAC-CE format.

Proposal 2: The maximum number of activated TCI states indicated by MAC-CE for TCI indication for UE specific PDSCH should be relaxed to 16.

Proposal 3: TCI states configured by PDSCH-Config can divided into 2 TCI state set when MAC-CE mapping the activated TCI state to codepoints of TCI field in DCI, and activated TCI states indicated by MAC-CE in each TCI state set are mapped to codepoints of TCI field in DCI separately with increasing order.

# Conclusion

This paper mainly discusses about MAC CE signalling enhancement for TCI indication of single-PDCCH based multi-TRP/panel transmission. Based on the above discussion, we have the following proposal:

Proposal 1: The MAC-CE for TCI states activation/deactivation for UE specific PDSCH specified in Rel-15 can be reused for TCI indication of signal-PDCCH based multi-TRP/panel transmission. Not introduce new MAC-CE format.

Proposal 2: The maximum number of activated TCI states indicated by MAC-CE for TCI indication for UE specific PDSCH should be relaxed to 16.

Proposal 3: TCI states configured by PDSCH-Config can divided into 2 TCI state set when MAC-CE mapping the activated TCI state to codepoints of TCI field in DCI, and activated TCI states indicated by MAC-CE in each TCI state set are mapped to codepoints of TCI field in DCI separately with increasing order.

# References

[1] R1-1903697, LS on support of Enhancements on multi-TRP/panel transmission.