3GPP TSG-RAN WG2 Meeting #107	R2-1911282

# Prague, Czech Republic,26th - 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0171 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 5GC to EPC inter-RAT inter-system handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to correct the Stage-2 description to be align with the Stage-3 procedure on inter-RAT inter-system handover changed by the agreed CRs (R2-1908518 and R2-1908519). According to the CRs, full configuration procedure is not performed to reset radio bearers for inter-RAT inter-system handover althogh fullconfig flag is always set in the RRCConnectionReconfigration massage carried by MobilityFromNRCommand. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text “A full configuration is required to reset the radio bearers.” is removed from 5GC to EPC inter-RAT inter-system handover characters. Impact analysis Impacted 5G architecture options: Standalone, NE-DC and NR-DC Impacted functionality: From 5GC to EPC inter-RAT handover.. Inter-operability: none as the corresponding Stage 3 mechanisms are already captured TS 36.331 and TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 description is not aligned with Stage-3 procedure, and keeps wrong description on inter-RAT inter-system handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

9.3.2.2	Handover and redirection

The source NG-RAN node decides between handover or redirection to EPS based on radio criteria and availability of the N26 interface.

NOTE:	Information about the availability of the N26 interface may be configured by OAM at the NG-RAN.

Inter RAT handover is characterised by the following:

-	The Source RAT configures Target RAT measurement and reporting.

-	The source RAT decides on the preparation initiation and provides the necessary information to the target RAT in the format required by the target RAT.

-	Radio resources are prepared in the target RAT before the handover.

-	The RRC reconfiguration message from the target RAT is delivered to the source RAT via a transparent container, and is passed to the UE by the source RAT in the handover command.

-	In-sequence and lossless handovers are not supported.

-	Security procedures for handover to E-UTRA/EPC should follow legacy inter-RAT handover procedures.

E of Changes