3GPP TSG-RAN2 107                                            R2-1911318

Prague, Czech Republic, 26th Aug – 30th Aug 2019

Agenda item:	10.4.5.1

Source:	Samsung

Title:	Discussion on Inter-RAT Cell Selection from NR to GSM/UTRAN

Document for:	Discussion & Decision

# Introduction

Cell re-selection requires broadcast/dedicated signalling support i.e. cell re-selection parameters. We do not have this e.g. GSM, UTRAN case so inter-RAT cell reselection from NR to GSM/UTRAN is not supported in Rel-15. However, it seems not crystal clear whether GSM/UTRAN inter-RAT cell selection is supported or not in Rel-15. In this contribution we discuss this aspect.

# Discussion

Inter-RAT cell selection from NR to GSM/UTRAN is not specified in 38.304. There are two possible interpretations i.e.

1: Inter-RAT cell selection from NR to GSM/UTRAN is neither specified nor supported.

2: Inter-RAT cell selection from NR to GSM/UTRAN is not specified but could be supported.

In our understanding, interpretation 2 is more correct because nothing can prevent inter-RAT cell selection from NR to GSM/UTRAN.

Proposal 1: RAN2 is asked to discuss whether inter-RAT cell selection from NR to GSM/UTRAN is supported or not in Rel-15.

If interpretation 2 is correct, we would like to further discuss whether the following clauses need to be captured in 38.304, same as in 36.304 as below:

5.2.3.4  GSM case in Cell Selection

The cell selection criteria and procedures in GSM are specified in TS 43.022 [9].

5.2.3.5            UTRAN case in Cell Selection

The cell selection criteria and procedures in UTRAN are specified in TS 25.304 [8].

Proposal 2: RAN2 is asked to discuss whether to capture GSM/UTRAN cell selection in 38.304, same as in 36.304.

It is specified in 5.2.4.1,

-	The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

It would be good to clarify whether dedicated priorities (for NR/EUTRA) and the remaining validity time (i.e. T322 in UTRA and T3230 in GERAN) are inherited if inter-RAT cell selection from NR to GSM/UTRAN happens.

Proposal 3: RAN2 is asked to whether dedicated priorities and the remaining validity time (i.e. T322 in UTRA and T3230 in GERAN) are inheritied if inter-RAT cell selection from NR to GSM/UTRAN happens.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 is asked to discuss whether inter-RAT cell selection from NR to GSM/UTRAN is supported or not in Rel-15.

Proposal 2: RAN2 is asked to discuss whether to capture GSM/UTRAN cell selection in 38.304, same as in 36.304.

Proposal 3: RAN2 is asked to whether dedicated priorities and the remaining validity time (i.e. T322 in UTRA and T3230 in GERAN) are inheritied if inter-RAT cell selection from NR to GSM/UTRAN happens.

# References